# PACLITAXEL    CAS # 33069624   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   B   .   .   E   .   G   .   I   J   .   L

NFPA HAZARD CODES (H,F,R,O)  3   0   0

HUMAN TERATOGEN - DESIGNATED AREA MAY BE REQUIRED

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes severe eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SENSITIZATION

Respiratory: May cause allergic respiratory reaction.

skin: May cause allergic skin reaction.

TARGET ORGAN(S) OR SYSTEM(S)

Cardiovascular system. Nerves. Eyes. Kidneys. Liver. Bone marrow.

SIGNS AND SYMPTOMS OF EXPOSURE

Alopecia. Damage to the liver. Damage to the kidneys. Nausea,

headache, and vomiting. Can cause severe cardiotoxicity and bone

marrow suppression.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Oxidizing agents.

FIRE EXTINGUISHER:  Noncombustible. Use extinguishing media appropriate

tosurrounding fire conditions.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

Store at 2-8°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid breathing dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. Store at

2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 37/38 41 42/43 62 68

Risk Statements: Irritating to respiratory system and skin. Risk

of serious damage to eyes. May cause sensitization by inhalation

and skin contact. Possible risk of impaired fertility. Possible

risk of irreversible effects.

S: 22 26 36/37/39 45

Safety Statements: Do not breathe dust. In case of contact with

eyes, rinse immediately with plenty of water and seek medical

advice. Wear suitable protective clothing, gloves, and eye/face

protection. In case of accident or if you feel unwell, seek

medical advice immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PALLADIUM    CAS # 7440053

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   2

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Reacts violently in air

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

WASTE CHARACTERISTIC HAZARD:   REACTIVE

INCOMPATIBILITIES:Strong acids, Halogens.

FIRE EXTINGUISHER:  Use extinguishing media appropriate to surroundingfire

conditions.

REACTIVE PROPERTIES

HANDLING: Avoid contact and inhalation. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: F

Indication of Danger: Highly Flammable.

R: 17

Risk Statements: Spontaneously flammable in air.

S: 17

Safety Statements: Keep away from combustible material.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  10 mg/m3

DOE Short Term Exposure Limit  30 mg/m3

DOE Ceiling Limit 50 mg/m3

Immediately Dangerous to Life and Health   200 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PALLADIUM    CAS # 7440326

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   2

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Reacts violently in air

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

WASTE CHARACTERISTIC HAZARD:   REACTIVE

INCOMPATIBILITIES:Oxygen, Aluminum Carbon dioxide., Halogens, Chlorinated

solvents, Strong acids, Strong oxidizing agents

FIRE EXTINGUISHER:  Use approved class D extinguishers or smother with

drysand, dry ground limestone, or dry clay. Do not use water. Do not use dry

chemical powder   extinguisher on this material.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat, sparks, and open flame. Handle and store under argon\. SPECIAL

REQUIREMENTS  Air and moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: F

Indication of Danger: Highly Flammable.

R: 17

Risk Statements: Spontaneously flammable in air.

S: 16 33 27 36/37/39

Safety Statements: Keep away from sources of ignition - no

smoking. Take precautionary measures against static discharges.

Take off immediately all contaminated clothing. Wear suitable

protective clothing, gloves, and eye/face protection.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  10 mg/m3

DOE Short Term Exposure Limit  30 mg/m3

DOE Ceiling Limit 50 mg/m3

Immediately Dangerous to Life and Health   50 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PALLADIUM CHLORIDE    CAS # 7647101

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  3   0   0

ACUTE TOXICTY RISK INDEX 2.3 - LD50  2704.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes severe eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SENSITIZATION

skin: May cause allergic skin reaction.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Avoid breathing dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 37/38 41 43

Risk Statements: Irritating to respiratory system and skin. Risk

of serious damage to eyes. May cause sensitization by skin

contact.

S: 26 36/37/39

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .15 mg/m3

DOE Short Term Exposure Limit  .4 mg/m3

DOE Ceiling Limit 3 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PALLADIUM HYDROXIDE    CAS # 12135227

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Acids, Alcohols, Oxidizing agents, Sulfur.

FIRE EXTINGUISHER:  Carbon dioxide. Dry chemical powder. Water spray.

REACTIVE PROPERTIES

HANDLING: Use protective clothing, gloves, and mask. Do not breathe dust.

Avoid contact with eyes, skin, and clothing. Avoid    prolonged or repeated

exposure. STORAGE: Keep tightly closed. Store in a cool dry place.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .125 mg/m3

DOE Short Term Exposure Limit  .3 mg/m3

DOE Ceiling Limit 2.5 mg/m3

Immediately Dangerous to Life and Health   75 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PALLADIUM(II) ACETATE    CAS # 3375313

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 1.9 - LD50  6774.2 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes severe eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 41

Risk Statements: Risk of serious damage to eyes.

S: 26 39

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PALLADIUM(II) ACETYLACETONATE    CAS # 14024614

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 2.4 - LD50  2000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

May liberate 2,4-pentanedione upon decomposition.

2,4-Pentanedione has the following toxicological hazards: toxic,

irritant, neurological hazard, teratogen, possible mutagen,

target organ - thymus. In humans, 2,4-pentanedione is reported

to cause contact dermatitis and contact urticaria. To the best

of our knowledge, the chemical, physical, and toxicological

properties have not been thoroughly investigated.

CONDITIONS AGGRAVATED BY EXPOSURE

May cause nervous system disturbances.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

VAPOR PRESSURE .010000 mm Hg @ 20 °C

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Palladium oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36

Risk Statements: Irritating to eyes.

S: 26

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PALLADIUM(II) BROMIDE    CAS # 13444945

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Noncombustible. Use extinguishing media appropriate to

surrounding fire conditions.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PALLADIUM(II) IODIDE    CAS # 7790387

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin. Causes eye and skin irritation.

TARGET ORGAN(S) OR SYSTEM(S)

Thyroid.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause: Gastrointestinal disturbances. To the best

of our knowledge, the chemical, physical, and toxicological

properties have not been thoroughly investigated. Prolonged

exposure to iodides may produce iodism in sensitive individuals.

Symptoms of exposure include: skin rash, running nose, headache

and irritation of the mucous membrane. For severe cases the skin

may show pimples, boils, hives, blisters and black and blue

spots. Iodides are readily diffused across the placenta.

Neonatal deaths from respiratory distress secondary to goiter

have been reported. Iodides have been known to cause

drug-induced fevers, which are usually of short duration.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Mineral acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam. Water

TOXIC EMISSIONS WHEN BURNED: Hydrogen iodide

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PALLADIUM(II) NITRATE HYDRATE    CAS # 10102053

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  3   0   1   OX

ACUTE TOXICTY RISK INDEX 3.4 - LD50   200.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: May be harmful if inhaled. Material is extremely

destructive to the tissue of the mucous membranes and upper

respiratory tract.

Ingestion: May be harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Blood. Central nervous system.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Oxidizing Agent

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Alcohols, Alkali metals, Amines, Bases,  Aluminum, Brass

Copper, Copper alloys Corrodes steel., Galvanized    iron, Organic materials,

Reducing agents

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Palladium  oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from combustible    materials, heat, sparks, and open flame. Store in a cool

dry    place. Store under nitrogen\. SPECIAL REQUIREMENTS  Hygroscopic. Store

under inert gas.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: O C

Indication of Danger: Oxidizing. Corrosive.

R: 8 34

Risk Statements: Contact with combustible material may cause

fire. Causes burns.

S: 17 26 36/37/39 45

Safety Statements: Keep away from combustible material. In case

of contact with eyes, rinse immediately with plenty of water and

seek medical advice. Wear suitable protective clothing, gloves,

and eye/face protection. In case of accident or if you feel

unwell, seek medical advice immediately (show the label where

possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PALLADIUM(II) SULFATE    CAS # 13566035

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   F   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be harmful

if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Material is extremely destructive to tissue of the   mucous

membranes and upper respiratory tract, eyes, and skin. To

the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC   CORROSIVE

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Noncombustible. Use extinguishing media appropriate

tosurrounding fire conditions.

TOXIC EMISSIONS WHEN BURNED: Palladium oxides Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place. Store   with desiccant\. SPECIAL REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34

Risk Statements: Causes burns.

S: 22 26 36/37/39 45

Safety Statements: Do not breathe dust. In case of contact with

eyes, rinse immediately with plenty of water and seek medical

advice. Wear suitable protective clothing, gloves, and eye/face

protection. In case of accident or if you feel unwell, seek

medical advice immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PALLADIUM(II) SULFIDE    CAS # 12125223

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Sulfur oxides Hydrogen sulfide gas

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

STORAG   Suitable: Keep tightly closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATIO   Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PALMITIC ACID    CAS # 57103

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   1   0

ACUTE TOXICTY RISK INDEX 1.7 - LD50 10000.0 mg/Kg

INHALATION RISK INDEX   1.1 - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

VAPOR PRESSURE .950 mm Hg @ 20 °C

FLASH POINT  402.8 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

d - Organic Acid/Flammable/Toxic

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong reducing  agents, Bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36

Risk Statements: Irritating to eyes.

S: 26

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  25 mg/m3

DOE Short Term Exposure Limit  50 mg/m3

DOE Ceiling Limit 50 mg/m3

Immediately Dangerous to Life and Health   50 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PALMITIC ACID SODIUM    CAS # 408355

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PALMITOLEIC ACID    CAS # 373499

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   2   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material is irritating to

mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  143.6 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

d - Organic Acid/Flammable/Toxic

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at -20°C

Keep tightly closed. Keep away from heat and open flame.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame. Store at -20░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PANCREATIN    CAS # 8049476

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  3   0   0

ACUTE TOXICTY RISK INDEX 1.7 - LD50 10000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material is irritating to

mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SENSITIZATION

Respiratory: May cause allergic respiratory reaction.

TARGET ORGAN(S) OR SYSTEM(S)

Lungs.

SIGNS AND SYMPTOMS OF EXPOSURE

Repeated exposure can cause: Asthma.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Materials to Avoid: O

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at -20°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. Handle in a    closed system. STORAGE:

Keep tightly closed. Store at -20░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 36/37/38 42

Risk Statements: Irritating to eyes, respiratory system and

skin. May cause sensitization by inhalation.

S: 22 24 26 36/37

Safety Statements: Do not breathe dust. Avoid contact with skin.

In case of contact with eyes, rinse immediately with plenty of

water and seek medical advice. Wear suitable protective clothing

and gloves.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PANTOTHENIC ACID HEMICALCIUM (D-)    CAS # 137086

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   F   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 1.7 - LD50 10000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   CORROSIVE

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Calcium oxide

Store at 2-8°C

Air, light, and moisture sensitive.

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. Store at

2-8░C SPECIAL REQUIREMENTS  Air, light, and moisture sensitive.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PANTOTHENOL    CAS # 16485102

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Hygroscopic.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PAPAIN    CAS # 9001734

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  3   0   0

ACUTE TOXICTY RISK INDEX 2.1 - LD50  4000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SENSITIZATION

Respiratory: May cause allergic respiratory reaction.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause asthma, skin eruptions, emphysema,

anaphylactic shock, and proteolytic action. To the best of our

knowledge, the chemical, physical, and toxicological properties

have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Materials to Avoid: O

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at 2-8°C

Keep tightly closed.

Light sensitive.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

2-8░C SPECIAL REQUIREMENTS  Light sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 36/37/38 42

Risk Statements: Irritating to eyes, respiratory system and

skin. May cause sensitization by inhalation.

S: 22 24 26 36/37

Safety Statements: Do not breathe dust. Avoid contact with skin.

In case of contact with eyes, rinse immediately with plenty of

water and seek medical advice. Wear suitable protective clothing

and gloves.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PARAFFIN    CAS # 64742514

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   1   OX

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Lung irritation. Prolonged exposure can cause: Ingestion of

large quantities will produce a laxative effect and may be

irritating to the digestive tract. There is also a risk of

esophageal, gastric, small intestinal, and rectal obstruction

due to osmotic disturbance.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  235 °F

Oxidizing Agent

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam. Foam and water spray are effective but may   cause

frothing.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store

away from heat. Store in a   cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

UNITED STATES REGULATORY INFORMATION

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PARAFFIN    CAS # 8002742

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

ACUTE TOXICTY RISK INDEX 2 - LD50  5000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Lung irritation. Prolonged exposure can cause: Ingestion of

large quantities will produce a laxative effect and may be

irritating to the digestive tract. There is also a risk of

esophageal, gastric, small intestinal, and rectal obstruction

due to osmotic disturbance.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  235 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam. Foam and water spray are effective but may   cause

frothing.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store

away from heat. Store in a   cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

UNITED STATES REGULATORY INFORMATION

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  2 mg/m3

DOE Short Term Exposure Limit  15 mg/m3

DOE Ceiling Limit 15 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PARAFORMALDEHYDE    CAS # 30525894   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   G   .   I   J   K   L

NFPA HAZARD CODES (H,F,R,O)  3   2   0

ACUTE TOXICTY RISK INDEX 4 - LD50   592.0 mg/Kg

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION HAZARD

INHALATION RISK INDEX   2.0 - LC50   1070.0

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes severe eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. Harmful if inhaled.

Ingestion: Ingestion can cause immediate burning pain in the

mouth, throat, abdomen; severe swelling of the larynx and

skeletal paralysis affecting the ability to breathe, circulatory

shock and convulsions. Harmful if swallowed.

SENSITIZATION

skin: May cause allergic skin reaction.

TARGET ORGAN(S) OR SYSTEM(S)

Eyes. Kidneys. Liver. Heart.

SIGNS AND SYMPTOMS OF EXPOSURE

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Exposure can cause: Drying, cracking, or irritation of

the skin. Pulmonary edema. Effects may be delayed. Pronounced

irreversible ocular damage, eye burns.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

VAPOR PRESSURE    1.1 mm Hg @ 20 °C

FLASH POINT  158 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Acid anhydrides, Strong oxidizing agents, Strong reducing

agents Paraformaldehyde is incompatible with   isocyanates, oxides, and

bronze Copper, Copper alloys, Brass,   Steel.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Formaldehyde  Hazardous Decomposition Products

Formed Upon Contact with Water:   Decomposition by acids alkalies and hot

water with liberation of   formaldehyde

REACTIVE PROPERTIES

HANDLING: Avoid breathing dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep container closed. Keep

away from heat, sparks, and open flame\. SPECIAL REQUIREMENTS Handle and

store under inert gas. Moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 20/22-37/38-40-41-43

Risk Statements: Harmful by inhalation and if swallowed.

Irritating to respiratory system and skin. Limited evidence of a

carcinogenic effect. Risk of serious damage to eyes. May cause

sensitization by skin contact.

S: 26-36/37/39-45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  4 mg/m3

DOE Short Term Exposure Limit  12.5 mg/m3

DOE Ceiling Limit 75 mg/m3

Immediately Dangerous to Life and Health   100 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PARAROSANILINE    CAS # 25620784   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    A   .   .   .   E   .   .   .   .   .   K   .

NFPA HAZARD CODES (H,F,R,O)  1   0   1   OX

SPECIAL CARCINOGEN - DESIGNATED AREA MAY BE REQUIRED

CHRONIC TOXICITY RISK INDEX 3.3 - TD50    21.2 mg/Kg

ACUTE TOXICTY RISK INDEX 1.4 - LD50 18650.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Oxidizing Agent

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

WASTE CHARACTERISTIC HAZARD:   TOXIC

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, STORAGE: Keep

tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PARAROSANILINE ACETATE    CAS # 6035945

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

STORAGE

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 40

Risk Statements: Limited evidence of a carcinogenic effect.

S: 22 36

Safety Statements: Do not breathe dust. Wear suitable protective

clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PATENT BLUE V    CAS # 129179

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   F   G   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  1   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   CORROSIVE

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PCB    CAS # 1336363

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    A   B   .   .   .   .   .   .   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   1   0

SPECIAL CARCINOGEN - DESIGNATED AREA MAY BE REQUIRED

CHRONIC TOXICITY RISK INDEX 4.7 - TD50     1.0 mg/Kg

HUMAN TERATOGEN - DESIGNATED AREA MAY BE REQUIRED

REPRODUCTIVE RISK INDEX   6.1

ACUTE TOXICTY RISK INDEX 1.8 - LD50  7087.0 mg/Kg

NEUROTOXIC - RISK INDEX   4.0

INHALATION RISK INDEX   <1  - LC50

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

FIRE EXTINGUISHER: WATER SPRAY.    CARBON DIOXIDE, DRY CHEMICAL POWDER OR

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: IDE  HYDROGEN CHLORIDE GAS

REACTIVE PROPERTIES

HOT VAPORS EXPLODE ON CONTACT WITH POWDERED ALUMINUM. REACT EXPLOSIVELY

WHENIGNITED WITH A HIGH ENERGY SOURCE.  FORM EXPLOSIVE MIXTURES WITH

POTASSIUM,POTASSIUM ALLOYS, OR SODIUM.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  1 mg/m3

DOE Short Term Exposure Limit  3 mg/m3

DOE Ceiling Limit 5 mg/m3

Immediately Dangerous to Life and Health   5 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PEANUT OIL    CAS # 8002037

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

investigated.

IRRITATION DATA

skin

Human

300 mg

3D

I

Remarks: Mild irritation effect

skin

Rat

100 mg

24H

Remarks: Mild irritation effect

skin

Rabbit

100 mg

24H

Remarks: Moderate irritation effect

skin

Guinea pig

100 mg

24H

Remarks: Moderate irritation effect

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  228.2 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PECTINASE    CAS # 9032751

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SENSITIZATION

Sensitization: Prolonged or repeated exposure may cause allergic

reactions in certain sensitive individuals.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at 2-8°C

Keep container closed. Store away from heat.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep container closed. Store

away from heat. Store at 2-8░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PELARGONIN CHLORIDE    CAS # 17334586

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen chloride gas Carbon  monoxide

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

-20░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENICILLIN G POTASSIUM    CAS # 113984

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  3   0   0

ACUTE TOXICTY RISK INDEX 1.9 - LD50  6700.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SENSITIZATION

Respiratory: May cause allergic respiratory reaction.

skin: May cause allergic skin reaction.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Acids, glycerol alcohol, Heavy metal salts,  Oxidizing

agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Avoid breathing dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 42/43

Risk Statements: May cause sensitization by inhalation and skin

contact.

S: 36/37

Safety Statements: Wear suitable protective clothing and gloves.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENTABROMOPHENOL    CAS # 608719

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   E   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 3.4 - LD50   200.0 mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: Toxic if absorbed through skin.

Eye Contact: Causes eye irritation.

Inhalation: Toxic if inhaled. Material is irritating to mucous

membranes and upper respiratory tract.

Ingestion: Toxic if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Eyes.

SIGNS AND SYMPTOMS OF EXPOSURE

Depending on the intensity and duration of exposure, effects may

vary from mild irritation to severe destruction of tissue.

Prolonged contact can cause: To the best of our knowledge, the

chemical, physical, and toxicological properties have not been

thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen bromide gas

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: T N

Indication of Danger: Toxic. Dangerous for the environment.

R: 23/24/25 36/37/38 50/53

Risk Statements: Toxic by inhalation, in contact with skin and

if swallowed. Irritating to eyes, respiratory system and skin.

Very toxic to aquatic organisms, may cause long-term adverse

effects in the aquatic environment.

S: 26 36/37 45 60 61

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing and gloves. In case of accident or

if you feel unwell, seek medical advice immediately (show the

label where possible). This material and its container must be

disposed of as hazardous waste. Avoid release to the

environment. Refer to special instructions/safety data sheets.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENTABROMOPHENYL ETHER    CAS # 1163195

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   C   .   .   .   .   .   .   .   K   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

EXTREMELY TOXIC - DESIGNATED AREA MAY BE REQUIRED

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin. Causes eye and skin irritation.

TARGET ORGAN(S) OR SYSTEM(S)

Liver. Kidneys. Thyroid. Spleen.

IRRITATION DATA

Eyes

Rabbit

0.1 ml

Remarks: Severe irritation effect

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen bromide gas

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 20/21/22

Risk Statements: Harmful by inhalation, in contact with skin and

if swallowed.

S: 36/37

Safety Statements: Wear suitable protective clothing and gloves.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENTACHLOROBENZENE (1,2,3,4,5-)    CAS # 608935   HAZARDOUS CHEMICAL OF

CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   3   0

ACUTE TOXICTY RISK INDEX 2.7 - LD50  1080.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSUR skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Liver. Kidneys. Blood. Thyroid.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Flammable

VAPOR PRESSURE .002800 mm Hg @ 20 °C

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder,

oappropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep container closed. Keep

away from heat, sparks, and open flame.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: F Xn N

Indication of Danger: Highly Flammable. Harmful. Dangerous for

the environment.

R: 11 22 50/53

Risk Statements: Highly flammable. Harmful if swallowed. Very

toxic to aquatic organisms, may cause long-term adverse effects

in the aquatic environment.

S: 41 46 50 60 61

Safety Statements: In case of fire and/or explosion do not

breathe fumes. If swallowed, seek medical advice immediately and

show this container or label. Do not mix with... This material

and its container must be disposed of as hazardous waste. Avoid

release to the environment. Refer to special instructions/safety

data sheets.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  10 mg/m3

DOE Short Term Exposure Limit  30 mg/m3

DOE Ceiling Limit 150 mg/m3

Immediately Dangerous to Life and Health   400 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENTACHLOROPHENOL    CAS # 87865

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    A   .   C   D   E   .   .   .   I   .   K   .

NFPA HAZARD CODES (H,F,R,O)  3   0   0

SPECIAL CARCINOGEN - DESIGNATED AREA MAY BE REQUIRED

CHRONIC TOXICITY RISK INDEX 3.5 - TD50    13.1 mg/Kg

EXTREMELY TOXIC - DESIGNATED AREA MAY BE REQUIRED

ACUTE TOXICTY RISK INDEX 4.3 - LD50    27.0 mg/Kg

NEUROTOXIC - RISK INDEX   5.0

INHALATION HAZARD  INHALATION RISK INDEX   <1  - LC50

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

VAPOR PRESSURE .001000 mm Hg @ 20 °C

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

d - Organic Acid/Flammable/Toxic

WASTE CHARACTERISTIC HAZARD:   TOXIC

FIRE EXTINGUISHER: WATER SPRAY.    CARBON DIOXIDE, DRY CHEMICAL POWDER OR

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: CTS  TOXIC FUMES OF:  CARB

Immediately Dangerous to Life and Health  2.5 mg/m3

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .5 mg/m3

DOE Short Term Exposure Limit  2.5 mg/m3

DOE Ceiling Limit 2.5 mg/m3

Immediately Dangerous to Life and Health   2.5 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENTAERYTHRITOL    CAS # 115775

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   F   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

ACUTE TOXICTY RISK INDEX 1.4 - LD50 19500.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE   To the best of our knowledge, the chemical,

physical, and toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

VAPOR PRESSURE***** mm Hg @ 20 °C

FLASH POINT  302 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   CORROSIVE

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids, Acid chlorides, Acid

anhydrides.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep container closed. Store in a cool dry place.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  15 mg/m3

DOE Short Term Exposure Limit  30 mg/m3

DOE Ceiling Limit 50 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENTAERYTHRITOL TRIACRYLATE    CAS # 3524683

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   G   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  3   1   0

ACUTE TOXICTY RISK INDEX 2.3 - LD50  1830.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes severe eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

SENSITIZATION

skin: May cause allergic skin reaction.

SIGNS AND SYMPTOMS OF EXPOSURE

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  230 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong acids, Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at 2-8°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/38 43

Risk Statements: Irritating to eyes and skin. May cause

sensitization by skin contact.

S: 39

Safety Statements: Wear eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENTAERYTHRITYL TETRABROMIDE    CAS # 3229003

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen bromide gas

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Do not get in eyes, on skin, on   clothing.

STORAGE: Keep tightly closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENTAERYTHRITYL TETRACHLORIDE    CAS # 3228997

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Do not get in eyes, on skin, on    clothing.

STORAGE: Keep tightly closed. Store in a cool dry place.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENTAFLUOROPHENOL    CAS # 771619

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   2   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Damage to the eyes.

SIGNS AND SYMPTOMS OF EXPOSURE

Depending on the intensity and duration of exposure, effects may

vary from mild irritation to severe destruction of tissue.

Prolonged contact can cause: Damage to the eyes. To the best of

our knowledge, the chemical, physical, and toxicological

properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

VAPOR PRESSURE    2.0 mm Hg @ 20 °C

FLASH POINT  162 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Bases, Acid  chlorides, Acid

anhydrides.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen fluoride

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENTAFLUOROPHENYLBORONIC ACID    CAS # 1582247

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen fluoride Borane/boron oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENTAFLUOROPYRIDINE    CAS # 700163

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   3   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Vapor or mist is irritating to the eyes, mucous

membranes, and upper respiratory tract.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

FLASH POINT  77 °F

Forms ignitable mixtures in air at room temperature - Danger of remote

ignition and flashback

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

a - Organic Base/Flammable/Toxic

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen fluoride

REACTIVE PROPERTIES

User Exposure: Avoid contact and inhalation. Do not get in eyes, on skin,

on clothing. STORAGE: Keep tightly closed. Keep away from heat, sparks, and

open flame. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 10 20/21/22

Risk Statements: Flammable. Harmful by inhalation, in contact

with skin and if swallowed.

S: 26 28

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. After

contact with skin, wash immediately with plenty of water.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENTAFLUOROSTYRENE (2,3,4,5,6-)    CAS # 653349

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   3   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

FLASH POINT  93.2 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen fluoride

Store at -20°C

Keep tightly closed. Keep away from heat, sparks, and open flame.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

STORAGE: Keep tightly closed. Keep away from heat, sparks, and open flame.

Store at -20░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

R: 10

Risk Statements: Flammable.

S: 23 24/25

Safety Statements: Do not breathe vapor. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENTAGLYCINE    CAS # 7093676

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 2.6 - LD50  1100.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENTALOXY BIPHENYLCARBONYL (4':4-)    CAS # 52364713

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

ACUTE TOXICTY RISK INDEX 2 - LD50  5000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  235 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Store in a cool dry place.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENTAMETHYLPIPERIDINE (1,2,2,6,6-)    CAS # 79550

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   2   0

ACUTE TOXICTY RISK INDEX 3.2 - LD50   887.1 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material is irritating to

mucous membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  131 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat, sparks, and open flame.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 10 22 36/37/38

Risk Statements: Flammable. Harmful if swallowed. Irritating to

eyes, respiratory system and skin.

S: 26 36/37

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing and gloves.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENTAMIDINE    CAS # 140647

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  3   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SENSITIZATION

skin: May cause allergic skin reaction.

TARGET ORGAN(S) OR SYSTEM(S)

Cardiovascular system. Kidneys. Pancreas.

SIGNS AND SYMPTOMS OF EXPOSURE

The use of pentamidine in conjunction with other medication with

nephrotoxic effects may increase the potential for

nephrotoxicity. Side effects that have been reported:

drowsiness, increased urination, loss of appetite, flushed dry

skin, fruit-like breath odor, unusual thirst, anxiety, chills,

cold sweats, headache, nausea, nervousness, shakiness, rapid or

irregular pulse, unusual tiredness or weakness, unusual hunger,

blurred vision, dizziness, confusion, hallucinations, skin rash,

unusual bleeding or bruising, sore throat and fever, vomiting,

pancreatitis, anemia, thrombocytopenia, severe neutropenia,

hypoglycemia, abnormal liver function, and sudden death. Other

symptoms of exposure can include moderate to severe chest

tightness, wheezing, coughing, shortness of breath, increased

mucous discharge, stuffy nose, burning eyes, sneezing, and

lightheadedness.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxides

Store at -20°C

Hygroscopic.

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

-20░C SPECIAL REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38 43

Risk Statements: Irritating to eyes, respiratory system and

skin. May cause sensitization by skin contact.

S: 26 36/37

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing and gloves.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENTANE    CAS # 109660   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   C   .   .   F   G   H   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   4   0

EXTREMELY TOXIC - DESIGNATED AREA MAY BE REQUIRED

ACUTE TOXICTY RISK INDEX 1 - LD50         mg/Kg

NEUROTOXIC - RISK INDEX   2.0

INHALATION RISK INDEX   <1  - LC50 ********

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Central nervous system. Heart. Lungs.

SIGNS AND SYMPTOMS OF EXPOSURE

Contact with eyes can cause redness, tearing, and blurred

vision. Prolonged or repeated contact with skin can cause

defatting and dermatitis. CNS depression. Damage to the lungs.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

VAPOR PRESSURE  434.3 mm Hg @ 20 °C

FLASH POINT  -57 °F

Store in an explosion-proof refrigerator or tightly stoppered in a

well-ventilated area

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC   CORROSIVE

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. Open   carefully. STORAGE: Keep

container closed. Keep away from heat, sparks, and open flame. Store in a

cool dry place. Store under nitrogen\. SPECIAL REQUIREMENTS   May develop

pressure. Refrigerate before opening.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: F+ Xn N

Indication of Danger: Extremely Flammable. Harmful. Dangerous

for the environment.

R: 12 51/53 65 66 67

Risk Statements: Extremely flammable. Toxic to aquatic

organisms, may cause long-term adverse effects in the aquatic

environment. Harmful: may cause lung damage if swallowed.

Repeated exposure may cause skin dryness or cracking. Vapors may

cause drowsiness and dizziness.

S: 9 16 29 33 61 62

Safety Statements: Keep container in a well-ventilated place.

Keep away from sources of ignition - no smoking. Do not empty

into drains. Take precautionary measures against static

discharges. Avoid release to the environment. Refer to special

instructions/safety data sheets. If swallowed, do not induce

vomiting: seek medical advice immediately and show this

container or label.

OSHA REGULATORY LIMITS

OSHA Permissible Exposure Limit 600 ppm

OSHA Short Term Exposure Limit  750 ppm

ACGIH RECOMMENDED LIMITS

ACGIH Threshold Limit Value 600 ppm

Immediately Dangerous to Life and Health  1500 ppm

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  610 ppm

DOE Short Term Exposure Limit  610 ppm

DOE Ceiling Limit 610 ppm

Immediately Dangerous to Life and Health   1500 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENTANEDIOL (1,5-)    CAS # 111295

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   F   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   1   0

ACUTE TOXICTY RISK INDEX 2.4 - LD50  2000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause: Gastrointestinal disturbances. Nausea,

headache, and vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE .010000 mm Hg @ 20 °C

FLASH POINT  276.8 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   CORROSIVE

INCOMPATIBILITIES:Acid chlorides, Acid anhydrides, Oxidizing agents,

Chloroformates, Reducing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Use protective clothing, gloves, and mask. Do not breathe vapor.

Avoid contact with eyes, skin, and clothing. STORAGE: Keep tightly closed.

Store in a cool dry place\. SPECIAL REQUIREMENTS  Hygroscopic.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  7.5 mg/m3

DOE Short Term Exposure Limit  25 mg/m3

DOE Ceiling Limit 150 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENTANEDIOL (2,4-)    CAS # 625694

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

ACUTE TOXICTY RISK INDEX 1.9 - LD50  6860.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  213.8 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Acid chlorides, Acid anhydrides, Oxidizing agents,

Chloroformates, Reducing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Use protective clothing, gloves, and mask. Do not breathe vapor.

Avoid contact with eyes, skin, and clothing. STORAGE: Keep tightly closed.

Store in a cool dry place\. SPECIAL REQUIREMENTS  Hygroscopic.-

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENTANEDIONE (2,3-)    CAS # 600146

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   3   0

ACUTE TOXICTY RISK INDEX 2.2 - LD50  3000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material is irritating to

mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

FLASH POINT  66.2 °F

Forms ignitable mixtures in air at room temperature - Danger of remote

ignition and flashback

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

INCOMPATIBILITIES:Oxidizing agents, Reducing agents, Strong  bases.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Water may be effective for cooling, but may not    effect extinguishment.

Store at 2-8°C

Keep container closed. Keep away from heat, sparks, and open flame.

REACTIVE PROPERTIES

HANDLING: Avoid breathing vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep container closed. Keep

away from heat, sparks, and open flame. Store at 2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: F Xi

Indication of Danger: Highly Flammable. Irritant.

R: 11 36/37/38

Risk Statements: Highly flammable. Irritating to eyes,

respiratory system and skin.

S: 16 26 36

Safety Statements: Keep away from sources of ignition - no

smoking. In case of contact with eyes, rinse immediately with

plenty of water and seek medical advice. Wear suitable

protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENTANEDIONE (2,4-)    CAS # 123546

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   F   G   .   I   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   2   0

ACUTE TOXICTY RISK INDEX 4 - LD50    55.0 mg/Kg

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION HAZARD

INHALATION RISK INDEX   3.4 - LC50

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

Ingestion: Toxic if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Thymus. Nerves.

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

CONDITIONS AGGRAVATED BY EXPOSURE

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE    6.0 mm Hg @ 20 °C

FLASH POINT  100.4 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC   CORROSIVE

INCOMPATIBILITIES:Strong oxidizing agents, Reducing agents,  Strong bases.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Water may be effective for cooling, but may not    effect extinguishment.

Store at 2-8°C

Keep tightly closed. Keep away from heat, sparks, and

REACTIVE PROPERTIES

HANDLING skin, and clothing. Avoid prolonged or repeated exposure. STORAGE:

Keep tightly closed. Keep away from heat, sparks, and Store at 2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

Risk Statements: Flammable. Harmful if swallowed.

S: 21 23 24/25

Safety Statements: When using do not smoke. Do not breathe

vapor. Avoid contact with skin and eyes.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  15 ppm

DOE Short Term Exposure Limit  50 ppm

DOE Ceiling Limit 100 ppm

Immediately Dangerous to Life and Health   100 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENTANOL (1-)    CAS # 71410

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   2   0

ACUTE TOXICTY RISK INDEX 1.9 - LD50  5660.0 mg/Kg

INHALATION RISK INDEX   1.4 - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Lachrymator. Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. Harmful if inhaled.

Ingestion: May be harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Central nervous system.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE    1.5 mm Hg @ 20 °C

FLASH POINT  120.2 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Carbon Dioxide, dry chemical powder, or appropriate    foam. Water can be

applied as a spray or fog and if properly    applied is capable of

extinguishing the fi

REACTIVE PROPERTIES

HANDLING: Avoid breathing vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 10 20 37 66

Risk Statements: Flammable. Harmful by inhalation. Irritating to

respiratory system. Repeated exposure may cause skin dryness or

cracking.

S: 46

Safety Statements: If swallowed, seek medical advice immediately

and show this container or label.

Immediately Dangerous to Life and Health  500 ppm

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  100 ppm

DOE Short Term Exposure Limit  100 ppm

DOE Ceiling Limit 100 ppm

Immediately Dangerous to Life and Health   100 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENTANOL (2-)    CAS # 6032297

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  0   2   0

ACUTE TOXICTY RISK INDEX 3 - LD50   439.4 mg/Kg

INHALATION HAZARD

INHALATION RISK INDEX   2.6 - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. Harmful if inhaled.

Ingestion: May be harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Central nervous system. Eyes.

SIGNS AND SYMPTOMS OF EXPOSURE

Prolonged exposure can cause: Nausea, headache, and vomiting.

Narcotic effect.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

VAPOR PRESSURE    4.1 mm Hg @ 20 °C

FLASH POINT  93.2 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Water spray.

REACTIVE PROPERTIES

HANDLING: Avoid breathing vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 10 20 37 66

Risk Statements: Flammable. Harmful by inhalation. Irritating to

respiratory system. Repeated exposure may cause skin dryness or

cracking.

S: 46

Safety Statements: If swallowed, seek medical advice immediately

and show this container or label.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  100 ppm

DOE Short Term Exposure Limit  125 ppm

DOE Ceiling Limit 125 ppm

Immediately Dangerous to Life and Health   500 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENTANOL (3-)    CAS # 584021

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   2   0

ACUTE TOXICTY RISK INDEX 2.4 - LD50  1870.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Harmful if inhaled. Material is irritating to mucous

membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Prolonged exposure can cause: Nausea, headache, and vomiting.

Narcotic effect.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

FLASH POINT  93.2 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Water spray.

REACTIVE PROPERTIES

HANDLING: Avoid breathing vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 10 20 37 66

Risk Statements: Flammable. Harmful by inhalation. Irritating to

respiratory system. Repeated exposure may cause skin dryness or

cracking.

S: 46

Safety Statements: If swallowed, seek medical advice immediately

and show this container or label.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  100 ppm

DOE Short Term Exposure Limit  125 ppm

DOE Ceiling Limit 125 ppm

Immediately Dangerous to Life and Health   500 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENTANONE (2-)    CAS # 107879

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   .   F   G   H   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   3   0

ACUTE TOXICTY RISK INDEX 2.4 - LD50  2140.0 mg/Kg

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION HAZARD  INHALATION RISK INDEX   2.7 - LC50  22000.0

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled.

Ingestion: Harmful if swallowed.

Multiple Routes: Vapor or mist is irritating to the eyes, mucous

membranes, and upper respiratory tract.

TARGET ORGAN(S) OR SYSTEM(S)

Nerves. Liver. Heart.

SIGNS AND SYMPTOMS OF EXPOSURE

May cause decrease in blood pressure. CNS depression. Narcotic

effect. Nausea, dizziness, and headache. Exposure to and/or

consumption of alcohol may increase toxic effects.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

VAPOR PRESSURE   27.0 mm Hg @ 20 °C

FLASH POINT  45 °F

Forms ignitable mixtures in air at room temperature - Danger of remote

ignition and flashback

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC   CORROSIVE

INCOMPATIBILITIES:Oxidizing agents, Strong bases, Reducing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Water may be effective for cooling, but may not   effect extinguishment.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep container closed. Keep

away from heat, sparks, and open flame. Store in a cool dry place. Store

under nitrogen\. SPECIAL REQUIREMENTS  Store under inert gas.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: F Xn

Indication of Danger: Highly Flammable. Harmful.

R: 11 22 36/37/38

Risk Statements: Highly flammable. Harmful if swallowed.

Irritating to eyes, respiratory system and skin.

S: 9 16 29 33

Safety Statements: Keep container in a well-ventilated place.

Keep away from sources of ignition - no smoking. Do not empty

into drains. Take precautionary measures against static

discharges.

OSHA REGULATORY LIMITS

OSHA Permissible Exposure Limit 200 ppm

OSHA Short Term Exposure Limit  250 ppm

Immediately Dangerous to Life and Health  1500 ppm

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  200 ppm

DOE Short Term Exposure Limit  250 ppm

DOE Ceiling Limit 250 ppm

Immediately Dangerous to Life and Health   1500 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENTANONE (3-)    CAS # 96220

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   3   0

ACUTE TOXICTY RISK INDEX 2.4 - LD50  2140.0 mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   2.7 - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material is irritating to

mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

VAPOR PRESSURE   28.1 mm Hg @ 20 °C

FLASH POINT  44.6 °F

Forms ignitable mixtures in air at room temperature - Danger of remote

ignition and flashback

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE

INCOMPATIBILITIES:Oxidizing agents, Strong bases.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam

Water may be effective for cooling, but may not   effect extinguishment.

Store at 2-8°C

Keep container closed. Keep away from heat, sparks, and open flame.

REACTIVE PROPERTIES

HANDLING: Avoid contact with eyes, skin, and clothing. Do   not breathe

vapor. Avoid prolonged or repeated exposure. STORAGE: Keep container closed.

Keep away from heat, sparks, and open flame\. Store at 2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: F Xi

Indication of Danger: Highly Flammable. Irritant.

R: 11 37 66 67

Risk Statements: Highly flammable. Irritating to respiratory

system. Repeated exposure may cause skin dryness or cracking.

Vapors may cause drowsiness and dizziness.

S: 9 16 25 33

Safety Statements: Keep container in a well-ventilated place.

Keep away from sources of ignition - no smoking. Avoid contact

with eyes. Take precautionary measures against static discharges.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  200 ppm

DOE Short Term Exposure Limit  300 ppm

DOE Ceiling Limit 300 ppm

Immediately Dangerous to Life and Health   750 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENTENOIC ACID (4-)    CAS # 591800

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  0   2   0

ACUTE TOXICTY RISK INDEX 3 - LD50   470.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin. Causes burns.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  192.2 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases, Strong  reducing

agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

STORAGE: Keep tightly closed. Keep away from heat and open flame. Store in a

cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 22 34

Risk Statements: Harmful if swallowed. Causes burns.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENTOBARBITAL SODIUM    CAS # 57330

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 3.6 - LD50   118.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: Toxic if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Central nervous system.

SIGNS AND SYMPTOMS OF EXPOSURE

Prolonged or repeated exposure can lead to habituation or

addiction. Respiratory depression and sedation. Exposure can

cause: Gastrointestinal disturbances.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: T

Indication of Danger: Toxic.

R: 25

Risk Statements: Toxic if swallowed.

S: 45

Safety Statements: In case of accident or if you feel unwell,

seek medical advice immediately (show the label where possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .15 mg/m3

DOE Short Term Exposure Limit  .5 mg/m3

DOE Ceiling Limit 3.5 mg/m3

Immediately Dangerous to Life and Health   50 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENTYL VALERATE    CAS # 2173560

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   2   0

ACUTE TOXICTY RISK INDEX 1.1 - LD50 35420.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

TOXICITY DATA

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  177.8 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

STORAGE

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 23 24/25

Safety Statements: Do not breathe vapor. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENTYNE (1-)    CAS # 627190

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   3   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

Multiple Routes: Harmful if inhaled or swallowed. Vapor or mist

is irritating to the eyes, mucous membranes, and upper

respiratory tract.

SIGNS AND SYMPTOMS OF EXPOSURE

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

VAPOR PRESSURE  351.6 mm Hg @ 20 °C

FLASH POINT  -4 °F

Store in an explosion-proof refrigerator or tightly stoppered in a

well-ventilated area

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE

INCOMPATIBILITIES:Oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Water may be effective for cooling, but may not    effect extinguishment.

Store at 2-8°C

Keep tightly closed. Keep away from heat, sparks, and

open flame. Store in a cool dry place.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Keep away from heat, sparks, and open flame.

Store in a cool dry place. Store at 2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: F Xn

Indication of Danger: Highly Flammable. Harmful.

R: 11 36/37/38 65

Risk Statements: Highly flammable. Irritating to eyes,

respiratory system and skin. Harmful: may cause lung damage if

swallowed.

S: 16 26 36 62

Safety Statements: Keep away from sources of ignition - no

smoking. In case of contact with eyes, rinse immediately with

plenty of water and seek medical advice. Wear suitable

protective clothing. If swallowed, do not induce vomiting: seek

medical advice immediately and show this container or label.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENTYNOIC ACID (4-)    CAS # 6089094

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin. Causes burns.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Bases, Oxidizing agents, Reducing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Water spray.

Store at 2-8°C

Keep tightly closed. Store in a cool dry place.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place. Store at 2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34

Risk Statements: Causes burns.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PENTYNOL (1:3-)    CAS # 4187864

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   E   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  0   3   0

ACUTE TOXICTY RISK INDEX 3.8 - LD50   225.8 mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

FLASH POINT  85 °F

Forms ignitable mixtures in air at room temperature - Danger of remote

ignition and flashback

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

INCOMPATIBILITIES:Strong acids, Strong oxidizing agents, Strong  reducing

agents, Acid chlorides, Acid anhydrides, Phosphorus    halides.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Water spray.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat, sparks, and open flame. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: T

Indication of Danger: Toxic.

R: 10 23/24/25

Risk Statements: Flammable. Toxic by inhalation, in contact with

skin and if swallowed.

S: 16 36/37/39 45

Safety Statements: Keep away from sources of ignition - no

smoking. Wear suitable protective clothing, gloves, and eye/face

protection. In case of accident or if you feel unwell, seek

medical advice immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PEPPERMINT OIL YAKIMA FCC    CAS # 8006904

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   2   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause: Dermatitis. To the best of our knowledge,

the chemical, physical, and toxicological properties have not

been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  150.8 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at 2-8°C

Keep tightly closed. Keep away from heat and open flame. Store in a cool dry

place.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Keep away from heat and open flame. Store in a

cool dry place. Store at 2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 23 24/25

Safety Statements: Do not breathe vapor. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PEPSIN    CAS # 9001756

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  3   0   0

ACUTE TOXICTY RISK INDEX 2.1 - LD50  4000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material is irritating to

mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SENSITIZATION

Respiratory: May cause allergic respiratory reaction.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Oxidizing agents Incompatible with alcohol,

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at 2-8°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid breathing dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 36/37/38 42

Risk Statements: Irritating to eyes, respiratory system and

skin. May cause sensitization by inhalation.

S: 22 24 26 36/37

Safety Statements: Do not breathe dust. Avoid contact with skin.

In case of contact with eyes, rinse immediately with plenty of

water and seek medical advice. Wear suitable protective clothing

and gloves.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PEPSTATIN    CAS # 26305033

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   0   0

ACUTE TOXICTY RISK INDEX 2.4 - LD50  2000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

CONDITIONS AGGRAVATED BY EXPOSURE

A protease inhibitor. The toxicological properties have not been

thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong acids, Strong bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

Store at 2-8°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. Store at

2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PEPTONE    CAS # 73049737

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  10 mg/m3

DOE Short Term Exposure Limit  30 mg/m3

DOE Ceiling Limit 50 mg/m3

Immediately Dangerous to Life and Health   250 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PERCHLORIC ACID    CAS # 7601903

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   1   1   OX

ACUTE TOXICTY RISK INDEX 2.6 - LD50  1100.0 mg/Kg

INHALATION HAZARD

INHALATION RISK INDEX   2.4 - LC50

ROUTE OF EXPOSURE

skin Contact: Causes severe burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes severe burns.

Inhalation: May be harmful if inhaled. Material is extremely

destructive to the tissue of the mucous membranes and upper

respiratory tract.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE    5.4 mm Hg @ 20 °C

FLASH POINT  235 °F

Oxidizing Agent

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong bases, Strong acids, Amines, Phosphorus halides,

Alcohols, Organic materials, Finely powdered metals,   Strong reducing

agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Chlorine

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from combustible   materials, heat, sparks, and open flame.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: O C

Indication of Danger: Oxidizing. Corrosive.

R: 5 8 35

Risk Statements: Heating may cause an explosion. Contact with

combustible material may cause fire. Causes severe burns.

S: 23 26 36 45

Safety Statements: Do not breathe vapor. In case of contact with

eyes, rinse immediately with plenty of water and seek medical

advice. Wear suitable protective clothing. In case of accident

or if you feel unwell, seek medical advice immediately (show the

label where possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  1 ppm

DOE Short Term Exposure Limit  3.5 ppm

DOE Ceiling Limit 20 ppm

Immediately Dangerous to Life and Health   100 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PERCHLOROMETHYL MERCAPTAN    CAS # 594423

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   E   .   G   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 5 - LD50    82.6 mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   4.2 - LC50     91.8

ROUTE OF EXPOSURE

Multiple Routes: May be fatal if inhaled, swallowed, or absorbed

through skin.

TARGET ORGAN(S) OR SYSTEM(S)

Damage to the liver. Damage to the kidneys.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema   Symptoms of

exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Damage to the liver. Damage to the kidneys. CNS

depression. To the best of our knowledge, the chemical,

physical, and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

VAPOR PRESSURE  104.4 mm Hg @ 20 °C

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Sulfur oxides Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place\. SPECIAL REQUIREMENTS  Moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: T

Indication of Danger: Toxic.

R: 23/24/25 34

Risk Statements: Toxic by inhalation, in contact with skin and

if swallowed. Causes burns.

S: 7/9 26 36/37/39 45

Safety Statements: Keep container tightly closed and in

well-ventilated place. In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

OSHA REGULATORY LIMITS

OSHA Permissible Exposure Limit .1 ppm

ACGIH RECOMMENDED LIMITS

ACGIH Threshold Limit Value .1 ppm

Immediately Dangerous to Life and Health  10 ppm

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .013 ppm

DOE Short Term Exposure Limit  .013 ppm

DOE Ceiling Limit .3 ppm

Immediately Dangerous to Life and Health   .9 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PERCOLL    CAS # 65455529

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

Store at 2-8°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

2-8░C

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  10 mg/m3

DOE Short Term Exposure Limit  30 mg/m3

DOE Ceiling Limit 50 mg/m3

Immediately Dangerous to Life and Health   250 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PERFLUORO-COMPOUND FC-75    CAS # 335364

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen fluoride

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  3.42 mg/m3

DOE Short Term Exposure Limit  10.3 mg/m3

DOE Ceiling Limit 17.1 mg/m3

Immediately Dangerous to Life and Health   75 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PERFLUORODECALIN    CAS # 306945

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   2   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  119 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Water spray.

TOXIC EMISSIONS WHEN BURNED: Hydrogen fluoride

REACTIVE PROPERTIES

HANDLING: Avoid breathing vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure\. Refrigerate before opening. STORAGE:

Keep tightly closed. Keep away from heat and open flame\. SPECIAL

REQUIREMENTS    May develop pressure. Open carefully.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PERFLUOROMETHYL CYCLOHEXANE    CAS # 355022

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

VAPOR PRESSURE   82.7 mm Hg @ 20 °C

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen fluoride

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PERFLUOROMETHYL DIOXANONANE (2H:5:36-)    CAS # 3330141

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen fluoride

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PERFLUOROOCTANESULFONYL FLUORIDE (1-)    CAS # 307357

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be harmful

if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

VAPOR PRESSURE   10.0 mm Hg @ 20 °C

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen fluoride Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame. Store in a cool dry place.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PERFLUROOCTANETHIOL    CAS # 34451268

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Fluorine Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PERHYDROISOQUINOLINE    CAS # 6329619

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   2   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material is irritating to

mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  175 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame\. SPECIAL REQUIREMENTS Handle and store under inert

gas. Air sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36/37/39

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PERINAPHTHENONE    CAS # 548390

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PERIODIC ACID    CAS # 10450609

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   0   1   OX

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Oxidizing Agent

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong reducing agents, Finely powdered metals, Strong

bases Dimethyl sulfoxide. (DMSO)

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen iodide

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from combustible   materials, heat, sparks, and open flame\. SPECIAL

REQUIREMENTS  Light sensitive. Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: O C

Indication of Danger: Oxidizing. Corrosive.

R: 8 34

Risk Statements: Contact with combustible material may cause

fire. Causes burns.

S: 17 26 36/37/39 45

Safety Statements: Keep away from combustible material. In case

of contact with eyes, rinse immediately with plenty of water and

seek medical advice. Wear suitable protective clothing, gloves,

and eye/face protection. In case of accident or if you feel

unwell, seek medical advice immediately (show the label where

possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .18 mg/m3

DOE Short Term Exposure Limit  .18 mg/m3

DOE Ceiling Limit .5 mg/m3

Immediately Dangerous to Life and Health   5 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PERIODINANE    CAS # 87413090

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: Harmful if absorbed through skin.

Eye Contact: Causes eye irritation.

Inhalation: Harmful if inhaled. Material is irritating to mucous

membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

The chemical, physical, and toxicological properties of this

product have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong reducing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen iodide

Store at -20°C

Handle and store under inert gas. May develop pressure. Open carefully. Air

and moisture sensitive.

Keep tightly closed. Keep away from combustible materials, heat, sparks, and

open flame. Handle and

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from combustible    materials, heat, sparks, and open flame. Handle and store

under    nitrogen. Store at -20░C SPECIAL REQUIREMENTS Handle and store under

inert gas. May develop pressure. Open    carefully. Air and moisture

sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 20/21/22 36/37/38

Risk Statements: Harmful by inhalation, in contact with skin and

if swallowed. Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PEROXIDASE    CAS # 9003990

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SENSITIZATION

Sensitization: Prolonged or repeated exposure may cause allergic

reactions in certain sensitive individuals.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Use extinguishing media appropriate to surrounding fire

conditions.

Store at -20°C

Keep container closed. Store away from heat.

REACTIVE PROPERTIES

HANDLING: Avoid contact with eyes, skin, and clothing.    Avoid inhalation.

Avoid prolonged or repeated exposure. STORAGE: Keep container closed. Store

away from heat. Store at -20░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PERYLENE    CAS # 198550

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS Handle and store under inert gas.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 24/25 22

Safety Statements: Avoid contact with skin and eyes. Do not

breathe dust.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PETROLATUM    CAS # 8009038

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   H   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   2   0

ACUTE TOXICTY RISK INDEX 1.1 - LD50 37931.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Flammable

FLASH POINT  100 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE

INCOMPATIBILITIES:Materials to Avoid: S

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

UNITED STATES REGULATORY INFORMATION

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  15 mg/m3

DOE Short Term Exposure Limit  50 mg/m3

DOE Ceiling Limit 350 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PETROLEUM ETHER    CAS # 101316465

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   .   .   .   .   I   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   4   1

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION HAZARD  INHALATION RISK INDEX   3.3 - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: Harmful if absorbed through skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: Harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Central nervous system. Kidneys. Heart. Blood.

SIGNS AND SYMPTOMS OF EXPOSURE

Aspiration or inhalation may cause chemical pneumonitis.

Prolonged or repeated exposure to skin causes defatting and

dermatitis. Exposure can cause: Headache, dizziness, drowsiness,

incoordination, slowed reaction time, slurred speech, giddiness

and unconsciousness. Nausea.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

VAPOR PRESSURE  442.6 mm Hg @ 20 °C

FLASH POINT  -57 °F

Store in an explosion-proof refrigerator or tightly stoppered in a

well-ventilated area

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep container closed. Keep

away from heat, sparks, and open flame. Store in a cool dry place. Handle and

Store under nitrogen\. SPECIAL REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T

Indication of Danger: Toxic.

R: 45 22

Risk Statements: May cause cancer. Harmful if swallowed.

S: 53 45

Safety Statements: Avoid exposure - obtain special instructions

before use. In case of accident or if you feel unwell, seek

medical advice immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PETROLEUM ETHER    CAS # 10316465

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   .   .   .   .   I   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   4   1

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION HAZARD  INHALATION RISK INDEX   3.3 - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: Harmful if absorbed through skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: Harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Central nervous system. Kidneys. Heart. Blood.

SIGNS AND SYMPTOMS OF EXPOSURE

Aspiration or inhalation may cause chemical pneumonitis.

Prolonged or repeated exposure to skin causes defatting and

dermatitis. Exposure can cause: Headache, dizziness, drowsiness,

incoordination, slowed reaction time, slurred speech, giddiness

and unconsciousness. Nausea.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

VAPOR PRESSURE  442.6 mm Hg @ 20 °C

FLASH POINT  -57 °F

Store in an explosion-proof refrigerator or tightly stoppered in a

well-ventilated area

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep container closed. Keep

away from heat, sparks, and open flame. Store in a cool dry place. Handle and

Store under nitrogen\. SPECIAL REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T

Indication of Danger: Toxic.

R: 45 22

Risk Statements: May cause cancer. Harmful if swallowed.

S: 53 45

Safety Statements: Avoid exposure - obtain special instructions

before use. In case of accident or if you feel unwell, seek

medical advice immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PETROLEUM ETHER    CAS # 8030306

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   .   .   .   .   I   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   4   0

ACUTE TOXICTY RISK INDEX 2 - LD50  5000.0 mg/Kg

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION HAZARD  INHALATION RISK INDEX   3.3 - LC50 ********

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: Harmful if absorbed through skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: Harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Central nervous system. Kidneys. Heart. Blood.

SIGNS AND SYMPTOMS OF EXPOSURE

Aspiration or inhalation may cause chemical pneumonitis.

Prolonged or repeated exposure to skin causes defatting and

dermatitis. Exposure can cause: Headache, dizziness, drowsiness,

incoordination, slowed reaction time, slurred speech, giddiness

and unconsciousness. Nausea.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

VAPOR PRESSURE  442.6 mm Hg @ 20 °C

FLASH POINT  -57 °F

Store in an explosion-proof refrigerator or tightly stoppered in a

well-ventilated area

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep container closed. Keep

away from heat, sparks, and open flame. Store in a cool dry place. Handle and

Store under nitrogen\. SPECIAL REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T

Indication of Danger: Toxic.

R: 45 22

Risk Statements: May cause cancer. Harmful if swallowed.

S: 53 45

Safety Statements: Avoid exposure - obtain special instructions

before use. In case of accident or if you feel unwell, seek

medical advice immediately (show the label where possible).

OSHA REGULATORY LIMITS

OSHA Permissible Exposure Limit 100 ppm

ACGIH RECOMMENDED LIMITS

ACGIH Threshold Limit Value 400 ppm

Immediately Dangerous to Life and Health  1000 ppm

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  100 ppm

DOE Short Term Exposure Limit  100 ppm

DOE Ceiling Limit 500 ppm

Immediately Dangerous to Life and Health   1000 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHARMALYTE    CAS # 70852561

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at 2-8°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

2-8░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHE-ARG B-NAPHTHYLAMIDE HYDROCHLORIDE    CAS # 100929995

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   1

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

Store at -20°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

-20░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHE-GLY    CAS # 721904

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

S

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

REACTIVE PROPERTIES

STORAGE

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHE-PRO    CAS # 7669650

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

S

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

Store at -20°C

REACTIVE PROPERTIES

STORAGE Store at -20░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENANTHRENE    CAS # 85018

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 2.3 - LD50  2258.1 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: Harmful if swallowed.

SENSITIZATION

Sensitization: Causes photosensitiveity. Exposure to light can

result in allergic reactions resulting in dermatologic lesions,

which can vary from sunburnlike responses to edematous,

vesiculated lesions, or bullae

SIGNS AND SYMPTOMS OF EXPOSURE

The chemical, physical, and toxicological properties of this

product have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

VAPOR PRESSURE .000300 mm Hg @ 20 °C

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Materials to Avoid: O

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place. Handle    and store under nitrogen\. SPECIAL REQUIREMENTS

Handle and store under inert gas.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn N

Indication of Danger: Harmful. Dangerous for the environment.

R: 22 36/37/38 50

Risk Statements: Harmful if swallowed. Irritating to eyes,

respiratory system and skin. Very toxic to aquatic organisms.

S: 26 60 61

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. This

material and its container must be disposed of as hazardous

waste. Avoid release to the environment. Refer to special

instructions/safety data sheets.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .4 mg/m3

DOE Short Term Exposure Limit  1 mg/m3

DOE Ceiling Limit 7.5 mg/m3

Immediately Dangerous to Life and Health   300 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENANTHRENE-D    CAS # 1517222

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 2.3 - LD50         mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: Harmful if swallowed.

SENSITIZATION

Sensitization: Causes photosensitiveity. Exposure to light can

result in allergic reactions resulting in dermatologic lesions,

which can vary from sunburnlike responses to edematous,

vesiculated lesions, or bullae

SIGNS AND SYMPTOMS OF EXPOSURE

The chemical, physical, and toxicological properties of this

product have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place. Store under nitrogen\. SPECIAL REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn-N

Indication of Danger: Harmful. Dangerous for the environment.

R: 22-36/37/38-50/53

Risk Statements: Harmful if swallowed. Irritating to eyes,

respiratory system and skin. Very toxic to aquatic organisms,

may cause long-term adverse effects in the aquatic environment.

S: 26-60-61

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. This

material and its container must be disposed of as hazardous

waste. Avoid release to the environment. Refer to special

instructions/safety data sheets.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENANTHRENEQUINONE (9,10-)    CAS # 84117

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER: WATER SPRAY.    CARBON DIOXIDE, DRY CHEMICAL POWDER OR

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: IDE

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENANTHRIDINE    CAS # 229878

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes severe eye irritation.

Inhalation: May be harmful if inhaled. Material is irritating to

mucous membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  212 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22 37/38 41

Risk Statements: Harmful if swallowed. Irritating to respiratory

system and skin. Risk of serious damage to eyes.

S: 26 39

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENANTHROLINE (1,10-)    CAS # 5144898

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 3.6 - LD50   132.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: Toxic if swallowed.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Hygroscopic. Store under inert gas.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T N

Indication of Danger: Toxic. Dangerous for the environment.

R: 25 50/53

Risk Statements: Toxic if swallowed. Very toxic to aquatic

organisms, may cause long-term adverse effects in the aquatic

environment.

S: 45 60 6   Safety Statements: In case of accident or if you feel unwell,

seek medical advice immediately (show the label where possible).

This material and its container must be disposed of as hazardous

waste. Avoid release to the environment. Refer to special

instructions/safety data sheets.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENANTHROLINE (1,10-)    CAS # 66717

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: Toxic if swallowed.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Store under inert gas. Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T N

Indication of Danger: Toxic. Dangerous for the environment.

R: 25 50/53

Risk Statements: Toxic if swallowed. Very toxic to aquatic

organisms, may cause long-term adverse effects in the aquatic

environment.

S: 45 60 61

Safety Statements: In case of accident or if you feel unwell,

seek medical advice immediately (show the label where possible).

This material and its container must be disposed of as hazardous

waste. Avoid release to the environment. Refer to special

instructions/safety data sheets.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .1 mg/m3

DOE Short Term Exposure Limit  .3 mg/m3

DOE Ceiling Limit 2 mg/m3

Immediately Dangerous to Life and Health   30 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENANTHROLINE HYDROCHLORIDE MONOHYDRATE    CAS # 3829865

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: Toxic if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: T N

Indication of Danger: Toxic. Dangerous for the environment.

R: 25 50/53

Risk Statements: Toxic if swallowed. Very toxic to aquatic

organisms, may cause long-term adverse effects in the aquatic

environment.

S: 45 60 61

Safety Statements: In case of accident or if you feel unwell,

seek medical advice immediately (show the label where possible).

This material and its container must be disposed of as hazardous

waste. Avoid release to the environment. Refer to special

instructions/safety data sheets.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENAZINE    CAS # 92820

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

a - Organic Base/Flammable/Toxic

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENAZINE METHOSULFATE    CAS # 299116

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Abdominal pain, nausea, vomiting. Headache. To the best of our

knowledge, the chemical, physical, and toxicological properties

have not been thoroughly investigated.

CONDITIONS AGGRAVATED BY EXPOSURE

Causes sneezing. Do not inhale.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Light sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENETOLE    CAS # 103731

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   2   1

ACUTE TOXICTY RISK INDEX 1.8 - LD50  7096.8 mg/Kg

INHALATION RISK INDEX   1.2 - LC50

ROUTE OF EXPOSURE

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. May cause irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE .967 mm Hg @ 20 °C

FLASH POINT  134.6 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Water spray.

REACTIVE PROPERTIES

HANDLING: Avoid breathing vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 23 24/25

Safety Statements: Do not breathe vapor. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENETOLE    CAS # 55685

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   2   0

ACUTE TOXICTY RISK INDEX 2.1 - LD50  7096.8 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. May cause irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  134.6 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Water spray.

REACTIVE PROPERTIES

HANDLING: Avoid breathing vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: T N

Indication of Danger: Toxic. Dangerous for the environment.

R: 25 34 48/24/25 50/53

Risk Statements: Toxic if swallowed. Causes burns. Toxic: danger

of serious damage to health by prolonged exposure in contact

with skin and if swallowed. Very toxic to aquatic organisms, may

cause long-term adverse effects in the aquatic environment.

S: 23 24/25 37 45 60 61

Safety Statements: Do not breathe vapor. Avoid contact with skin

and eyes. Wear suitable gloves. In case of accident or if you

feel unwell, seek medical advice immediately (show the label

where possible). This material and its container must be

disposed of as hazardous waste. Avoid release to the

environment. Refer to special instructions/safety data sheets.

Immediately Dangerous to Life and Health  10 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENOBARBITAL SODIUM    CAS # 57307

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   I   J   K   L

NFPA HAZARD CODES (H,F,R,O)  3   0   0

ACUTE TOXICTY RISK INDEX 3.5 - LD50   150.0 mg/Kg

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: Toxic if swallowed.

SENSITIZATION

skin: May cause allergic skin reaction.

TARGET ORGAN(S) OR SYSTEM(S)

Nerves. Liver. Heart.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Alkali metals, Ammonia, Peroxides.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: T

Indication of Danger: Toxic.

R: 25 40 43

Risk Statements: Toxic if swallowed. Limited evidence of a

carcinogenic effect. May cause sensitization by skin contact.

S: 22 36/37/39 45

Safety Statements: Do not breathe dust. Wear suitable protective

clothing, gloves, and eye/face protection. In case of accident

or if you feel unwell, seek medical advice immediately (show the

label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENOL    CAS # 108952   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   E   .   .   .   I   J   K   L

NFPA HAZARD CODES (H,F,R,O)  4   2   0

REPRODUCTIVE RISK INDEX   2.7

ACUTE TOXICTY RISK INDEX 5 - LD50   410.0 mg/Kg

NEUROTOXIC - RISK INDEX   5.0

INHALATION HAZARD  INHALATION RISK INDEX   1.8 - LC50     54.9

ROUTE OF EXPOSURE

skin Contact: Causes burns. Causes blisters on contact with skin.

skin Absorption: Toxic if absorbed through skin. Readily

absorbed through skin.

Eye Contact: Causes burns.

Inhalation: Toxic if inhaled. Material is extremely destructive

to the tissue of the mucous membranes and upper respiratory

tract.

Ingestion: Toxic if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Central nervous system. Kidneys. Liver. Pancreas. Spleen.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Exposure can cause: Ingestion can cause circulatory

collapse, tachypnea, paralysis, convulsions, coma, necrosis of

mouth and G.I. tract, jaundice, death from respiratory failure,

sometimes from cardiac arrest.

CONDITIONS AGGRAVATED BY EXPOSURE

May cause nervous system disturbances.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

VAPOR PRESSURE .360 mm Hg @ 20 °C

FLASH POINT  174.2 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases, Strong acids.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame. Handle and store under nitrogen\. SPECIAL

REQUIREMENTS Handle and store under inert gas. Light sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T C

Indication of Danger: Toxic. Corrosive.

R: 23/24/25 34 48/20/21/22 68

Risk Statements: Toxic by inhalation, in contact with skin and

if swallowed. Causes burns. Harmful: danger of serious damage to

health by prolonged exposure through inhalation in contact with

skin, and if swallowed. Possible risk of irreversible effects.

S: 24/25 26 28 36/37/39 45

Safety Statements: Avoid contact with skin and eyes. In case of

contact with eyes, rinse immediately with plenty of water and

seek medical advice. After contact with skin, wash immediately

with plenty of soap-suds. Wear suitable protective clothing,

gloves, and eye/face protection. In case of accident or if you

feel unwell, seek medical advice immediately (show the label

where possible).

OSHA REGULATORY LIMITS

OSHA Permissible Exposure Limit 5 ppm

ACGIH RECOMMENDED LIMITS

ACGIH Threshold Limit Value 5 ppm

Immediately Dangerous to Life and Health  250 ppm

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  5 ppm

DOE Short Term Exposure Limit  15 ppm

DOE Ceiling Limit 23 ppm

Immediately Dangerous to Life and Health   200 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENOL RED    CAS # 143748

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material is irritating to

mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENOL RED SODIUM SALT    CAS # 34487611

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam\. Protective Equipment: Wear self-contained breathing

apparatus and protective clothing to prevent contact with skin and eyes.

Specific Hazard(s): Emits t

TOXIC EMISSIONS WHEN BURNED: Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, STORAGE: Keep tightly

closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  7.5 mg/m3

DOE Short Term Exposure Limit  25 mg/m3

DOE Ceiling Limit 150 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENOLPHTHALEIN    CAS # 77098

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   C   .   .   .   .   .   .   J   K   L

NFPA HAZARD CODES (H,F,R,O)  2   0   0

EXTREMELY TOXIC - DESIGNATED AREA MAY BE REQUIRED

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SENSITIZATION

Sensitization: Prolonged or repeated exposure may cause allergic

reactions in certain sensitive individuals.

TARGET ORGAN(S) OR SYSTEM(S)

Adrenal medulla. Kidneys. Thymus. Bowel.

SIGNS AND SYMPTOMS OF EXPOSURE

Exerts regulatory influence on metabolism of electrolytes and

water. Exposure can cause: Diarrhea, upset stomach, weakness.

Electrolyte imbalance.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

d - Organic Acid/Flammable/Toxic

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 40

Risk Statements: Limited evidence of a carcinogenic effect.

S: 36/37

Safety Statements: Wear suitable protective clothing and gloves.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .75 mg/m3

DOE Short Term Exposure Limit  2.5 mg/m3

DOE Ceiling Limit 15 mg/m3

Immediately Dangerous to Life and Health   400 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENOLPHTHALEIN CARBINOL DUSULFATE TRIPOTASSIUM    CAS # 62625165

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. May cause irritation.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

STORAGE

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENOLPHTHALEIN DIPHOSPHATE PYRIDINE    CAS # 2090826

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. May cause irritation.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at 2-8°C

REACTIVE PROPERTIES

STORAGE   Store at 2-8░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENOLPHTHALEIN DIPHOSPHATE TETRASODIUM    CAS # 68807909

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER: WATER SPRAY.    CARBON DIOXIDE, DRY CHEMICAL POWDER OR

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: ON MONOXIDEPHOSPHORUS  HAZA

REACTIVE PROPERTIES

VIOLENT CONDENSATION REACTION WITH ACETALDEHYDE. EXPLOSIVE WHEN MIXED

WITHMAGNESIUM.VIOLENT CONDENSATION REACTION WITH ACETALDEHYDE. EXPLOSIVE

WHENMIXED WITH MAGNESIUM.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENOLPHTHALEIN DIPHOSPHATE TETRASODIUM HYDRATE    CAS # 123334098

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENOLPHTHALEIN GLUCURONIC ACID    CAS # 108448299

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at -20°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

-20░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENOLPHTHALEIN GLUCURONIC ACID    CAS # 15265266

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at -20°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. Store at

-20°C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENOTHIAZINE    CAS # 92842

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  3   0   0

ACUTE TOXICTY RISK INDEX 2 - LD50  5000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SENSITIZATION

Sensitization: Causes photosensitiveity. Exposure to light can

result in allergic reactions resulting in dermatologic lesions,

which can vary from sunburnlike responses to edematous,

vesiculated lesions, or bullae

skin: May cause allergic skin reaction.

TARGET ORGAN(S) OR SYSTEM(S)

Liver. Blood. Kidneys.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause: Anemia. Discoloration of the skin.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

a - Organic Base/Flammable/Toxic

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Air and light sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38 43

Risk Statements: Irritating to eyes, respiratory system and

skin. May cause sensitization by skin contact.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENOXAZINE    CAS # 135671

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENOXYACETAMIDE    CAS # 621885

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong acids, Strong bases, Strong oxidizing agents, Strong

reducing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure.STORAGE: Keep container closed. Store in

a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 20/21/22 36/37/38

Risk Statements: Harmful by inhalation, in contact with skin and

if swallowed. Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENOXYACETYL CHLORIDE    CAS # 701995   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   1   W

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin. Causes burns.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  226.4 °F

Reacts violently with water

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   REACTIVE

INCOMPATIBILITIES:Water, Alcohols, Oxidizing agents, Strong  bases.

FIRE EXTINGUISHER:  Carbon dioxide. Dry chemical powder.  Do not use water.

TOXIC EMISSIONS WHEN BURNED: Hydrogen chloride gas Phosgene gas

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place\. SPECIAL REQUIREMENTS  Moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 14 34 36/37

Risk Statements: Reacts violently with water. Causes burns.

Irritating to eyes and respiratory system.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENOXYETHANOL (2-)    CAS # 122996   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   2

ACUTE TOXICTY RISK INDEX 2.6 - LD50  1260.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes severe eye irritation   Inhalation: Material may be

irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

Explosive

VAPOR PRESSURE .0110 mm Hg @ 20 °C

FLASH POINT  249.8 °F

Forms explosive unstable peroxides

Test periodically or discard

SEGREGATION: SHELF # 3

STORAGE GROUP(S):

k - Explosive/Unstable

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   REACTIVE

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Store under inert gas.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22 36

Risk Statements: Harmful if swallowed. Irritating to eyes.

S: 26

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  20 ppm

DOE Short Term Exposure Limit  20 ppm

DOE Ceiling Limit 20 ppm

Immediately Dangerous to Life and Health   100 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENOXYMETHYLPENICILLENIC ACID    CAS # 87081

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 1.8 - LD50  8000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

d - Organic Acid/Flammable/Toxic

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER: NONCOMBUSTIBLE.    USE EXTINGUISHING MEDIA APPROPRIATE TO

SURROUNDING FIRE CONDITIONS.

TOXIC EMISSIONS WHEN BURNED: SULFUR OXIDESHAZARDOUS POLYMERIZATION

REACTIVE PROPERTIES

MINOR AMOUNTS OF ACID CAUSE ACROLEIN TO POYLMERIZE WITH RELEASE OF

HEAT.BENZYL ALCOHOL CONTAINING ACIDIC CONSTITUENTS AND DISSOLVED IRON WAS

FOUND TOPOLYMERIZE WITH A RAPID TEMPERATURE INCREASE WHEN HEATED IN EXCESS

OF100C.(AMINES, PYRIDINE, AND ALKALI HYDROXIDES ACT AS INHIBITORS AND

PREVENTPOLYMERIZATION.) REACT VIGOROUSLY WITH LITHIUM ALUMINUM HYDRIDE.

HIGHCONCENTRATIONS OF ACID REACT EXPLOSIVELY WITH NICKEL NITRIDE AND

HEAT.INITIATE A FAST DECOMPOSITION OF SODIUM OZONATE. REACTS WITH

THORIUMPHOSPHIDE TO RELEASE SPONTANEOUSLY FLAMMABLE PHSOPHINE. REACTS

VIOLENTLY WITHTRI-ISO-BUTYL ALUMINUM.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENOXYMETHYLPENICILLINIC ACID    CAS # 132989

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  1   0   0

ACUTE TOXICTY RISK INDEX 2.7 - LD50  1040.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: Harmful if swallowed.

SENSITIZATION

Sensitization: May cause allergic reaction.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Potassium oxides Sulfur oxides  Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22-42/43

Risk Statements: Harmful if swallowed. May cause sensitization

by inhalation and skin contact.

S: 26-36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENOXYPHENYLBORONIC ACID (4-)    CAS # 51067380

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Boron oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYL ACETATE    CAS # 122792

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   2   0

ACUTE TOXICTY RISK INDEX 2.5 - LD50  1630.0 mg/Kg

INHALATION RISK INDEX   1.3 - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE .275 mm Hg @ 20 °C

FLASH POINT  170 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids, Strong bases, Strong

reducing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid breathing vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22

Risk Statements: Harmful if swallowed.   S: 36

Safety Statements: Wear suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYL BENZOATE    CAS # 93992

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 2.1 - LD50  3951.6 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

VAPOR PRESSURE .002400 mm Hg @ 20 °C

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids, Strong bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Do not get in eyes, on skin, on   clothing

STORAGE: Keep tightly closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22 38

Risk Statements: Harmful if swallowed. Irritating to skin.

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYL CHLOROFORMATE    CAS # 1885149

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   E   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   2   0

ACUTE TOXICTY RISK INDEX 2.5 - LD50  1410.0 mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   3.1 - LC50

ROUTE OF EXPOSURE

Multiple Routes: May be fatal if inhaled, swallowed, or absorbed

through skin. Causes burns.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE   63.1 mm Hg @ 20 °C

FLASH POINT  156.2 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Bases, Strong bases, Alcohols, Amines.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Do not use water.

TOXIC EMISSIONS WHEN BURNED: Hydrogen chloride gas

Store at 2-8°C

Keep tightly closed. Keep away from heat and open flame. Store under

nitrogen.

May develop pressure. Open carefully. Moisture sensitive.

REACTIVE PROPERTIES

HANDLING: Open carefully. Do not breathe vapor. Do not get    in eyes, on

skin, on clothing. Avoid prolonged or repeated    exposure. STORAGE: Keep

tightly closed. Keep away from heat and open flame. Store under nitrogen.

Store at 2-8░C SPECIAL REQUIREMENTS    May develop pressure. Open carefully.

Moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: T+

Indication of Danger: Very toxic.

R: 26 34

Risk Statements: Very toxic by inhalation. Causes burns.

S: 26 28 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. After

contact with skin, wash immediately with plenty of water. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .04 ppm

DOE Short Term Exposure Limit  .125 ppm

DOE Ceiling Limit .75 ppm

Immediately Dangerous to Life and Health   4 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYL CHLOROTHIONOFORMATE    CAS # 1005567

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   2   1

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  177.8 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Sulfur oxides Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame. Store in a cool dry place. Store under nitrogen\.

SPECIAL REQUIREMENTS  Moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34

Risk Statements: Causes burns.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYL DICHLOROPHOSPHATE    CAS # 770127   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  0   1   1   W

ACUTE TOXICTY RISK INDEX 2.8 - LD50   850.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin. Causes burns.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE .0334 mm Hg @ 20 °C

FLASH POINT  235 °F

Reacts violently with water

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   REACTIVE   TOXIC

INCOMPATIBILITIES:Strong bases, Strong oxidizing agents, Alcohols.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Do not use water.

TOXIC EMISSIONS WHEN BURNED: Hydrogen chloride gas

Store at 2-8°C

Moisture sensitive.

Keep tightly closed. Store in a cool dry place.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Handle under nitrogen. STORAGE: Keep tightly closed. Store in a cool dry

place\. Store at 2-8░C SPECIAL REQUIREMENTS  Moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34 37

Risk Statements: Causes burns. Irritating to respiratory system.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYL DISULFIDE    CAS # 882337

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

VAPOR PRESSURE .000300 mm Hg @ 20 °C

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYL ETHANEDIOL (1,2-)    CAS # 16355003

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Acids, Acid chlorides, Acid anhydrides, Oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Use protective clothing, gloves, and mask. Avoid   contact and

inhalation. STORAGE: Store in a cool dry place.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYL ISOCYANATE    CAS # 103719

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   .   .   G   .   I   J   K   L

NFPA HAZARD CODES (H,F,R,O)  3   2   0

ACUTE TOXICTY RISK INDEX 5 - LD50   800.0 mg/Kg

NEUROTOXIC - RISK INDEX   5.0

INHALATION HAZARD  INHALATION RISK INDEX   2.0 - LC50     22.0

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Lachrymator. Causes burns.

Inhalation: May be fatal if inhaled. Material is extremely

destructive to the tissue of the mucous membranes and upper

respiratory tract.

Ingestion: Harmful if swallowed.

SENSITIZATION

Respiratory: May cause allergic respiratory reaction.

Sensitization: Prolonged or repeated exposure may cause allergic

reactions in certain sensitive individuals.

TARGET ORGAN(S) OR SYSTEM(S)

Blood. Central nervous system.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause: Cyanosis. To the best of our knowledge, the

chemical, physical, and toxicological properties have not been

thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE    1.4 mm Hg @ 20 °C

FLASH POINT  123.8 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

INCOMPATIBILITIES:Water, Alcohols, Strong bases, Amines, Acids, Strong

oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide. Dry chemical powder.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. Note:   cloudiness or precipitation is

unavoidable and should not affect   use. STORAGE: Keep tightly closed. Keep

away from heat and open flame. Store under nitrogen\. SPECIAL REQUIREMENTS

Store under inert gas. Moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: T+

Indication of Danger: Very toxic.

R: 10 22 26 34 42 52

Risk Statements: Flammable. Harmful if swallowed. Very toxic by

inhalation. Causes burns. May cause sensitization by inhalation.

Harmful to aquatic organisms.

S: 16 26 28 36/37/39 45

Safety Statements: Keep away from sources of ignition - no

smoking. In case of contact with eyes, rinse immediately with

plenty of water and seek medical advice. After contact with

skin, wash immediately with plenty of water. Wear suitable

protective clothing, gloves, and eye/face protection. In case of

accident or if you feel unwell, seek medical advice immediately

(show the label where possible).

Immediately Dangerous to Life and Health  25 mg/m3

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .3 mg/m3

DOE Short Term Exposure Limit  .75 mg/m3

DOE Ceiling Limit 2 mg/m3

Immediately Dangerous to Life and Health   2 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYL ISOTHIOCYANATE    CAS # 103720

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   G   .   I   J   K   L

NFPA HAZARD CODES (H,F,R,O)  3   2   1

ACUTE TOXICTY RISK INDEX 3.2 - LD50   280.6 mg/Kg

NEUROTOXIC - RISK INDEX   5.0

INHALATION RISK INDEX   1.6 - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns. Lachrymator.

Inhalation: May be harmful if inhaled. Material is extremely

destructive to the tissue of the mucous membranes and upper

respiratory tract.

Ingestion: Toxic if swallowed.

SENSITIZATION

Sensitization: May cause allergic respiratory and skin reactions

TARGET ORGAN(S) OR SYSTEM(S)

Kidneys. Bladder.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Repeated exposure may cause asthma. Prolonged contact

can cause: Lung irritation, chest pain, and edema which may be

fatal. To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE .123 mm Hg @ 20 °C

FLASH POINT  190 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Water, Alcohols, Strong bases, Amines, Acids,  Strong

oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Store at 2-8°C

Moisture sensitive.

Keep tightly closed. Keep away from heat, sparks, and open flame.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. Note:    cloudiness or precipitation is

unavoidable and should not affect    use. STORAGE: Keep tightly closed. Keep

away from heat, sparks, and open flame. Store at 2-8░C SPECIAL REQUIREMENTS

Moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: T N

Indication of Danger: Toxic. Dangerous for the environment.

R: 10 25 34 42/43 51/53

Risk Statements: Flammable. Toxic if swallowed. Causes burns.

May cause sensitization by inhalation and skin contact. Toxic to

aquatic organisms, may cause long-term adverse effects in the

aquatic environment.

S: 16 23 36/37/39 45 60 61

Safety Statements: Keep away from sources of ignition - no

smoking. Do not breathe vapor. Wear suitable protective

clothing, gloves, and eye/face protection. In case of accident

or if you feel unwell, seek medical advice immediately (show the

label where possible). This material and its container must be

disposed of as hazardous waste. Avoid release to the

environment. Refer to special instructions/safety data sheets.

Immediately Dangerous to Life and Health  25 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYL PHENYLPHOSPHORAMIDOCHLORIDATE (N-)    CAS # 51766213   HAZARDOUS

CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Avoid contact and inhalation. Do not get in eyes, on skin, on

clothing. STORAGE: Keep tightly closed. Store in a cool dry place. Store

under nitrogen\. SPECIAL REQUIREMENTS  Moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYL PROPANEDIONE (1:1,2-)    CAS # 579077

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   2   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material is irritating to

mucous membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  184 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 36/37/38 22

Risk Statements: Irritating to eyes, respiratory system and

skin. Harmful if swallowed.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  2.5 mg/m3

DOE Short Term Exposure Limit  7.5 mg/m3

DOE Ceiling Limit 50 mg/m3

Immediately Dangerous to Life and Health   250 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYL PROPIONALDEHYDE (2-)    CAS # 93538

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   2   0

ACUTE TOXICTY RISK INDEX 2.2 - LD50  2800.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  169 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Water spray.

REACTIVE PROPERTIES

HANDLING: Avoid breathing vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame. Store in a cool dry place. Store under nitrogen.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36/37/39

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYL PROPYLAMINE (3:1-)    CAS # 2038575

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   2   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  195.8 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Acids, Acid chlorides, Acid anhydrides, Strong  oxidizing

agents Carbon dioxide.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Water spray.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame. Store in a cool dry place.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYL PROPYNE (1:1-)    CAS # 673325

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   2   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Vapor or mist is irritating to the eyes, mucous

membranes, and upper respiratory tract.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  144 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

STORAGE: Keep tightly closed. Keep away from heat and open flame. Store at

2-8░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYL SALICYLATE    CAS # 118558

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  2   1   0

ACUTE TOXICTY RISK INDEX 2.2 - LD50  3000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SENSITIZATION

Sensitization: Prolonged or repeated exposure may cause allergic

reactions in certain sensitive individuals.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  235 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Light sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  4 mg/m3

DOE Short Term Exposure Limit  12.5 mg/m3

DOE Ceiling Limit 75 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYL SEMICARBAZIDE (1-)    CAS # 537473

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYL TRIFLUOROMETHYL SULPHIDE    CAS # 456564

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   2   0

INHALATION HAZARD  INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause: Nausea, dizziness, and headache.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

FLASH POINT  100 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Water spray.

TOXIC EMISSIONS WHEN BURNED: Sulfur oxides Hydrogen fluoride

REACTIVE PROPERTIES

HANDLING: Avoid breathing vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

R: 10

Risk Statements: Flammable.

S: 16 36

Safety Statements: Keep away from sources of ignition - no

smoking. Wear suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYL TRIMETHYLSILYLOXY ETHYLENE (1:1-)    CAS # 13735814   HAZARDOUS

CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   2   1   W

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Vapor or mist is irritating to the eyes, mucous

membranes, and upper respiratory tract.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  168.8 °F

Reacts violently with water

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   REACTIVE

INCOMPATIBILITIES:Oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide. Dry chemical powder. Water spray.

TOXIC EMISSIONS WHEN BURNED: Silicon oxide

Store at 2-8°C

Keep tightly closed. Keep away from heat and open flame.

Moisture sensitive.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame. Store at 2-8░C SPECIAL REQUIREMENTS  Moisture

sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYL VINYL SULFONE    CAS # 5535488

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SENSITIZATION

skin: May cause allergic skin reaction.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

CONDITIONS AGGRAVATED BY EXPOSURE

Causes sneezing. Do not inhale.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Do not get in eyes, on skin, on    clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38 43

Risk Statements: Irritating to eyes, respiratory system and

skin. May cause sensitization by skin contact.

S: 26 36/37/39

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYL VINYL SULFOXIDE    CAS # 20451530

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  230 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 24/25

Safety Statements: Avoid contact with skin and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYL-SEPHAROSE    CAS # 69106958

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER: WATER SPRAY.    CARBON DIOXIDE, DRY CHEMICAL POWDER OR

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: ATURE OF DECOMPOSITION PRODUCTS NOT KNOWNHAZA

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYL-SEPHAROSE(R)CL-4B    CAS # 69106598

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   C   .   .   .   .   .   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   3   0

EXTREMELY TOXIC - DESIGNATED AREA MAY BE REQUIRED

ACUTE TOXICTY RISK INDEX 1 - LD50         mg/Kg

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50 ********

ROUTE OF EXPOSURE

Eye Contact: May cause eye irritation.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Flammable

VAPOR PRESSURE   40.0 mm Hg @ 20 °C

FLASH POINT  96.8 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

Store at 2-8°C

REACTIVE PROPERTIES

STORAGE   Store at 2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 24/25

Safety Statements: Avoid contact with skin and eyes.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  10 mg/m3

DOE Short Term Exposure Limit  30 mg/m3

DOE Ceiling Limit 50 mg/m3

Immediately Dangerous to Life and Health   250 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLACETALDEHYDE    CAS # 122781

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   F   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   1   0

ACUTE TOXICTY RISK INDEX 1.1 - LD50 43000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  240.8 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   CORROSIVE

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Water spray\. Protective Equipment: Wear self-contained breathing apparatus

and protective clothing to prevent contact with skin and eyes.    Specific

Hazard(s): Emits t

Store at 2-8°C

Keep tightly closed. Keep away from heat and open flame.

REACTIVE PROPERTIES

User Exposure: Do not breathe vapor. Avoid contact with eyes, skin, and

clothing. Avoid prolonged or repeated exposure. STORAGE    flame. Store at

2-8░C SPECIAL REQUIREMENTS

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22 36/37/38

Risk Statements: Harmful if swallowed. Irritating to eyes,

respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLACETALDEHYDE DIMETHYL ACETAL    CAS # 101484   HAZARDOUS CHEMICAL OF

CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   2   2

ACUTE TOXICTY RISK INDEX 2.2 - LD50  3500.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

Explosive

FLASH POINT  192.2 °F

Forms explosive unstable peroxides

Test periodically or discard

SEGREGATION: SHELF # 3

STORAGE GROUP(S):

k - Explosive/Unstable

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   REACTIVE

INCOMPATIBILITIES:Acids, Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Do not use water.

REACTIVE PROPERTIES

HANDLING: Avoid breathing vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame\. SPECIAL REQUIREMENTS  Moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 23 24/25

Safety Statements: Do not breathe vapor. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLACETIC ACID    CAS # 103822

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   1   1

ACUTE TOXICTY RISK INDEX 2.3 - LD50  2250.0 mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   1.1 - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause: Nausea, dizziness, and headache. To the best

of our knowledge, the chemical, physical, and toxicological

properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

VAPOR PRESSURE .210 mm Hg @ 20 °C

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

d - Organic Acid/Flammable/Toxic

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases, Strong  reducing

agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLACETIC ACID BENZYL ESTER    CAS # 102169

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   2   1

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  200 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLACETONE    CAS # 103797

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  0   2   1

ACUTE TOXICTY RISK INDEX 3 - LD50   550.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  194 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid breathing vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLACETYL CHLORIDE    CAS # 103800   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   1   W

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin. Causes burns.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE .133 mm Hg @ 20 °C

FLASH POINT  215.6 °F

Reacts violently with water

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   REACTIVE

INCOMPATIBILITIES:Water, Alcohols, Oxidizing agents, Strong  bases.

FIRE EXTINGUISHER:  Carbon dioxide. Dry chemical powder.  Do not use water.

TOXIC EMISSIONS WHEN BURNED: Hydrogen chloride gas Phosgene gas

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34 37

Risk Statements: Causes burns. Irritating to respiratory system.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLACETYLENE    CAS # 536743

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   C   D   .   .   G   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   3   0

EXTREMELY TOXIC - DESIGNATED AREA MAY BE REQUIRED

ACUTE TOXICTY RISK INDEX 4.7 - LD50    10.0 mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   4.1 - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation. Lachrymator.

Inhalation: May be harmful if inhaled. Material is irritating to

mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Liver. Kidneys. Central nervous system.

SIGNS AND SYMPTOMS OF EXPOSURE

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

VAPOR PRESSURE    9.3 mm Hg @ 20 °C

FLASH POINT  80.6 °F

Forms ignitable mixtures in air at room temperature - Danger of remote

ignition and flashback

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

INCOMPATIBILITIES:Acids, Oxidizing agents, Halogens, Alkali  metals.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Carbon Dioxide, dry chemical powder, or appropriate    foam. Water can be

applied as a spray or fog and if properly    applied is capable of

extinguishing the fi

Store at 2-8°C

Keep container closed. Keep away from heat, sparks, and open flame.

Moisture sensitive.

REACTIVE PROPERTIES

HANDLING: Avoid breathing vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep container closed. Keep

away from heat, sparks, and open flame. Store at 2-8░C SPECIAL REQUIREMENTS

Moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 10 36/37/38 40 65

Risk Statements: Flammable. Irritating to eyes, respiratory

system and skin. Limited evidence of a carcinogenic effect.

Harmful: may cause lung damage if swallowed.

S: 16 26 36/37/39 45

Safety Statements: Keep away from sources of ignition - no

smoking. In case of contact with eyes, rinse immediately with

plenty of water and seek medical advice. Wear suitable

protective clothing, gloves, and eye/face protection. In case of

accident or if you feel unwell, seek medical advice immediately

(show the label where possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  40 mg/m3

DOE Short Term Exposure Limit  100 mg/m3

DOE Ceiling Limit 500 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLALANINE (3-)    CAS # 63912

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

CONDITIONS AGGRAVATED BY EXPOSURE

Warning! To persons with phenylketonuria! This material can

release phenylalanine

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

d - Organic Acid/Flammable/Toxic

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLALANINE (BOC,L-)    CAS # 13734344

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   F   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   CORROSIVE

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

User Exposure: Avoid inhalation. Avoid contact with eyes, skin, and

clothing. Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLALANINE (D-)    CAS # 673063

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX .2 - LD50******** mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Acids, Bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid breathing dust. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep container closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLALANINE (DL-)    CAS # 150301

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX .2 - LD50******** mg/Kg

INHALATION RISK INDEX   <1  - LC50

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure.     Suitable: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLALANINE (FMOC,L-)    CAS # 35661406

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLALANINE BUTYL ESTER HYDROCHLORIDE (TERT-)    CAS # 15100751

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER: appropriate foam.

Store at -20°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure.STORAGE: Keep tightly closed. Store at

-20░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLALANINE DEAMINASE    CAS # 9024286

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SENSITIZATION

Sensitization: Prolonged or repeated exposure may cause allergic

reactions in certain sensitive individuals.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxides Hydrogen chloride

gas

Store at -20°C

REACTIVE PROPERTIES

STORAGE Store at -20░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 24/25

Safety Statements: Avoid contact with skin and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLALANINE ETHYL ESTER (L-)    CAS # 3182932

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLALANINE METHYL ESTER (L-)    CAS # 7524507

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

Store at -20°C

Keep tightly closed. Store in a cool dry place.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place. Store at -20░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLALANINE METHYL ESTER HYDROCHLORIDE (DL-)    CAS # 5619078

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

S

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

Store at 2-8°C

REACTIVE PROPERTIES

STORAGE Store at 2-8░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLANTHRACENE (9-)    CAS # 602551

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin   andclothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLAZOBENZYL CHLORIDE    CAS # 104245

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   1

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34

Risk Statements: Causes burns.

S: 26 27 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Take

off immediately all contaminated clothing. Wear suitable

protective clothing, gloves, and eye/face protection. In case of

accident or if you feel unwell, seek medical advice immediately

(show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLBENZIMIDAZOLE (2-)    CAS # 716790

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes severe eye irritation.

Inhalation: May be harmful if inhaled. Material is irritating to

mucous membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

The chemical, physical, and toxicological properties of this

product have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22 37/38 41

Risk Statements: Harmful if swallowed. Irritating to respiratory

system and skin. Risk of serious damage to eyes.

S: 26 39

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLBENZOQUINONE    CAS # 363031

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SENSITIZATION

skin: May cause allergic skin reaction.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Do not get in eyes, on skin, on   clothing. Avoid

prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in a cool

dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 20/21/22 36/37/38 42/43

Risk Statements: Harmful by inhalation, in contact with skin and

if swallowed. Irritating to eyes, respiratory system and skin.

May cause sensitization by inhalation and skin contact.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLBENZYLAMINE (N-)    CAS # 103322

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   1

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  235 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Acids, Acid chlorides, Acid anhydrides, Oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

STORAGE: Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 37/39

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable gloves and eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLBIGUANIDE HYDROCHLORIDE (1-)    CAS # 55572

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 2.1 - LD50  3871.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLBISTRIFLUOROMETHANESULPHONIMIDE (N-)    CAS # 37595747

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxides Hydrogen fluoride

REACTIVE PROPERTIES

HANDLING: Avoid contact and inhalation. Do not get in eyes, on skin, on

clothing. STORAGE: Keep tightly closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLBORIC ACID    CAS # 98806

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

ACUTE TOXICTY RISK INDEX 2.8 - LD50   740.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

d - Organic Acid/Flammable/Toxic

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Boron oxides

REACTIVE PROPERTIES

HANDLING: Avoid breathing dust. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Store in a cool dry place\. SPECIAL

REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22

Risk Statements: Harmful if swallowed.

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  3 mg/m3

DOE Short Term Exposure Limit  7.5 mg/m3

DOE Ceiling Limit 60 mg/m3

Immediately Dangerous to Life and Health   300 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLBUTANONE (1,2-)    CAS # 1007325

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   2   1

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. May cause irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  194 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases, Strong reducing

agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Water spray.

Store at 2-8°C

Keep tightly closed. Keep away from heat and open flame. Store in a cool dry

place.

REACTIVE PROPERTIES

HANDLING: Avoid breathing vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame. Store in a cool dry place\. Store at 2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 23 24/25

Safety Statements: Do not breathe vapor. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLCINNAMIC ACID    CAS # 91485

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

d - Organic Acid/Flammable/Toxic

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLCYCLOPENTANEACETIC ACID    CAS # 3900934

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

d - Organic Acid/Flammable/Toxic

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Materials to Avoid: S

FIRE EXTINGUISHER: appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure.STORAGE: Keep tightly closed. Store in a

cool dry place.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLDICHLOROBORANE    CAS # 873518   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   3   1   W

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin. Causes burns.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

FLASH POINT  57.2 °F

Forms ignitable mixtures in air at room temperature - Danger of remote

ignition and flashback

Reacts violently with water

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   REACTIVE

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen chloride gas Boron oxides

REACTIVE PROPERTIES

User Exposure: Do not breathe vapor. Do not get in eyes, on skin, on

clothing. Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

Keep away from heat, sparks, and open flame. Store in a cool dry place.

Handle and store under   nitrogen\. SPECIAL REQUIREMENTS  Moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: F C

Indication of Danger: Highly Flammable. Corrosive.

R: 11 34 37

Risk Statements: Highly flammable. Causes burns. Irritating to

respiratory system.

S: 16 26 36/37/39 45

Safety Statements: Keep away from sources of ignition - no

smoking. In case of contact with eyes, rinse immediately with

plenty of water and seek medical advice. Wear suitable

protective clothing, gloves, and eye/face protection. In case of

accident or if you feel unwell, seek medical advice immediately

(show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLDIETHANOLAMINE    CAS # 120070

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  1   1   0

ACUTE TOXICTY RISK INDEX 2.7 - LD50   980.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material is irritating to

mucous membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

VAPOR PRESSURE .010000 mm Hg @ 20 °C

FLASH POINT  392 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Acids, Acid chlorides, Acid anhydrides, Oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place. Store under nitrogen\. SPECIAL REQUIREMENTS  Store under

inert gas. Light sensitive. Air and moisture   sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22 41 37/38

Risk Statements: Harmful if swallowed. Risk of serious damage to

eyes. Irritating to respiratory system and skin.

S: 26 36/37/39

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLDODECANE (1-)    CAS # 123013

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   F   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   1   0

ACUTE TOXICTY RISK INDEX 1.7 - LD50 10000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  228.2 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   CORROSIVE

INCOMPATIBILITIES:Nitric acid, Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLENEDIAMINE (1,2-)    CAS # 95545

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   .   J   K   L

NFPA HAZARD CODES (H,F,R,O)  3   1   0

ACUTE TOXICTY RISK INDEX 5 - LD50  1070.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50     28.2

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: Harmful if absorbed through skin.

Eye Contact: Causes eye irritation.

Inhalation: Harmful if inhaled. Material may be irritating to

mucous membranes and upper respiratory tract.

Ingestion: Toxic if swallowed.

SENSITIZATION

skin: May cause allergic skin reaction.

TARGET ORGAN(S) OR SYSTEM(S)

Bladder. Liver. Kidneys.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

VAPOR PRESSURE .010000 mm Hg @ 20 °C

FLASH POINT  277 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

a - Organic Base/Flammable/Toxic

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

Store at 2-8°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T N

Indication of Danger: Toxic. Dangerous for the environment.

R: 20/21 25 36 40 43 50/53 68

Risk Statements: Harmful by inhalation and in contact with skin.

Toxic if swallowed. Irritating to eyes. Limited evidence of a

carcinogenic effect. May cause sensitization by skin contact.

Very toxic to aquatic organisms, may cause long-term adverse

effects in the aquatic environment. Possible risk of

irreversible effects.

S: 28 36/37 45 60 61

Safety Statements: After contact with skin, wash immediately

with plenty of soap-suds. Wear suitable protective clothing and

gloves. In case of accident or if you feel unwell, seek medical

advice immediately (show the label where possible). This

material and its container must be disposed of as hazardous

waste. Avoid release to the environment. Refer to special

instructions/safety data sheets.

ACGIH RECOMMENDED LIMITS

ACGIH Threshold Limit Value .1 mg/m3

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .1 mg/m3

DOE Short Term Exposure Limit  .3 mg/m3

DOE Ceiling Limit 200 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLENEDIAMINE (1,4-)    CAS # 106503   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   E   F   G   .   .   J   K   L

NFPA HAZARD CODES (H,F,R,O)  3   1   0

ACUTE TOXICTY RISK INDEX 4 - LD50    80.0 mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   2.2 - LC50    920.0

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: Toxic if absorbed through skin.

Eye Contact: Causes eye irritation.

Inhalation: Toxic if inhaled. Material may be irritating to

mucous membranes and upper respiratory tract.

Ingestion: Toxic if swallowed.

SENSITIZATION

skin: May cause allergic skin reaction.

TARGET ORGAN(S) OR SYSTEM(S)

Liver. Kidneys.

SIGNS AND SYMPTOMS OF EXPOSURE

Absorption into the body leads to the formation of methemoglobin

which in sufficient concentration causes cyanosis. Onset may be

delayed 2 to 4 hours or longer.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

VAPOR PRESSURE .220 mm Hg @ 20 °C

FLASH POINT  230 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

a - Organic Base/Flammable/Toxic

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC   CORROSIVE

INCOMPATIBILITIES:Acids, Acid chlorides, Acid anhydrides, Chloroformates,

Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T N

Indication of Danger: Toxic. Dangerous for the environment.

R: 23/24/25 36 43 50/53

Risk Statements: Toxic by inhalation, in contact with skin and

if swallowed. Irritating to eyes. May cause sensitization by

skin contact. Very toxic to aquatic organisms, may cause

long-term adverse effects in the aquatic environment.

S: 28 36/37 45 60 61

Safety Statements: After contact with skin, wash immediately

with plenty of soap-suds. Wear suitable protective clothing and

gloves. In case of accident or if you feel unwell, seek medical

advice immediately (show the label where possible). This

material and its container must be disposed of as hazardous

waste. Avoid release to the environment. Refer to special

instructions/safety data sheets.

ACGIH RECOMMENDED LIMITS

ACGIH Threshold Limit Value .1 mg/m3

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .1 mg/m3

DOE Short Term Exposure Limit  .6 mg/m3

DOE Ceiling Limit 4 mg/m3

Immediately Dangerous to Life and Health   25 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLENEDIAMINE (META-)    CAS # 108452

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   E   .   .   .   .   J   K   L

NFPA HAZARD CODES (H,F,R,O)  3   1   0

ACUTE TOXICTY RISK INDEX 3.2 - LD50   280.0 mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   1.6 - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: Toxic if absorbed through skin.

Eye Contact: Causes eye irritation.

Inhalation: Toxic if inhaled. Material is irritating to mucous

membranes and upper respiratory tract.

Ingestion: Toxic if swallowed.

SENSITIZATION

skin: May cause allergic skin reaction.

TARGET ORGAN(S) OR SYSTEM(S)

Liver. Kidneys. Bladder.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

Exposure can cause: Nausea, dizziness, and headache. Dermatitis.

Pulmonary edema. Effects may be delayed. May cause discoloration

of the skin.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

VAPOR PRESSURE .120 mm Hg @ 20 °C

FLASH POINT  230 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

a - Organic Base/Flammable/Toxic

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Acids, Acid chlorides, Acid anhydrides, Chloroformates,

Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T N

Indication of Danger: Toxic. Dangerous for the environment.

R: 23/24/25 36 43 50/53 68

Risk Statements: Toxic by inhalation, in contact with skin and

if swallowed. Irritating to eyes. May cause sensitization by

skin contact. Very toxic to aquatic organisms, may cause

long-term adverse effects in the aquatic environment. Possible

risk of irreversible effects.

S: 28 36/37 45 60 61

Safety Statements: After contact with skin, wash immediately

with plenty of soap-suds. Wear suitable protective clothing and

gloves. In case of accident or if you feel unwell, seek medical

advice immediately (show the label where possible). This

material and its container must be disposed of as hazardous

waste. Avoid release to the environment. Refer to special

instructions/safety data sheets.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .1 mg/m3

DOE Short Term Exposure Limit  .3 mg/m3

DOE Ceiling Limit 10 mg/m3

Immediately Dangerous to Life and Health   125 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLENEDIAMINE DIHYDROCHLORIDE (1,2-)    CAS # 615281

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   C   .   E   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  3   0   0

EXTREMELY TOXIC - DESIGNATED AREA MAY BE REQUIRED

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: Harmful if absorbed through skin.

Eye Contact: Causes eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. Harmful if inhaled.

Ingestion: Toxic if swallowed.

SENSITIZATION

skin: May cause allergic skin reaction.

TARGET ORGAN(S) OR SYSTEM(S)

Liver. Bladder.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T-N

Indication of Danger: Toxic. Dangerous for the environment.

R: 20/21-25-36-40-43-50/53-68

Risk Statements: Harmful by inhalation and in contact with skin.

Toxic if swallowed. Irritating to eyes. Limited evidence of a

carcinogenic effect. May cause sensitization by skin contact.

Very toxic to aquatic organisms, may cause long-term adverse

effects in the aquatic environment. Possible risk of

irreversible effects.

S: 28-36/37-45-60-61

Safety Statements: After contact with skin, wash immediately

with plenty of soap-suds. Wear suitable protective clothing and

gloves. In case of accident or if you feel unwell, seek medical

advice immediately (show the label where possible). This

material and its container must be disposed of as hazardous

waste. Avoid release to the environment. Refer to special

instructions/safety data sheets.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  10 mg/m3

DOE Short Term Exposure Limit  30 mg/m3

DOE Ceiling Limit 125 mg/m3

Immediately Dangerous to Life and Health   125 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLETHANOL (1-)    CAS # 1517697

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   2   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes severe eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: Harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Kidneys.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  186.8 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22 38 41

Risk Statements: Harmful if swallowed. Irritating to skin. Risk

of serious damage to eyes.

S: 26 39

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLETHANOL (2-)    CAS # 60128

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   1   0

ACUTE TOXICTY RISK INDEX 5 - LD50  1790.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50    500.0

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: Harmful if absorbed through skin. Readily

absorbed through skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Central nervous system.

SIGNS AND SYMPTOMS OF EXPOSURE

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE .0628 mm Hg @ 20 °C

FLASH POINT  215.6 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 21/22 36/38

Risk Statements: Harmful in contact with skin and if swallowed.

Irritating to eyes and skin.

S: 26 28 36/37

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. After

contact with skin, wash immediately with plenty of water. Wear

suitable protective clothing and gloves.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLETHYLAMINE    CAS # 64040

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   I   J   K   L

NFPA HAZARD CODES (H,F,R,O)  3   2   0

ACUTE TOXICTY RISK INDEX 3.2 - LD50   300.0 mg/Kg

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   1.8 - LC50

ROUTE OF EXPOSURE

skin Contact: Causes severe burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes severe burns.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be harmful

if inhaled.

Ingestion: Harmful if swallowed.

SENSITIZATION

Sensitization: Prolonged or repeated exposure may cause allergic

reactions in certain sensitive individuals.

TARGET ORGAN(S) OR SYSTEM(S)

Nerves.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE .300 mm Hg @ 20 °C

FLASH POINT  177.8 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

a - Organic Base/Flammable/Toxic

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Acids, Acid chlorides, Acid anhydrides, Strong  oxidizing

agents Carbon dioxide.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: and nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame. Store under nitrogen\. Incompatible Materials:

Absorbs carbon dioxide from air\. SPECIAL REQUIREMENTS  Air sensitive. Store

under inert gas.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 22 34

Risk Statements: Harmful if swallowed. Causes burns.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLETHYLENEDIAMINE (N-)    CAS # 1664400

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes irritation. Vapor or mist is irritating

to the eyes, mucous membranes, and upper respiratory tract.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  235 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Acids, Acid chlorides, Acid anhydrides,  Oxidizing agents,

Chloroformates.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

STORAGE: Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34 37

Risk Statements: Causes burns. Irritating to respiratory system.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLGLYCINE (2-)    CAS # 2835065

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

Store at -20°C

Keep tightly closed. Store in a cool dry place.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place\. Store at -20░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLGLYCINE (2-)    CAS # 2935355

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

Store at -20°C

Keep tightly closed. Store in a cool dry place.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place\. Store at -20░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLGLYCINE (N-)    CAS # 103015

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   1

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLGLYCINE CHLORIDE HYDROCHLORIDE (D,ALPHA-)    CAS # 39878870

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  0   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  302 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 22 34 42

Risk Statements: Harmful if swallowed. Causes burns. May cause

sensitization by inhalation.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLGLYCINOL (2-)    CAS # 20989177

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLGLYOXAL HYDRATE    CAS # 1075065

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   F   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 2.5 - LD50  1612.9 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material is irritating to

mucous membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

CONDITIONS AGGRAVATED BY EXPOSURE

Causes sneezing. Do not inhale.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC   CORROSIVE

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at 2-8°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22 36/37/38

Risk Statements: Harmful if swallowed. Irritating to eyes,

respiratory system and skin.

S: 22 26 36

Safety Statements: Do not breathe dust. In case of contact with

eyes, rinse immediately with plenty of water and seek medical

advice. Wear suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLGLYOXAL MONOHYDRATE    CAS # 1074120

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material is irritating to

mucous membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

CONDITIONS AGGRAVATED BY EXPOSURE

Causes sneezing. Do not inhale.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

VAPOR PRESSURE .0961 mm Hg @ 20 °C

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22 36/37/38

Risk Statements: Harmful if swallowed. Irritating to eyes,

respiratory system and skin.

S: 22 26 36

Safety Statements: Do not breathe dust. In case of contact with

eyes, rinse immediately with plenty of water and seek medical

advice. Wear suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLHYDRAZINE    CAS # 100630   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   E   .   .   H   I   J   K   L

NFPA HAZARD CODES (H,F,R,O)  3   2   0

ACUTE TOXICTY RISK INDEX 4 - LD50   188.0 mg/Kg

NEUROTOXIC - RISK INDEX   5.0

INHALATION HAZARD  INHALATION RISK INDEX   1.6 - LC50    657.2

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: Toxic if absorbed through skin. Readily

absorbed through skin.

Eye Contact: Causes eye irritation.

Inhalation: Toxic if inhaled. Material may be irritating to

mucous membranes and upper respiratory tract.

Ingestion: Toxic if swallowed.

SENSITIZATION

skin: May cause allergic skin reaction.

Sensitization: Will occur

TARGET ORGAN(S) OR SYSTEM(S)

Blood. Liver. Kidneys.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause: Damage to the liver. Damage to the kidneys.

Blood effects.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE .1000 mm Hg @ 20 °C

FLASH POINT  192.2 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

a - Organic Base/Flammable/Toxic

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame. Store in a cool dry place. Store under nitrogen.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T N

Indication of Danger: Toxic. Dangerous for the environment.

R: 45 23/24/25 36/38 43 48/23/24/25 68 50

Risk Statements: May cause cancer. Also toxic by inhalation, in

contact with skin and if swallowed. Irritating to eyes and skin.

May cause sensitization by skin contact. Also toxic: danger of

serious damage to health by prolonged exposure through

inhalation, in contact with skin and if swallowed. Also possible

risks of irreversible effects. Very toxic to aquatic organisms.

S: 53 45 61

Safety Statements: Restricted to professional users. Attention -

Avoid exposure - obtain special instructions before use. In case

of accident or if you feel unwell, seek medical advice

immediately (show the label where possible). Avoid release to

the environment. Refer to special instructions/safety data

sheets.

OSHA REGULATORY LIMITS

OSHA Permissible Exposure Limit 5 ppm

OSHA Short Term Exposure Limit  10 ppm

ACGIH RECOMMENDED LIMITS

ACGIH Threshold Limit Value .1 ppm

Immediately Dangerous to Life and Health  15 ppm

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .1 ppm

DOE Short Term Exposure Limit  .3 ppm

DOE Ceiling Limit 2 ppm

Immediately Dangerous to Life and Health   15 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLHYDRAZINE    CAS # 59881   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    A   .   .   D   E   .   .   .   .   .   K   L

NFPA HAZARD CODES (H,F,R,O)  2   2   0

SPECIAL CARCINOGEN - DESIGNATED AREA MAY BE REQUIRED

CHRONIC TOXICITY RISK INDEX 3.1 - TD50    41.0 mg/Kg

ACUTE TOXICTY RISK INDEX 3 - LD50   188.0 mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   1.6 - LC50   2610.0

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: Toxic if absorbed through skin. Readily

absorbed through skin.

Eye Contact: Causes eye irritation.

Inhalation: Toxic if inhaled. Material may be irritating to

mucous membranes and upper respiratory tract.

Ingestion: Toxic if swallowed.

SENSITIZATION

skin: May cause allergic skin reaction.

Sensitization: Will occur

TARGET ORGAN(S) OR SYSTEM(S)

Blood. Liver. Kidneys.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause: Damage to the liver. Damage to the kidneys.

Blood effects.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

VAPOR PRESSURE .1000 mm Hg @ 20 °C

FLASH POINT  192.2 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

a - Organic Base/Flammable/Toxic

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame. Store in a cool dry place. Store under nitrogen.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T N

Indication of Danger: Toxic. Dangerous for the environment.

R: 45 23/24/25 36/38 43 48/23/24/25 68 50

Risk Statements: May cause cancer. Also toxic by inhalation, in

contact with skin and if swallowed. Irritating to eyes and skin.

May cause sensitization by skin contact. Also toxic: danger of

serious damage to health by prolonged exposure through

inhalation, in contact with skin and if swallowed. Also possible

risks of irreversible effects. Very toxic to aquatic organisms.

S: 53 45 61

Safety Statements: Restricted to professional users. Attention -

Avoid exposure - obtain special instructions before use. In case

of accident or if you feel unwell, seek medical advice

immediately (show the label where possible). Avoid release to

the environment. Refer to special instructions/safety data

sheets.

Immediately Dangerous to Life and Health  15 ppm

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  50 mg/m3

DOE Short Term Exposure Limit  150 mg/m3

DOE Ceiling Limit 250 mg/m3

Immediately Dangerous to Life and Health   250 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLHYDROQUINONE    CAS # 1079216   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid contact and inhalation. Do not get in eyes, on skin, on

clothing. STORAGE: Keep tightly closed. Store in a cool dry place. Handle

and store under nitrogen\. SPECIAL REQUIREMENTS  Air and moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLINDOLE (2-)    CAS # 948652

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 2.3 - LD50  2300.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Severe eye irritant.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 37/38 41

Risk Statements: Irritating to respiratory system and skin. Risk

of serious damage to eyes.

S: 26 39

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLMAGNESIUM BROMIDE    CAS # 100583   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   4   2   W

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: May be harmful if inhaled. Material is extremely

destructive to the tissue of the mucous membranes and upper

respiratory tract.

Ingestion: Harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Skeletal muscle. G.I. System. Central nervous system. Liver.

Kidneys.

SIGNS AND SYMPTOMS OF EXPOSURE

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Inhalation may result in spasm, inflammation and edema

of the larynxand bronchi, chemical pneumonitis, and pulmonary

edema. Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Flammable

Explosive

FLASH POINT  -40 °F

Store in an explosion-proof refrigerator or tightly stoppered in a

well-ventilated area

Forms explosive unstable peroxides

Test periodically or discard

Reacts violently with water

SEGREGATION: SHELF # 3

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

k - Explosive/Unstable

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   REACTIVE   TOXIC

INCOMPATIBILITIES:Reacts violently with water., Oxidizing  agents, Oxygen,

Alcohols, Acids

FIRE EXTINGUISHER:  Dry chemical powder.  Do not use water.

TOXIC EMISSIONS WHEN BURNED: Magnesium oxide Hydrogen bromide gas

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat, sparks, and open flame. Handle and store under nitrogen\.

Incompatible Materials: Do not allow contact with water       SPECIAL

REQUIREMENTS  Light sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: F+-C

Indication of Danger: Extremely Flammable. Corrosive.

R: 12-14-22-34

Risk Statements: Extremely flammable. Reacts violently with

water. Harmful if swallowed. Causes burns.

S: 16-26-36/37/39-45

Safety Statements: Keep away from sources of ignition - no

smoking. In case of contact with eyes, rinse immediately with

plenty of water and seek medical advice. Wear suitable

protective clothing, gloves, and eye/face protection. In case of

accident or if you feel unwell, seek medical advice immediately

(show the label where possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  1.5 mg/m3

DOE Short Term Exposure Limit  5 mg/m3

DOE Ceiling Limit 35 mg/m3

Immediately Dangerous to Life and Health   150 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLMALEIC ANHYDRIDE    CAS # 36122357

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin. High concentrations are extremely destructive to

tissues of the mucous membranes and upper respiratory tract,

eyes, and skin.

SIGNS AND SYMPTOMS OF EXPOSURE

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Water spray.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 20/21/22

Risk Statements: Harmful by inhalation, in contact with skin and

if swallowed.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLMERCAPTOTETRAZOLE (1:5-)    CAS # 86931

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 2.4 - LD50  5645.2 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Absorption: May be harmful if absorbed through the skin.

Inhalation: May be harmful if inhaled. Material is irritating to

mucous membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

Multiple Routes: Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Flammable

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam. Use water spray to cool fire-exposed    containers.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxides

Store at -20°C

Keep tightly closed. Keep away from heat, sparks, and open flame. Store in a

cool dry place.

REACTIVE PROPERTIES

HANDLING: Avoid breathing dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat, sparks, and open flame. Store in a cool dry place. Store at -20░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 5 22

Risk Statements: Heating may cause an explosion. Harmful if

swallowed.

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLMERCURIC ACETATE    CAS # 62384   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   B   C   .   .   .   .   .   I   J   .   .

NFPA HAZARD CODES (H,F,R,O)  3   2   0

HUMAN TERATOGEN - DESIGNATED AREA MAY BE REQUIRED

REPRODUCTIVE RISK INDEX   4.6

EXTREMELY TOXIC - DESIGNATED AREA MAY BE REQUIRED

ACUTE TOXICTY RISK INDEX 4.1 - LD50    41.0 mg/Kg

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: May be fatal if inhaled, swallowed, or absorbed

through skin. Causes burns

TARGET ORGAN(S) OR SYSTEM(S)

Kidneys. G.I. System. Nerves.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting.

CONDITIONS AGGRAVATED BY EXPOSURE

May cause nervous system disturbances.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong reducing agents, Strong

acids.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Mercury/mercury oxides

Store at 2-8°C

Keep tightly closed. Store in a cool dry place.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place\. Store at 2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T N

Indication of Danger: Toxic. Dangerous for the environment.

R: 25 34 48/24/25 50/53

Risk Statements: Toxic if swallowed. Causes burns. Toxic: danger

of serious damage to health by prolonged exposure in contact

with skin and if swallowed. Very toxic to aquatic organisms, may

cause long-term adverse effects in the aquatic environment.

S: 23 24/25 37 45 60 61

Safety Statements: Do not breathe vapor. Avoid contact with skin

and eyes. Wear suitable gloves. In case of accident or if you

feel unwell, seek medical advice immediately (show the label

where possible). This material and its container must be

disposed of as hazardous waste. Avoid release to the

environment. Refer to special instructions/safety data sheets.

Immediately Dangerous to Life and Health  10 mg/m3

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .1 mg/m3

DOE Short Term Exposure Limit  .1 mg/m3

DOE Ceiling Limit 10 mg/m3

Immediately Dangerous to Life and Health   10 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLMERCURIC NITRATE    CAS # 8003052   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be fatal if inhaled, swallowed, or absorbed

through skin. Causes eye and skin irritation.

TARGET ORGAN(S) OR SYSTEM(S)

Nerves.

CONDITIONS AGGRAVATED BY EXPOSURE

May cause nervous system disturbances.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Mercury/mercury oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T N

Indication of Danger: Toxic. Dangerous for the environment.

R: 25 34 48/24/25 50/53

Risk Statements: Toxic if swallowed. Causes burns. Toxic: danger

of serious damage to health by prolonged exposure in contact

with skin and if swallowed. Very toxic to aquatic organisms, may

cause long-term adverse effects in the aquatic environment.

S: 23 24/25 37 45 60 61

Safety Statements: Do not breathe vapor. Avoid contact with skin

and eyes. Wear suitable gloves. In case of accident or if you

feel unwell, seek medical advice immediately (show the label

where possible). This material and its container must be

disposed of as hazardous waste. Avoid release to the

environment. Refer to special instructions/safety data sheets.

Immediately Dangerous to Life and Health  10 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLMERCURIC NITRATE BASIC    CAS # 2229198

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER: WATER SPRAY.    CARBON DIOXIDE, DRY CHEMICAL POWDER OR

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: XIDENITROGEN OXIDES    MERC

Immediately Dangerous to Life and Health  10 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLMETHYLENEBIS(TRICYCLOHEXYLPHOSPHINE)-RUTHENI    CAS # 172222309

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Aluminum, Chemically  active

metals, Acids, Bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen chloride gas Phosphorous oxides

Store at 2-8°C

Air and light sensitive. Handle and store under inert gas.

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

2-8░C SPECIAL REQUIREMENTS  Air and light sensitive. Handle and store under

inert gas.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLPHENOL (2-)    CAS # 90437   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  1   1   0

ACUTE TOXICTY RISK INDEX 2.4 - LD50  2000.0 mg/Kg

INHALATION RISK INDEX   1.6 - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes severe eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Kidneys.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

VAPOR PRESSURE***** mm Hg @ 20 °C

FLASH POINT  255.2 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: Xi N

Indication of Danger: Irritant. Dangerous for the environment.

R: 36/37/38 50

Risk Statements: Irritating to eyes, respiratory system and

skin. Very toxic to aquatic organisms.

S: 61 22

Safety Statements: Avoid release to the environment. Refer to

special instructions/safety data sheets. Do not breathe dust.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  25 mg/m3

DOE Short Term Exposure Limit  75 mg/m3

DOE Ceiling Limit 500 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLPIPERAZINE (1-)    CAS # 92546

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   G   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  3   1   0

ACUTE TOXICTY RISK INDEX 3.4 - LD50   210.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: Toxic if absorbed through skin.

Eye Contact: Causes burns.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be harmful

if inhaled.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  284 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

a - Organic Base/Flammable/Toxic

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids, Acid chlorides, Acid

anhydrides.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: T

Indication of Danger: Toxic.

R: 22 24 34

Risk Statements: Harmful if swallowed. Toxic in contact with

skin. Causes burns.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLPROPANOL (2:2-)    CAS # 617947

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   2   0

ACUTE TOXICTY RISK INDEX 2.6 - LD50  1300.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin. Vapor or mist is irritating to the eyes, mucous

membranes, and upper respiratory tract. High concentrations are

extremely destructive to tissues of the mucous membranes and

upper respiratory tract, eyes, and skin.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  188.6 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Acids, Acid chlorides, Acid anhydrides,  Oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Do not get in eyes, on skin, on    clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22 36/37/38

Risk Statements: Harmful if swallowed. Irritating to eyes,

respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  1 ppm

DOE Short Term Exposure Limit  3 ppm

DOE Ceiling Limit 20 ppm

Immediately Dangerous to Life and Health   100 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLPROPANOL (3,1-)    CAS # 122974

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

ACUTE TOXICTY RISK INDEX 2.3 - LD50  2300.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE .0150 mm Hg @ 20 °C

FLASH POINT  248 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 37/39

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable gloves and eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLPROPANOLAMINE HYDROCHLORIDE    CAS # 154416

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

ACUTE TOXICTY RISK INDEX 2.5 - LD50  1490.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: Harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Autonomic nervous system.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22

Risk Statements: Harmful if swallowed.

S: 45 36/37/39 22

Safety Statements: In case of accident or if you feel unwell,

seek medical advice immediately (show the label where possible).

Wear suitable protective clothing, gloves, and eye/face

protection. Do not breathe dust.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .06 mg/m3

DOE Short Term Exposure Limit  .15 mg/m3

DOE Ceiling Limit 1.25 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLPROPYNE (3:1-)    CAS # 10147112

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   2   1

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Vapor or mist is irritating to the eyes, mucous

membranes, and upper respiratory tract.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  126 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Keep away from heat and open flame. Store in a

cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 10 36/37/38

Risk Statements: Flammable. Irritating to eyes, respiratory

system and skin.

S: 16 26 36/37/39

Safety Statements: Keep away from sources of ignition - no

smoking. In case of contact with eyes, rinse immediately with

plenty of water and seek medical advice. Wear suitable

protective clothing, gloves, and eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLPROPYNOL (1:2:1-)    CAS # 4187875

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Vapor or mist is irritating to the eyes, mucous

membranes, and upper respiratory tract.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  210.2 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Acids, Acid chlorides, Acid anhydrides,  Oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22 36/37/38

Risk Statements: Harmful if swallowed. Irritating to eyes,

respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLPYRAZOLIDINONE (1,3-)    CAS # 92433

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 3.4 - LD50   200.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids, Strong bases.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: Xn N

Indication of Danger: Harmful. Dangerous for the environment.

R: 22 51/53

Risk Statements: Harmful if swallowed. Toxic to aquatic

organisms, may cause long-term adverse effects in the aquatic

environment.

S: 61

Safety Statements: Avoid release to the environment. Refer to

special instructions/safety data sheets.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLPYRIDINE (2-)    CAS # 1008895

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   1   1

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  230 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 37/39

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable gloves and eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLSELENENYL BROMIDE    CAS # 34837553

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   .   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   0   0

INHALATION HAZARD  INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: Toxic if inhaled. Material is extremely destructive

to the tissue of the mucous membranes and upper respiratory

tract.

Ingestion: Toxic if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Liver.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen bromide gas Selenium/selenium oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T N

Indication of Danger: Toxic. Dangerous for the environment.

R: 23/25 33 50/53

Risk Statements: Toxic by inhalation and if swallowed. Danger of

cumulative effects. Very toxic to aquatic organisms, may cause

long-term adverse effects in the aquatic environment.

S: 20/21 28 45 60 61

Safety Statements: When using do not eat, drink, or smoke. After

contact with skin, wash immediately with plenty of soap-suds. In

case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible). This material and

its container must be disposed of as hazardous waste. Avoid

release to the environment. Refer to special instructions/safety

data sheets.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLSELENENYL CHLORIDE    CAS # 5707040

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   E   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION HAZARD  INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: May be fatal if inhaled, swallowed, or absorbed

through skin.

TARGET ORGAN(S) OR SYSTEM(S)

Liver.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen chloride gas Selenium/selenium oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T N

Indication of Danger: Toxic. Dangerous for the environment.

R: 23/25 33 50/53

Risk Statements: Toxic by inhalation and if swallowed. Danger of

cumulative effects. Very toxic to aquatic organisms, may cause

long-term adverse effects in the aquatic environment.

S: 20/21 28 45 60 61

Safety Statements: When using do not eat, drink, or smoke. After

contact with skin, wash immediately with plenty of soap-suds. In

case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible). This material and

its container must be disposed of as hazardous waste. Avoid

release to the environment. Refer to special instructions/safety

data sheets.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLSULFONYL PYRROLE (1-)    CAS # 16851824

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLSULFONYLETHYLENE (BIS,1,2-)    CAS # 963155

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Avoid contact and inhalation. Do not get in eyes, on skin, on

clothing. STORAGE: Keep tightly closed. Store in a cool dry place\. SPECIAL

REQUIREMENTS  Light sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLSULFONYLINDOLE (1-)    CAS # 40899716

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLTETRAHYDROPYRIDINE HYDROCHLORIDE (4:1,2,3,6-    CAS # 43064126

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   E   .   .   .   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION HAZARD  INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin. May cause irritation.

TARGET ORGAN(S) OR SYSTEM(S)

Nerves.

SIGNS AND SYMPTOMS OF EXPOSURE

Warning! This compound is closely related to 1-methyl-4-phenyl-

1,2,3,6-tetrahydropyridine (MPTP), which is a potent neurotoxin

and causes an irreversible parkinsonian condition in humans. We

recommend that all precautions be taken to avoid contact by all

routes of exposure. See S.P. Markey, N.R. Schmuff, Medical

Research Reviews, vol. 6, no. 4, 389-429 (1986) for a literature

review. Exposure can cause: Neurotoxic effects. To the best of

our knowledge, the chemical, physical, and toxicological

properties have not been thoroughly investigated.

CONDITIONS AGGRAVATED BY EXPOSURE

May be fatal if enters bloodstream.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Handle in a closed system. Do not breathe dust.    Do not get in

eyes, on skin, on clothing. Do not use if skin is    cut or scratched. Wash

thoroughly after handling. Avoid    prolonged or repeated exposure. STORAGE:

Keep tightly closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: T

Indication of Danger: Toxic.

R: 23/24/25 40

Risk Statements: Toxic by inhalation, in contact with skin and

if swallowed. Limited evidence of a carcinogenic effect.

S: 26 27 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Take

off immediately all contaminated clothing. Wear suitable

protective clothing, gloves, and eye/face protection. In case of

accident or if you feel unwell, seek medical advice immediately

(show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLTHIAZOLYLTHIOUREA    CAS # 14901167

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: May cause irritation. May be harmful by

inhalation, ingestion, or skin absorption.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

STORAGE

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLTHIOUREA (1,2-)    CAS # 103855   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   C   .   E   .   .   .   .   J   K   L

NFPA HAZARD CODES (H,F,R,O)  3   0   1

EXTREMELY TOXIC - DESIGNATED AREA MAY BE REQUIRED

ACUTE TOXICTY RISK INDEX 5+ - LD50     3.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be fatal if swallowed.

SENSITIZATION

skin: May cause allergic skin reaction.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong bases, Strong acids, Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Sulfur oxides Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: T+

Indication of Danger: Very toxic.

R: 28-43

Risk Statements: Very toxic if swallowed. May cause

sensitization by skin contact.

S: 28-36/37-45

Safety Statements: After contact with skin, wash immediately

with plenty of soap-suds. Wear suitable protective clothing and

gloves. In case of accident or if you feel unwell, seek medical

advice immediately (show the label where possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  3 mg/m3

DOE Short Term Exposure Limit  3 mg/m3

DOE Ceiling Limit 3 mg/m3

Immediately Dangerous to Life and Health   3 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLTIN TRICHLORIDE    CAS # 1124192   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   1   W

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  235 °F

Reacts violently with water

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   REACTIVE

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Do not use water.

TOXIC EMISSIONS WHEN BURNED: Hydrogen chloride gas Tin/tin oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place. Handle   and store under nitrogen\. SPECIAL REQUIREMENTS

Moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34

Risk Statements: Causes burns.

S: 26 27 28 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Take

off immediately all contaminated clothing. After contact with

skin, wash immediately with plenty of water. Wear suitable

protective clothing, gloves, and eye/face protection. In case of

accident or if you feel unwell, seek medical advice immediately

(show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLTRIAZOLIDINE DIONE (4:1,2,4:3,5-)    CAS # 15988111

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLTRIBUTYLSTANNANE    CAS # 960167

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

INHALATION HAZARD  INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Vapor or mist is irritating to the eyes, mucous

membranes, and upper respiratory tract.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  235 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Tin/tin oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T N

Indication of Danger: Toxic. Dangerous for the environment.

R: 21 25 36/38 48/23/25 50/53

Risk Statements: Harmful in contact with skin. Toxic if

swallowed. Irritating to eyes and skin. Toxic: danger of serious

damage to health by prolonged exposure through inhalation and if

swallowed. Very toxic to aquatic organisms, may cause long-term

adverse effects in the aquatic environment.

S: 35 36/37/39 45 60 61

Safety Statements: This material and its container must be

disposed of in a safe way. Wear suitable protective clothing,

gloves, and eye/face protection. In case of accident or if you

feel unwell, seek medical advice immediately (show the label

where possible). This material and its container must be

disposed of as hazardous waste. Avoid release to the

environment. Refer to special instructions/safety data sheets.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLTRICHLOROMETHYL MERCURY    CAS # 3294573   HAZARDOUS CHEMICAL OF

CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   E   .   .   .   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   0   0

NEUROTOXIC - RISK INDEX   5.0

INHALATION HAZARD  INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be fatal if absorbed through skin.

Inhalation: May be fatal if inhaled. Material may be irritating

to mucous membranes and upper respiratory tract.

Ingestion: May be fatal if swallowed.

Nerves.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING    on clothing. Avoid prolonged or repeated exposure. STORAGE: Keep

tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T+ N

Indication of Danger: Very toxic. Dangerous for the environment.

R: 26/27/28 33 50/53

Risk Statements: Very toxic by inhalation, in contact with skin

aquatic organisms, may cause long-term adverse effects in the

aquatic environment.

S: 13 28 36 45 60 61

Safety Statements: Keep away from food, drink, and animal

feedingstuffs. After contact with skin, wash immediately with

of accident or if you feel unwell, seek medical advice

immediately (show the label where possible). This material and

its container must be disposed of as hazardous waste. Avoid

release to the environment. Refer to special instructions/safety

Immediately Dangerous to Life and Health  10 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLTRIMETHYLAMMONIUM BROMIDE    CAS # 4207561

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide. Dry chemical powder. Water spray.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen bromide gas

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place\. SPECIAL REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34 37

Risk Statements: Causes burns. Irritating to respiratory system.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLTRIMETHYLAMMONIUM CHLORIDE     CAS # 138249

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 3.6 - LD50   121.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T

Indication of Danger: Toxic.

R: 24/25

Risk Statements: Toxic in contact with skin and if swallowed.

S: 53 22 39 45   Safety Statements: Restricted to professional users.

Attention -

Avoid exposure - obtain special instructions before use. Do not

breathe dust. Wear eye/face protection. In case of accident or

if you feel unwell, seek medical advice immediately (show the

label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLTRIMETHYLAMMONIUM IODIDE    CAS # 98044

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

Eye Contact: Causes severe eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide. Dry chemical powder. Water spray.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen iodide

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place\. SPECIAL REQUIREMENTS  Light sensitive. Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38 41

Risk Statements: Irritating to eyes, respiratory system and

skin. Risk of serious damage to eyes.

S: 26 36/39

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing and eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLUREA    CAS # 64108

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

ACUTE TOXICTY RISK INDEX 2.4 - LD50  2000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22

Risk Statements: Harmful if swallowed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHENYLVINYLBORONIC ACID (TRANS,2-)    CAS # 6783057

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Boron oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHLORETIN    CAS # 60822

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.ROUTE OF

EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.ROUTE OF

EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Light sensitive.HANDLING: Avoid inhalation. Avoid contact with

eyes, skin, and clothing. Avoid prolonged or repeated exposure. STORAGE: Keep

tightly closed\. SPECIAL REQUIREMENTS  Light sensitive.HANDLING: Avoid

inhalation. Avoid contact with eyes, skin, and clothing. Avoid prolonged or

repeated exposure. STORAGE: Keep tightly closed\. SPECIAL REQUIREMENTS  Light

sensitive.   Light sensitive.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHLORIDZIN DIHYDRATE    CAS # 7061543

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

User Exposure: Do not breathe dust. Avoid contact with eyes, skin, and

clothing. Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHLOROGLUCINOL    CAS # 108736

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   F   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 2.1 - LD50  4000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   CORROSIVE

INCOMPATIBILITIES:Acid chlorides, Acid anhydrides, Bases, Oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHLOROGLUCINOL    CAS # 6099907

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Acid chlorides, Acid anhydrides, Bases, Oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  4 mg/m3

DOE Short Term Exposure Limit  12.5 mg/m3

DOE Ceiling Limit 75 mg/m3

Immediately Dangerous to Life and Health   400 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHLOXINE B    CAS # 18472872

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

ACUTE TOXICTY RISK INDEX 1.8 - LD50  8400.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

d - Organic Acid/Flammable/Toxic

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Materials to Avoid: S

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

TOXIC EMISSIONS WHEN BURNED: Hydrogen chloride gas Hydrogen bromide gasROUTE

OF EXPOSURE

REACTIVE PROPERTIES

HANDLING skin, and clothing. Avoid prolonged or repeated exposure. STORAGE:

Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHORBOL ACETATE MYRISTATE    CAS # 16561298

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   C   .   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

EXTREMELY TOXIC - DESIGNATED AREA MAY BE REQUIRED

ACUTE TOXICTY RISK INDEX 4.5 - LD50    15.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Bone marrow. Lungs. G.I. System. Blood. Skin.

SIGNS AND SYMPTOMS OF EXPOSURE

Phorbal derivatives are purgatives if taken internally. Tumor

promoter.

CONDITIONS AGGRAVATED BY EXPOSURE

Phorbol esters are some of the most potent tumor promoters known

to man. Some of the phorbol esters enhance tumor promotion of

known carcinogens ten fold. Use extreme caution when handling

these compounds with carcinogenic compounds.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Materials to Avoid: S

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at -20°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

-20░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 38

Risk Statements: Irritating to skin.

S: 36/37

Safety Statements: Wear suitable protective clothing and gloves.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHOSGENE    CAS # 75445   HAZARDOUS CHEMICAL OF CONCERN

DEPARTMENT OF HOMELAND SECURITY CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   C   D   .   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  4   0   0

EXTREMELY TOXIC - DESIGNATED AREA MAY BE REQUIRED

ACUTE TOXICTY RISK INDEX 5 - LD50         mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   <1  - LC50  25045.2

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be fatal

if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Symptoms may be delayed. Causes severe destruction of

the alveolae of the lungs resulting in pulmonary edema,

suffocation and possible death. The onset of the pulmonary edema

is delayed due to the slow hydrolysis of the material. Long term

effects are emphysema, fibrosis, coughing, bloody sputum and

general malaise. These symptoms may last several months.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Gas

VAPOR PRESSURE 1180.0 mm Hg @ 20 °C

SEGREGATION: SHELF # 3

STORAGE GROUP(S):

i - CorrosiveorNon-Toxic Gas

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Amines, Ammonia, Alcohols, Sodium, Potassium.

FIRE EXTINGUISHER:  Use extinguishing media appropriate to surrounding fire

conditions. Use water spray or fog nozzle to keep cylinder    cool. Move

cylinder away from fire if there is no risk.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Cylinder temperature should

not exceed 125░F (52░C)\. SPECIAL REQUIREMENTS  Moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T+

Indication of Danger: Very toxic.

R: 26 34

Risk Statements: Very toxic by inhalation. Causes burns.

S: 9 26 36/37/39 45

Safety Statements: Keep container in a well-ventilated place. In

case of contact with eyes, rinse immediately with plenty of

water and seek medical advice. Wear suitable protective

clothing, gloves, and eye/face protection. In case of accident

or if you feel unwell, seek medical advice immediately (show the

label where possible).

OSHA REGULATORY LIMITS

OSHA Permissible Exposure Limit .1 ppm

Immediately Dangerous to Life and Health  2 ppm

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .1 ppm

DOE Short Term Exposure Limit  .1 ppm

DOE Ceiling Limit .3 ppm

Immediately Dangerous to Life and Health   .75 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHOSPHATE GLASS    CAS # 68915311

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 2.2 - LD50  3053.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Ingestion may cause a shocklike state, fall of blood pressure,

slow pulse, cyanosis, and coma. To the best of our knowledge,

the chemical, physical, and toxicological properties have not

been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  12.5 mg/m3

DOE Short Term Exposure Limit  40 mg/m3

DOE Ceiling Limit 250 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHOSPHATIDYLCHOLINE (3-)    CAS # 8002435

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Phosphorous oxides Nitrogen  oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

UNITED STATES REGULATORY INFORMATION

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHOSPHOCREATINE    CAS # 108321171

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at -20°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

-20░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHOSPHOENOLPYRUVATE CARBOXYLASE    CAS # 9067770

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. May cause irritation.

SENSITIZATION

Sensitization: Prolonged or repeated exposure may cause allergic

reactions in certain sensitive individuals.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at -20°C

Keep container closed. Store away from heat.

REACTIVE PROPERTIES

STORAGE: Keep container closed. Store away from heat\. Store at -20░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHOSPHOENOLPYRUVATE MONOPOTASSIUM    CAS # 4265070

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Phosphorous oxides Potassium oxides

Store at -20°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

-20░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHOSPHOENOLPYRUVATE TRICYCLOHEXYLAMMONIUM    CAS # 35556708

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

Store at 2-8°C

Keep tightly closed. Store in a cool dry place.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place\. Store at 2-8░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHOSPHOENOLPYRUVATE TRISODIUM HYDRATE    CAS # 5541935

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at -20°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. Store at

-20░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22-24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHOSPHOGLUCONIC ACID TRISODIUM (6-)    CAS # 53411704

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Phosphorous oxides

Store at -20°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. Store at

-20░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHOSPHOGLUCOSE ISOMERASE    CAS # 9001416

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  1   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: May cause irritation. May be harmful by

inhalation, ingestion, or skin absorption.

SENSITIZATION

Sensitization: Prolonged or repeated exposure may cause allergic

reactions in certain sensitive individuals.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at 2-8°C

Keep container closed. Store away from heat.

REACTIVE PROPERTIES

STORAGE: Keep container closed. Store away from heat\. Store at 2-8°C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 24/25

Safety Statements: Avoid contact with skin and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHOSPHOGLYCERIC ACID DISODIUM    CAS # 80731108

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

TARGET ORGAN(S) OR SYSTEM(S)

Eyes. Nerves. Liver. Kidneys. Heart.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Phosphorous oxides

Store at -20°C

Keep tightly closed. Store in a cool dry place.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place\. Store at -20░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHOSPHOLIPASE A    CAS # 9001847

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  4   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SENSITIZATION

Sensitization: Prolonged or repeated exposure may cause allergic

reactions in certain sensitive individuals.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

CONDITIONS AGGRAVATED BY EXPOSURE

May be fatal if enters bloodstream. The toxicological properties

have not been thoroughly investigated. Basic phospholipases a in

general appear to be toxic, having lethal dose of about 500

ug/kg/mouse or even considerably less as exemplified by

presynaptic neurotoxins and myonecrotoxins. Many of the acidic

phospholipases A, however, are much less toxic and not lethal

even at 2000 ug/kg, although there are exceptions. An acidic

phospholipase a (isoelectric point 5.1) from naja nigricollis

has a lethal dose of 800 ug/kg in the mouse.*

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at -20°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Do not get in eyes, on skin, on    clothing.

Avoid prolonged or repeated exposure. Do not use if skin is cut or scratched.

Wash thoroughly after handling. STORAGE: Keep tightly closed. Store at -20░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHOSPHOLIPASE C    CAS # 9001869

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SENSITIZATION

Sensitization: Prolonged or repeated exposure may cause allergic

reactions in certain sensitive individuals.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause: Gastrointestinal disturbances. To the best

of our knowledge, the chemical, physical, and toxicological

properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Use extinguishing media appropriate to surroundingfire

conditions.

Store at -20°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid contact with eyes, skin, and clothing.   Avoid inhalation.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. Store at

-20°C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHOSPHOLIPASE D    CAS # 9001870

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  1   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: May cause irritation. May be harmful by

inhalation, ingestion, or skin absorption.

SENSITIZATION

Sensitization: Prolonged or repeated exposure may cause allergic

reactions in certain sensitive individuals.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

STORAGE: Keep container closed. Store away from heat. Store at -20░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHOSPHOMOLYBDIC ACID    CAS # 51429744

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   0   1   OX

NEUROTOXIC - RISK INDEX   4.0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be harmful

if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Oxidizing Agent

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Finely powdered metals, Organic materials.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Molybdenum oxides Phosphorous oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from combustible   materials, heat, sparks, and open flame.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: O-C

Indication of Danger: Oxidizing. Corrosive.

R: 8-34

Risk Statements: Contact with combustible material may cause

fire. Causes burns.

S: 17-26-36/37/39-45

Safety Statements: Keep away from combustible material. In case

of contact with eyes, rinse immediately with plenty of water and

seek medical advice. Wear suitable protective clothing, gloves,

and eye/face protection. In case of accident or if you feel

unwell, seek medical advice immediately (show the label where

possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  8.07 mg/m3

DOE Short Term Exposure Limit  8.07 mg/m3

DOE Ceiling Limit 8.07 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHOSPHORIC ACID (ORTHO-)    CAS # 7664382

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   H   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   0   0

ACUTE TOXICTY RISK INDEX 4 - LD50  1530.0 mg/Kg

INHALATION RISK INDEX   2.0 - LC50    850.0

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: May be harmful if inhaled. Material is extremely

destructive to the tissue of the mucous membranes and upper

respiratory tract.

Ingestion: Harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Bone marrow. Blood. Liver.

SIGNS AND SYMPTOMS OF EXPOSURE

May cause cyanosis (blue-gray coloring of skin and lips caused

by lack of oxygen). Symptoms of exposure may include burning

sensation, coughing, wheezing, laryngitis, shortness of breath,

headache, nausea, and vomiting. Inhalation may result in spasm,

inflammation and edema of the larynxand bronchi, chemical

pneumonitis, and pulmonary edema. Material is extremely

destructive to tissue of the mucous membranes and upper

respiratory tract, eyes, and skin.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

VAPOR PRESSURE    2.2 mm Hg @ 20 °C

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

f - Inorganic Acid (Non-Oxidizing)

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Phosphorous oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34

Risk Statements: Causes burns.

S: 26-36/37/39-45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

OSHA REGULATORY LIMITS

OSHA Permissible Exposure Limit 1 mg/m3

OSHA Short Term Exposure Limit 3 mg/m3

ACGIH RECOMMENDED LIMITS

ACGIH Threshold Limit Value 1 mg/m3

ACGIH Short Term Exposure Limit 3 mg/m3

Immediately Dangerous to Life and Health  1000 mg/m3

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  1 mg/m3

DOE Short Term Exposure Limit  3 mg/m3

DOE Ceiling Limit 500 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHOSPHOROUS ACID    CAS # 13598362

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   0   0

ACUTE TOXICTY RISK INDEX 2.4 - LD50  1895.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: May be harmful if inhaled. Material is extremely

destructive to the tissue of the mucous membranes and upper

respiratory tract.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Material is extremely destructive to tissue of the

mucous membranes and upper respiratory tract, eyes, and skin. To

the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong bases, Oxidizing agents.

FIRE EXTINGUISHER:  Water spray.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Store under inert gas. Air sensitive. Moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 22 35

Risk Statements: Harmful if swallowed. Causes severe burns.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  2 mg/m3

DOE Short Term Exposure Limit  6 mg/m3

DOE Ceiling Limit 50 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHOSPHOROUS PENTOXIDE    CAS # 1314563   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  3   0   1   W

ACUTE TOXICTY RISK INDEX 5 - LD50  1000.0 mg/Kg

INHALATION RISK INDEX   1.8 - LC50     61.0

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: Harmful if absorbed through skin.

Eye Contact: Causes burns.

Inhalation: Toxic if inhaled. Material is extremely destructive

to the tissue of the mucous membranes and upper respiratory

tract.

Ingestion: Ingestion can cause immediate burning pain in the

mouth, throat, abdomen; severe swelling of the larynx and

skeletal paralysis affecting the ability to breathe, circulatory

shock and convulsions.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

VAPOR PRESSURE .840 mm Hg @ 20 °C

Reacts violently with water

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

WASTE CHARACTERISTIC HAZARD:   REACTIVE   TOXIC

INCOMPATIBILITIES:Water, Sodium, Potassium, Ammonia, Peroxides, Magnesium,

Alcohols, Metals, Oxidizing agents Reaction with 95%   formic acid caused

rapid evolution of carbon monoxide. Dry   mixtures with sodium oxide, calcium

oxide react violently if   warmed or moi

FIRE EXTINGUISHER:  Use extinguishing media appropriate to surroundingfire

conditions. Do not use water.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place\. Incompatible Materials: Do not allow contact with water

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 3   Risk Statements: Causes severe burns.

S: 22 26 45

Safety Statements: Do not breathe dust. In case of contact with

eyes, rinse immediately with plenty of water and seek medical

advice. In case of accident or if you feel unwell, seek medical

advice immediately (show the label where possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  1 mg/m3

DOE Short Term Exposure Limit  1 mg/m3

DOE Ceiling Limit 10 mg/m3

Immediately Dangerous to Life and Health   50 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHOSPHORUS    CAS # 7723140

DEPARTMENT OF HOMELAND SECURITY CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   1   1   OX

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Depending on the intensity and duration of exposure, effects may

vary from mild irritation to severe destruction of tissue.

Stomach pains, vomiting, diarrhea. To the best of our knowledge,

the chemical, physical, and toxicological properties have not

been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Flammable

VAPOR PRESSURE .0300 mm Hg @ 20 °C

Oxidizing Agent

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Sulfur, Oxidizing agents Copper, Copper alloys, Oxygen,

Avoid contact with alkaline hydroxides, Bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid contact with eyes, skin, and clothing.   Avoid breathing

dust. Avoid prolonged or repeated exposure. STORAGE: Keep container closed.

Keep away from heat, sparks, and open flame\. SPECIAL REQUIREMENTS   Heat

sensitive.HANDLING: Avoid contact with eyes, skin, and clothing.   Avoid

breathing dust. Avoid prolonged or repeated exposure. STORAGE: Keep container

closed. Keep away from heat, sparks, and open flame\. SPECIAL REQUIREMENTS

Heat sensitive.HANDLING: Avoid contact with eyes, skin, and clothing.   Avoid

breathing dust. Avoid prolonged or repeated exposure. STORAGE: Keep container

closed. Keep away from heat, sparks, and open flame\. SPECIAL REQUIREMENTS

Heat sensitive.HANDLING: Avoid contact with eyes, skin, and clothing.   Avoid

breathing dust. Avoid prolonged or repeated exposure. STORAGE: Keep container

closed. Keep away from heat, sparks, and open flame\. SPECIAL REQUIREMENTS

Heat sensitive.HANDLING: Avoid contact with eyes, skin, and clothing.   Avoid

breathing dust. Avoid prolonged or repeated exposure. STORAGE: Keep container

closed. Keep away from heat, sparks, and open flame\. SPECIAL REQUIREMENTS

Heat sensitive.HANDLING: Avoid contact with eyes, skin, and clothing.   Avoid

breathing dust. Avoid prolonged or repeated exposure. STORAGE: Keep container

closed. Keep away from heat, sparks, and open flame\. SPECIAL REQUIREMENTS

Heat sensitive.HANDLING: Avoid contact with eyes, skin, and clothing.   Avoid

breathing dust. Avoid prolonged or repeated exposure. STORAGE: Keep container

closed. Keep away from heat, sparks, and open flame\. SPECIAL REQUIREMENTS

Heat sensitive.HANDLING: Avoid contact with eyes, skin, and clothing.   Avoid

breathing dust. Avoid prolonged or repeated exposure. STORAGE: Keep container

closed. Keep away from heat, sparks, and open flame\. SPECIAL REQUIREMENTS

Heat sensitive.HANDLING: Avoid contact with eyes, skin, and clothing.   Avoid

breathing dust. Avoid prolonged or repeated exposure. STORAGE: Keep container

closed. Keep away from heat, sparks, and open flame\. SPECIAL REQUIREMENTS

Heat sensitive.   Heat sensitive.

Immediately Dangerous to Life and Health  5 mg/m3

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .04 mg/m3

DOE Short Term Exposure Limit  .125 mg/m3

DOE Ceiling Limit .75 mg/m3

Immediately Dangerous to Life and Health   4 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHOSPHORUS (V) TRIBROMIDE OXIDE    CAS # 7789595   HAZARDOUS CHEMICAL OF

CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   0   1   W

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns. Lachrymator.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be harmful

if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Reacts violently with water

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

WASTE CHARACTERISTIC HAZARD:   REACTIVE   TOXIC

INCOMPATIBILITIES:Strong bases, Organic materials Reacts  violently with

water.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Do not use water.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Handle

and store under nitrogen\. Incompatible Materials: Do not allow contact with

water       SPECIAL REQUIREMENTS  Reacts violently with water. Store under

inert gas.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 14 34

Risk Statements: Reacts violently with water. Causes burns.

S: 26 36/37/39 43 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of fire, never use water. In case of accident or if you

feel unwell, seek medical advice immediately (show the label

where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHOSPHORUS OXYCHLORIDE    CAS # 10025873   HAZARDOUS CHEMICAL OF CONCERN

DEPARTMENT OF HOMELAND SECURITY CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   C   D   .   .   G   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  4   0   1   W

EXTREMELY TOXIC - DESIGNATED AREA MAY BE REQUIRED

ACUTE TOXICTY RISK INDEX 5 - LD50    36.0 mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   4.0 - LC50    331.7

ROUTE OF EXPOSURE

skin Contact: Causes severe burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Lachrymator. Causes severe burns.

Inhalation: May be fatal if inhaled. Material is extremely

destructive to the tissue of the mucous membranes and upper

respiratory tract.

Ingestion: Toxic if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

VAPOR PRESSURE   28.0 mm Hg @ 20 °C

Reacts violently with water

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

WASTE CHARACTERISTIC HAZARD:   REACTIVE   TOXIC

INCOMPATIBILITIES:Strong bases, Alcohols, Amines, Phenol Avoid contact with

metals., Strong oxidizing agents Reacts violently   with water. Acetone

reacts violently with phosphorous oxychloride.

FIRE EXTINGUISHER:  Noncombustible. Use extinguishing media appropriate

tosurrounding fire conditions. Do not use water.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store

under nitrogen\. Incompatible Materials: Do not allow contact with water

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T+ C

Indication of Danger: Very toxic. Corrosive.

R: 14 22 26 29 35 48/23

Risk Statements: Reacts violently with water. Harmful if

swallowed. Very toxic by inhalation. Contact with water

liberates toxic gas. Causes severe burns. Toxic: danger of

serious damage to health by prolonged exposure through

inhalation.

S: 7/8 26 45

Safety Statements: Keep container tightly closed and dry. In

case of contact with eyes, rinse immediately with plenty of

water and seek medical advice. In case of accident or if you

feel unwell, seek medical advice immediately (show the label

where possible).

OSHA REGULATORY LIMITS

OSHA Permissible Exposure Limit .1 ppm

ACGIH RECOMMENDED LIMITS

ACGIH Threshold Limit Value .1 mg/m3

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .1 ppm

DOE Short Term Exposure Limit  .479 ppm

DOE Ceiling Limit .479 ppm

Immediately Dangerous to Life and Health   .85 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHOSPHORUS PENTACHLORIDE    CAS # 10026138   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  3   0   1   W

ACUTE TOXICTY RISK INDEX 5 - LD50   660.0 mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   2.0 - LC50    205.0

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be fatal

if inhaled.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Inhalation may result in spasm, inflammation and edema

of the larynxand bronchi, chemical pneumonitis, and pulmonary

edema. Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin. To the

best of our knowledge, the chemical, physical, and toxicological

properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

VAPOR PRESSURE***** mm Hg @ 20 °C

Reacts violently with water

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

WASTE CHARACTERISTIC HAZARD:   REACTIVE   TOXIC

INCOMPATIBILITIES:Potassium, Sodium, Amines Reacts violently  with water.,

Alcohols

FIRE EXTINGUISHER:  Use extinguishing media appropriate to surrounding fire

conditions.  Do not use water.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place. Store under nitrogen\. Incompatible Materials: Do not allow

contact with water

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T+

Indication of Danger: Very toxic.

R: 14-22-26-34-48/20

Risk Statements: Reacts violently with water. Harmful if

swallowed. Very toxic by inhalation. Causes burns. Harmful:

danger of serious damage to health by prolonged exposure through

inhalation.

S: 7/8-26-36/37/39-45

Safety Statements: Keep container tightly closed and dry. In

case of contact with eyes, rinse immediately with plenty of

water and seek medical advice. Wear suitable protective

clothing, gloves, and eye/face protection. In case of accident

or if you feel unwell, seek medical advice immediately (show the

label where possible).

ACGIH RECOMMENDED LIMITS

ACGIH Threshold Limit Value .1 mg/m3

Immediately Dangerous to Life and Health  70 mg/m3

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  1 mg/m3

DOE Short Term Exposure Limit  2 mg/m3

DOE Ceiling Limit 12.5 mg/m3

Immediately Dangerous to Life and Health   70 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHOSPHORUS PENTASULFIDE    CAS # 1314803   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   .   F   G   H   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   2   1   W

ACUTE TOXICTY RISK INDEX 3.1 - LD50   389.0 mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

Eye Contact: Causes eye irritation. Lachrymator.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. Harmful if inhaled.

Ingestion: Harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Central nervous system. Lungs.

SIGNS AND SYMPTOMS OF EXPOSURE

Hydrogen sulfide is recognized by its characteristic odor of

"rotten eggs". The detectable, minimum perceptible odor occurs

at 0.13ppm. At concentrations of 20ppm hydrogen sulfide begins

respiratory tract and increases with concentration and exposure

time. The irritant action and odor of rotten eggs often provides

the first warning of hydrogen sulfide exposure. Unfortunately,

above 150ppm the gas exerts a paralyzing effect on the olfactory

(250ppm) may cause pulmonary edema. At concentrations over

500ppm, drowsiness, dizziness, excitement, headache, unstable

gait, and other systemic symptoms occur within a few minutes.

Sudden loss of consciousness without premonition, anxiety, or

concentrations above 700ppm. At concentrations of 1000-2000ppm

hydrogen sulfide is rapidly absorbed through the lung into the

blood. Initially hyperpnea occurs, followed by rapid collapse

and respiratory inhibition. At higher concentrations, hydrogen

sulfide exerts an immediate paralyzing effect on the respiratory

almost always results. Generally speaking, imminent death due to

asphyxia can happen at any time when the concentration reaches

1000ppm or more. Pulmonary edema. Effects may be delayed.

Exposure can cause: Symptoms of exposure may include burning

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  156.2 °F

Reacts violently with water

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   REACTIVE   TOXIC   CORROSIVE

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid contact with eyes, skin, and clothing. STORAGE: Keep tightly

closed. Keep away from heat, sparks, and open flame. Store in a cool dry

place.

Immediately Dangerous to Life and Health  250 mg/m3

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  1 mg/m3

DOE Short Term Exposure Limit  3 mg/m3

DOE Ceiling Limit 50 mg/m3

Immediately Dangerous to Life and Health   250 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHOSPHORUS TRIBROMIDE    CAS # 7789608   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   C   .   .   .   G   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  3   0   1   W

EXTREMELY TOXIC - DESIGNATED AREA MAY BE REQUIRED

ACUTE TOXICTY RISK INDEX 4 - LD50    50.0 mg/Kg

INHALATION HAZARD

INHALATION RISK INDEX   3.3 - LC50

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be harmful

if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

VAPOR PRESSURE    4.2 mm Hg @ 20 °C

Reacts violently with water

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

WASTE CHARACTERISTIC HAZARD:   REACTIVE   TOXIC

INCOMPATIBILITIES:Strong bases, Sodium, Potassium, Strong oxidizing agents,

Alcohols, Acids Reacts violently with water.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Do not use water.

TOXIC EMISSIONS WHEN BURNED: Hydrogen bromide gas Thermal decomposition may

produce toxic fumes of phosphorus oxides and/orphosphine

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place. Handle   and store under nitrogen\. Incompatible Materials:

Do not allow contact with water  Special REQUIREMENTS Handle and store under

inert gas.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 14 34 37

Risk Statements: Reacts violently with water. Causes burns.

Irritating to respiratory system.

S: 26 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. In

case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHOSPHORUS TRICHLORIDE    CAS # 7719122   HAZARDOUS CHEMICAL OF CONCERN

DEPARTMENT OF HOMELAND SECURITY CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   C   D   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  4   0   1   W

EXTREMELY TOXIC - DESIGNATED AREA MAY BE REQUIRED

ACUTE TOXICTY RISK INDEX 5 - LD50    18.0 mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   4.6 - LC50    280.3

ROUTE OF EXPOSURE

skin Contact: Causes severe burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes severe burns.

Inhalation: May be fatal if inhaled. Material is extremely

destructive to the tissue of the mucous membranes and upper

respiratory tract.

Ingestion: May be fatal if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

VAPOR PRESSURE   95.2 mm Hg @ 20 °C

Reacts violently with water

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

WASTE CHARACTERISTIC HAZARD:   REACTIVE   TOXIC

INCOMPATIBILITIES:Strong bases, Sodium, Potassium, Strong oxidizing agents,

Ammonia, Alcohols Dimethyl sulfoxide. (DMSO),   Metals

TOXIC EMISSIONS WHEN BURNED: Chlorine Hydrogen chloride gas Phosphorous

oxides  Hazardous Decomposition Products Formed Upon Contact with Water:

Hydrochloric acid Phosphine

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store

under nitrogen.   Unsuitable: Do not store in plastic\. Incompatible

Materials: Do not allow contact with water Avoid   contact with metals\.

SPECIAL REQUIREMENTS  Store under inert gas. Light sensitive. Avoid contact

with   metals.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T+ C

Indication of Danger: Very toxic. Corrosive.

R: 14 26/28 29 35 48/20

Risk Statements: Reacts violently with water. Very toxic by

inhalation and if swallowed. Contact with water liberates toxic

gas. Causes severe burns. Harmful: danger of serious damage to

health by prolonged exposure through inhalation.

S: 7/8 26 36/37/39 45

Safety Statements: Keep container tightly closed and dry. In

case of contact with eyes, rinse immediately with plenty of

water and seek medical advice. Wear suitable protective

clothing, gloves, and eye/face protection. In case of accident

or if you feel unwell, seek medical advice immediately (show the

label where possible).

Immediately Dangerous to Life and Health  25 ppm

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .5 ppm

DOE Short Term Exposure Limit  .62 ppm

DOE Ceiling Limit 2 ppm

Immediately Dangerous to Life and Health   5.6 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHOSPHORYLASE A    CAS # 9032104

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SENSITIZATION

Sensitization: Prolonged or repeated exposure may cause allergic

reactions in certain sensitive individuals.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at -20°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

-20░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHOSPHORYLASE B    CAS # 9012695

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SENSITIZATION

Sensitization: Prolonged or repeated exposure may cause allergic

reactions in certain sensitive individuals.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Use extinguishing media appropriate to surrounding fire

conditions.

Store at 2-8°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid contact with eyes, skin, and clothing.    Avoid inhalation.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHOSPHOTUNGSTIC ACID    CAS # 12501234

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   F   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

ACUTE TOXICTY RISK INDEX 2.2 - LD50  3300.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

Eye Contact: Causes burns.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be harmful

if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

vomiting. Material is extremely destructive to tissue of the

mucous membranes and upper respiratory tract, eyes, and skin. To

the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   CORROSIVE

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases.

FIRE EXTINGUISHER:  Dry chemical powder.

TOXIC EMISSIONS WHEN BURNED: Thermal decomposition may  Tungsten oxide

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, STORAGE: Keep

tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34

Risk Statements: Causes burns.

S: 26 36/37/39

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHOSPHOTUNGSTIC ACID HYDRATE    CAS # 12067991

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   F   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   0   0

ACUTE TOXICTY RISK INDEX 2.2 - LD50  3300.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be harmful

if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Material is extremely destructive to tissue of the

mucous membranes and upper respiratory tract, eyes, and skin. To

the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC   CORROSIVE

INCOMPATIBILITIES:Strong bases.

FIRE EXTINGUISHER:  Dry chemical powder.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Store under inert gas. Keep

tightly closed. Store in a   cool dry place\. SPECIAL REQUIREMENTS  Store

under inert gas. Air sensitive. Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34

Risk Statements: Causes burns.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHOSPHOTUNGSTIC ACID SODIUM    CAS # 51312426

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 2.5 - LD50  1600.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material is irritating to   mucous

membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Phosphorous oxides Tungsten oxide Sodium oxide

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION   Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22 36/37/38

Risk Statements: Harmful if swallowed. Irritating to eyes,

respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHOSVITIN    CAS # 9008962

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at -20°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

-20░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHTHALDIALDEHYDE (ORTHO-)    CAS # 643798

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  269.6 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at 2-8°C

Keep tightly closed. Store in a cool dry place.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Store in a cool dry place. Store at 2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 28 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. After

contact with skin, wash immediately with plenty of water. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHTHALIC ACID    CAS # 88993

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

ACUTE TOXICTY RISK INDEX 1.8 - LD50  8161.3 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

d - Organic Acid/Flammable/Toxic

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Materials to Avoid: S

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid contact and inhalation. Do not get in eyes, on skin, on

clothing. STORAGE: Keep tightly closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .02 mg/m3

DOE Short Term Exposure Limit  .06 mg/m3

DOE Ceiling Limit .5 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHTHALIC ACID MONOPOTASSIUM    CAS # 877247

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 2.2 - LD50  3200.0 mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SENSITIZATION

Sensitization: Prolonged or repeated exposure may cause allergic

reactions in certain sensitive individuals.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause: Asthmatic symptoms: cough, hoarseness,

rhinorrhea, wheezing, and conjunctivitis. Nausea. Vomiting. To

the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  12.5 mg/m3

DOE Short Term Exposure Limit  40 mg/m3

DOE Ceiling Limit 250 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHTHALIC ANHYDRIDE    CAS # 85449   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   G   .   I   .   .   L

NFPA HAZARD CODES (H,F,R,O)  3   1   0

ACUTE TOXICTY RISK INDEX 5 - LD50  1530.0 mg/Kg

NEUROTOXIC - RISK INDEX   1.0

INHALATION RISK INDEX   <1  - LC50    210.0

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes severe eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: Harmful if swallowed.

SENSITIZATION

Sensitization: May cause allergic respiratory and skin reactions

TARGET ORGAN(S) OR SYSTEM(S)

Kidneys. Liver.

SIGNS AND SYMPTOMS OF EXPOSURE

Prolonged exposure can cause: Damage to the liver. Damage to the

kidneys. Exposure to and/or consumption of alcohol may increase

toxic effects. To the best of our knowledge, the chemical,

physical, and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

VAPOR PRESSURE .010000 mm Hg @ 20 °C

FLASH POINT  305.6 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong acids, Strong bases, Strong oxidizing agents, Strong

reducing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22 37/38 41 42/43

Risk Statements: Harmful if swallowed. Irritating to respiratory

system and skin. Risk of serious damage to eyes. May cause

sensitization by inhalation and skin contact.

S: 23 24/25 26 37/39 46

Safety Statements: Do not breathe spray. Avoid contact with skin

and eyes. In case of contact with eyes, rinse immediately with

plenty of water and seek medical advice. Wear suitable gloves

and eye/face protection. If swallowed, seek medical advice

immediately and show this container or label.

OSHA REGULATORY LIMITS

OSHA Permissible Exposure Limit 1 ppm

ACGIH RECOMMENDED LIMITS

ACGIH Threshold Limit Value 1 ppm

Immediately Dangerous to Life and Health  60 mg/m3

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  12 mg/m3

DOE Short Term Exposure Limit  12 mg/m3

DOE Ceiling Limit 40 mg/m3

Immediately Dangerous to Life and Health   60 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHTHALIDE    CAS # 87412

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  305.6 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36

Risk Statements: Irritating to eyes.

S: 26-36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHTHALIMIDE    CAS # 85416

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   K   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

ACUTE TOXICTY RISK INDEX 1.5 - LD50 16129.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  302 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Materials to Avoid: S

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHTHALIMIDE POTASSIUM    CAS # 1074824

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid breathing dust. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Store in a cool dry place\. SPECIAL

REQUIREMENTS  Moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHTHALOCYANINE (29,31-)    CAS # 574936

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

NEUROTOXIC - RISK INDEX   5.0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Light sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHTHALOYL CHLORIDE    CAS # 88959   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   1   1   W

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns. Lachrymator.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be harmful

if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE .009900 mm Hg @ 20 °C

FLASH POINT  235 °F

Reacts violently with water

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   REACTIVE   TOXIC

INCOMPATIBILITIES:Water, Alcohols, Oxidizing agents, Strong

FIRE EXTINGUISHER:  Carbon dioxide. Dry chemical powder.  Do not use water.

TOXIC EMISSIONS WHEN BURNED: Hydrogen chloride gas Phosgene gas

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34 37

Risk Statements: Causes burns. Irritating to respiratory system.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .1 ppm

DOE Short Term Exposure Limit  .3 ppm

DOE Ceiling Limit 2 ppm

Immediately Dangerous to Life and Health   10 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHTHALOYLGLYCINE (N-)    CAS # 4702130

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place. Store at 2-8░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHYCOERYTHRIN    CAS # 11016174

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   .   .   .   .   I   J   K   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION HAZARD  INHALATION RISK INDEX   <1  - LC50

S

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

Store at 2-8°C

REACTIVE PROPERTIES

STORAGE Store at 2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: T+

Indication of Danger: Very toxic.

R: 46 5 26/27/28 32

Risk Statements: May cause heritable genetic damage. Heating may

cause an explosion. Very toxic by inhalation, in contact with

skin and if swallowed. Contact with acids liberates very toxic

gas.

S: 53 36/37/39 45

Safety Statements: Avoid exposure - obtain special instructions

before use. Wear suitable protective clothing, gloves, and

eye/face protection. In case of accident or if you feel unwell,

seek medical advice immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHYTAGEL    CAS # 71010521

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 3 - LD50  5000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50   6000.0

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause: Diarrhea. To the best of our knowledge, the

chemical, physical, and toxicological properties have not been

thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Hygroscopic.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHYTIC ACID SODIUM    CAS # 14306253

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 3

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PHYTOL    CAS # 7541493

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 2 - LD50  5000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PIALLYL PALLADIUM(II) CHLORIDE DIMER    CAS # 12012952

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

IRRITATION DATA

skin

Rabbit

100 mg

24H

Remarks: Severe irritation effect

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen chloride gas

Store at 2-8°C

Air and moisture sensitive. Store under inert gas.

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

2-8░C SPECIAL REQUIREMENTS  Air and moisture sensitive. Store under inert

gas.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PICLORAM    CAS # 1918021

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 2.1 - LD50  4200.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SENSITIZATION

Sensitization: Will not occur

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong acids, Acid chlorides, Acid anhydrides,  Strong

bases, Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36

Risk Statements: Irritating to eyes.

S: 26

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PICOLINE (2-)    CAS # 109068

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   2   0

ACUTE TOXICTY RISK INDEX 3 - LD50   790.0 mg/Kg

INHALATION HAZARD

INHALATION RISK INDEX   2.6 - LC50   2790.0

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin. High concentrations are extremely destructive to

tissues of the mucous membranes and upper respiratory tract,

eyes, and skin.

TARGET ORGAN(S) OR SYSTEM(S)

Damage to the liver. Damage to the kidneys.

SIGNS AND SYMPTOMS OF EXPOSURE

Damage to the liver. Damage to the kidneys. Symptoms of exposure

may include burning sensation, coughing, wheezing, laryngitis,

shortness of breath, headache, nausea, and vomiting. Exposure

can cause: Gastrointestinal disturbances.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

VAPOR PRESSURE    8.2 mm Hg @ 20 °C

FLASH POINT  84.2 °F

Forms ignitable mixtures in air at room temperature - Danger of remote

ignition and flashback

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

a - Organic Base/Flammable/Toxic

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

INCOMPATIBILITIES:Oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: and nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Do not get in eyes, on skin, on   clothing. Avoid

prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away from

heat, sparks, and open flame. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 10 20/21/22 36/37

Risk Statements: Flammable. Harmful by inhalation, in contact

with skin and if swallowed. Irritating to eyes and respiratory

system.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  2 ppm

DOE Short Term Exposure Limit  5 ppm

DOE Ceiling Limit 5 ppm

Immediately Dangerous to Life and Health   300 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PICOLINE (4-)    CAS # 108894

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   2   0

ACUTE TOXICTY RISK INDEX 4 - LD50   440.0 mg/Kg

INHALATION HAZARD

INHALATION RISK INDEX   2.6 - LC50   1240.0

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin. High concentrations are extremely destructive to

tissues of the mucous membranes and upper respiratory tract,

eyes, and skin.

TARGET ORGAN(S) OR SYSTEM(S)

Damage to the liver. Damage to the kidneys.

SIGNS AND SYMPTOMS OF EXPOSURE

Damage to the liver. Damage to the kidneys. Symptoms of exposure

may include burning sensation, coughing, wheezing, laryngitis,

shortness of breath, headache, nausea, and vomiting. Exposure

can cause: Gastrointestinal disturbances.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE    4.0 mm Hg @ 20 °C

FLASH POINT  102.2 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

a - Organic Base/Flammable/Toxic

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

INCOMPATIBILITIES:Oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: and nitrogen oxides

REACTIVE PROPERTIES

User Exposure: Avoid contact and inhalation. Avoid prolonged or   repeated

exposure. STORAGE: Keep tightly closed. Keep away from heat and open flame.

Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T

Indication of Danger: Toxic.

R: 10 20/22 24 36/37/38

Risk Statements: Flammable. Harmful by inhalation and if

swallowed. Toxic in contact with skin. Irritating to eyes,

respiratory system and skin.

S: 26 36 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing. In case of accident or if you feel

unwell, seek medical advice immediately (show the label where

possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PICOLINE OXIDE (N,2-)    CAS # 931191

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

Eye Contact: Causes severe eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause: Coughing, chest pains, difficulty in

breathing. Gastrointestinal disturbances. To the best of our

knowledge, the chemical, physical, and toxicological properties

have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  289.4 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Acids, Acid chlorides, Acid anhydrides,  Chloroformates.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid contact and inhalation. Avoid prolonged or    repeated

exposure. STORAGE: Keep tightly closed. Store in a cool dry place\. SPECIAL

REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PICOLINIC ACID (2-)    CAS # 98986

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 2.3 - LD50  2419.4 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

The chemical, physical, and toxicological properties of this

product have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Acids, Bases, Oxidizing agents, Reducing  agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22 36

Risk Statements: Harmful if swallowed. Irritating to eyes.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  10 mg/m3

DOE Short Term Exposure Limit  35 mg/m3

DOE Ceiling Limit 150 mg/m3

Immediately Dangerous to Life and Health   150 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PICOLINIC ACID OXIDE    CAS # 824408

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Materials to Avoid: A

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

STORAGE: Keep tightly closed. Store in a cool dry place\. SPECIAL

REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PICOLYL CHLORIDE HYDROCHLORIDE (2-)    CAS # 6959473

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  3   0   0

ACUTE TOXICTY RISK INDEX 3.2 - LD50   316.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be harmful

if inhaled.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Material is extremely destructive to tissue of the

mucous membranes and upper respiratory tract, eyes, and skin. To

the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Rubber, Plastics.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 22 34

Risk Statements: Harmful if swallowed. Causes burns.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PICOLYLAMINE (3-)    CAS # 3731520

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Vapor or mist is irritating to the eyes, mucous

membranes, and upper respiratory tract.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  226.4 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34 37

Risk Statements: Causes burns. Irritating to respiratory system.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PICRIC ACID    CAS # 88891   HAZARDOUS CHEMICAL OF CONCERN

DEPARTMENT OF HOMELAND SECURITY CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   E   .   .   .   .   J   K   L

NFPA HAZARD CODES (H,F,R,O)  3   1   3

ACUTE TOXICTY RISK INDEX 3.4 - LD50   200.0 mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   1.6 - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: Toxic if absorbed through skin. Readily

absorbed through skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. Toxic if inhaled.

Ingestion: Toxic if swallowed.

SENSITIZATION

skin: May cause allergic skin reaction.

TARGET ORGAN(S) OR SYSTEM(S)

Liver. Kidneys. Blood.

SIGNS AND SYMPTOMS OF EXPOSURE

May cause discoloration of the skin. Picric acid dust causes

sensitization dermatitis. This usually occurs on the face,

especially around the mouth and the sides of the nose; the

condition progresses from edema, through the formation of

papules and vesicles, to ultimate desquamation. Inhalation of

high concentrations of dust has caused unconsciousness,

weakness, muscle pain, and kidney problems. Swallowing picric

acid may cause a bitter taste, headache, dizziness, nausea,

vomiting, and diarrhea. High doses may cause destruction of the

red blood cells and damage to the kidneys and liver with blood

in the urine.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

Explosive

VAPOR PRESSURE .1000 mm Hg @ 20 °C

FLASH POINT  302 °F

Shock sensitive Explosive

SEGREGATION: SHELF # 3

STORAGE GROUP(S):

d - Organic Acid/Flammable/Toxic

k - Explosive/Unstable

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   REACTIVE   TOXIC

INCOMPATIBILITIES:Reducing agents, Heavy metals, Heavy metal

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep container closed. Keep

away from heat, sparks, and open flame\. SPECIAL REQUIREMENTS    Material

must be wet at all times. Do not allow material to    become dry.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: F-T

Indication of Danger: Highly Flammable. Toxic.

R: 1-4-11-23/24/25

Risk Statements: Explosive when dry. Forms very sensitive

explosive metallic compounds Highly flammable. Toxic by

inhalation, in contact with skin and if swallowed.

S: 36/37-35-45

Safety Statements: Wear suitable protective clothing and gloves.

This material and its container must be disposed of in a safe

way. In case of accident or if you feel unwell, seek medical

advice immediately (show the label where possible).

Immediately Dangerous to Life and Health  75 mg/m3

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .1 mg/m3

DOE Short Term Exposure Limit  .3 mg/m3

DOE Ceiling Limit 15 mg/m3

Immediately Dangerous to Life and Health   75 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PICROTOXIN    CAS # 124878

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   0   0

ACUTE TOXICTY RISK INDEX 2.2 - LD50  3250.0 mg/Kg

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

VAPOR PRESSURE .0000100 mm Hg @ 20 °C

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong bases, Strong acids, Strong oxidizing  agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: TION PRODUCTS HAZARDOUS POLYMER

REACTIVE PROPERTIES

HANDLING    on clothing. Avoid prolonged or repeated exposure. STORAGE: Keep

tightly closed\. SPECIAL REQUIREMENTS  Light sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: T+

Indication of Danger: Very toxic.

Risk Statements: Very toxic if swallowed.

S: 36/37/39-28-45

Safety Statements: Wear suitable protective clothing, gloves,

and eye/face protection. After contact with skin, wash

you feel unwell, seek medical advice immediately (show the label

where possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  3 mg/m3

DOE Short Term Exposure Limit  7.5 mg/m3

DOE Ceiling Limit 15 mg/m3

Immediately Dangerous to Life and Health   15 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PIMELIC ACID    CAS # 111160

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

ACUTE TOXICTY RISK INDEX 1.8 - LD50  7000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: Harmful if absorbed through skin.

Eye Contact: Causes eye irritation.

Inhalation: Harmful if inhaled. Material is irritating to mucous

membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

The chemical, physical, and toxicological properties of this

product have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

d - Organic Acid/Flammable/Toxic

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Bases, Oxidizing agents, Reducing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PINACOL    CAS # 76095

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   2   0

ACUTE TOXICTY RISK INDEX 1.6 - LD50 10903.2 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  171 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 38

Risk Statements: Irritating to skin.

S: 37

Safety Statements: Wear suitable gloves.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PINACRYPTOL YELLOW    CAS # 25910854

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Avoid contact and inhalation. Do not get in eyes, on skin, on

clothing. STORAG   Suitable: Keep tightly closed. Store in a cool dry place\.

SPECIAL REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PINACYANOL CHLORIDE    CAS # 2768903

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

Store at 2-8°C

Keep tightly closed. Store in a cool dry place.

REACTIVE PROPERTIES

HANDLING: Avoid contact and inhalation. Do not get in eyes, on skin, on

clothing. STORAGE: Keep tightly closed. Store in a cool dry place\. Store at

2-8°C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PINACYANOL IODIDE    CAS # 605914

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   E   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION HAZARD  INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen iodide

Store at 2-8°C

Keep container closed. Store in a cool dry place.

Light sensitive.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

STORAGE: Keep container closed. Store in a cool dry place\. Store at 2-8°C

SPECIAL REQUIREMENTS  Light sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: T

Indication of Danger: Toxic.

R: 23/24/25

Risk Statements: Toxic by inhalation, in contact with skin and

if swallowed.

S: 36/37/39 45

Safety Statements: Wear suitable protective clothing, gloves,

and eye/face protection. In case of accident or if you feel

unwell, seek medical advice immediately (show the label where

possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PINE OIL    CAS # 8002093

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   G   .   I   .   .   L

NFPA HAZARD CODES (H,F,R,O)  0   2   0

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material is irritating to

mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SENSITIZATION

Sensitization: May cause allergic respiratory and skin reactions

TARGET ORGAN(S) OR SYSTEM(S)

Central nervous system. Kidneys.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

Can cause CNS depression. Exposure can cause: Nausea, headache,

and vomiting. Asthmatic symptoms: cough, hoarseness, rhinorrhea,

wheezing, and conjunctivitis.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  161.6 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Store in a cool dry place.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PINENE (ALPHA-)    CAS # 7785264

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  3   3   0

ACUTE TOXICTY RISK INDEX 1.8 - LD50  8100.0 mg/Kg

INHALATION RISK INDEX   1.5 - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: Harmful if absorbed through skin. Readily

absorbed through skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. Harmful if inhaled.

Ingestion: Harmful if swallowed.

SENSITIZATION

skin: May cause allergic skin reaction.

SIGNS AND SYMPTOMS OF EXPOSURE

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

-

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

VAPOR PRESSURE    3.0 mm Hg @ 20 °C

FLASH POINT  91.4 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid breathing vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep container closed. Keep

away from heat, sparks, and open flame.    -

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn N

Indication of Danger: Harmful. Dangerous for the environment.

R: 10 20/21/22 36/37/38 43 51

Risk Statements: Flammable. Harmful by inhalation, in contact

with skin and if swallowed. Irritating to eyes, respiratory

system and skin. May cause sensitization by skin contact. Toxic

to aquatic organisms.

S: 16 26 36/37 60 61

Safety Statements: Keep away from sources of ignition - no

smoking. In case of contact with eyes, rinse immediately with

plenty of water and seek medical advice. Wear suitable

protective clothing and gloves. This material and its container

must be disposed of as hazardous waste. Avoid release to the

environment. Refer to special instructions/safety data sheets.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PIPECOLIC ACID    CAS # 4043872

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

d - Organic Acid/Flammable/Toxic

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER: WATER SPRAY.    CARBON DIOXIDE, DRY CHEMICAL POWDER OR

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: IDE  NITROGEN OXIDES

REACTIVE PROPERTIES

MINOR AMOUNTS OF ACID CAUSE ACROLEIN TO POYLMERIZE WITH RELEASE OF

HEAT.BENZYL ALCOHOL CONTAINING ACIDIC CONSTITUENTS AND DISSOLVED IRON WAS

FOUND TOPOLYMERIZE WITH A RAPID TEMPERATURE INCREASE WHEN HEATED IN EXCESS

OF100C.(AMINES, PYRIDINE, AND ALKALI HYDROXIDES ACT AS INHIBITORS AND

PREVENTPOLYMERIZATION.) REACT VIGOROUSLY WITH LITHIUM ALUMINUM HYDRIDE.

HIGHCONCENTRATIONS OF ACID REACT EXPLOSIVELY WITH NICKEL NITRIDE AND

HEAT.INITIATE A FAST DECOMPOSITION OF SODIUM OZONATE. REACTS WITH

THORIUMPHOSPHIDE TO RELEASE SPONTANEOUSLY FLAMMABLE PHSOPHINE. REACTS

VIOLENTLY WITHTRI-ISO-BUTYL ALUMINUM.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PIPECOLIC ACID HYDROCHLORIDE (DL-)    CAS # 5107108

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Avoid contact and inhalation. Do not get in eyes, on skin, on

clothing. STORAGE: Keep tightly closed. Store in a cool dry place\. SPECIAL

REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PIPECOLINIC ACID (L-)    CAS # 3105951

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: Causes eye and skin irritation. May be harmful

by inhalation, ingestion, or skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid contact and inhalation. Do not get in eyes, on skin, on

clothing. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26-36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PIPEMIDIC ACID    CAS # 51940444

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 1.5 - LD50 16000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

d - Organic Acid/Flammable/Toxic

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

STORAGE

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PIPERAZINE    CAS # 110850   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  2   2   0

ACUTE TOXICTY RISK INDEX 4 - LD50  1900.0 mg/Kg

INHALATION RISK INDEX   1.0 - LC50   1674.0

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. Harmful if

inhaled.

Ingestion: Harmful if swallowed.

SENSITIZATION

Respiratory: May cause allergic respiratory reaction.

skin: May cause allergic skin reaction.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Repeated exposure may cause asthma.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

VAPOR PRESSURE .160 mm Hg @ 20 °C

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place. Store under nitrogen\. SPECIAL REQUIREMENTS  Store under

inert gas. Light sensitive. Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34-42/43-52/53

Risk Statements: Causes burns. May cause sensitization by

inhalation and skin contact. Harmful to aquatic organisms, may

cause long-term adverse effects in the aquatic environment.

S: 22-26-36/37/39-45-61

Safety Statements: Do not breathe dust. In case of contact with

eyes, rinse immediately with plenty of water and seek medical

advice. Wear suitable protective clothing, gloves, and eye/face

protection. In case of accident or if you feel unwell, seek

medical advice immediately (show the label where possible).

Avoid release to the environment. Refer to special

instructions/safety data sheets.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  2 mg/m3

DOE Short Term Exposure Limit  6 mg/m3

DOE Ceiling Limit 40 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PIPERAZINE (1,BOC-)    CAS # 57260716

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: Causes eye and skin irritation. May be harmful

by inhalation, ingestion, or skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  235 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Store in a cool dry place\. SPECIAL

REQUIREMENTS  Hygroscopic. Handle under inert gas.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26-36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PIPERAZINE BISETHANESULFONIC ACID    CAS # 5625376

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

d - Organic Acid/Flammable/Toxic

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Sulfur oxides Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PIPERAZINE HEXAHYDRATE    CAS # 142632

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  3   2   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be harmful

if inhaled.

Ingestion: May be harmful if swallowed.

SENSITIZATION

Respiratory: May cause allergic respiratory reaction.

skin: May cause allergic skin reaction.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Repeated exposure may cause asthma. To the best of our

knowledge, the chemical, physical, and toxicological properties

have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  190 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids, Acid chlorides, Acid

anhydrides.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Light sensitive. Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34 42/43 52/53

Risk Statements: Causes burns. May cause sensitization by

inhalation and skin contact. Harmful to aquatic organisms, may

cause long-term adverse effects in the aquatic environment.

S: 22 26 36/37/39 45 61

Safety Statements: Do not breathe dust. In case of contact with

eyes, rinse immediately with plenty of water and seek medical

advice. Wear suitable protective clothing, gloves, and eye/face

protection. In case of accident or if you feel unwell, seek

medical advice immediately (show the label where possible).

Avoid release to the environment. Refer to special

instructions/safety data sheets.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PIPERAZINECARBOXALDEHYDE (1-)    CAS # 7755922

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material is irritating to

mucous membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  215 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22 37/38 41

Risk Statements: Harmful if swallowed. Irritating to respiratory

system and skin. Risk of serious damage to eyes.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PIPERIDINE    CAS # 110894

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  3   3   0

ACUTE TOXICTY RISK INDEX 3.1 - LD50   400.0 mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   3.3 - LC50   6500.0

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: Toxic if absorbed through skin.

Eye Contact: Causes burns.

Inhalation: Toxic if inhaled. Material is extremely destructive

to the tissue of the mucous membranes and upper respiratory

tract.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

VAPOR PRESSURE   33.7 mm Hg @ 20 °C

FLASH POINT  60.8 °F

Forms ignitable mixtures in air at room temperature - Danger of remote

ignition and flashback

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

a - Organic Base/Flammable/Toxic

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

INCOMPATIBILITIES:N-perchlorylpiperazine, N-nitrosoacetanilide,

Dicyanofurazan, Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: and nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep container closed. Keep

away from heat, sparks, and open flame.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: F-T

Indication of Danger: Highly Flammable. Toxic.

R: 11-23/24-34

Risk Statements: Highly flammable. Toxic by inhalation and in

contact with skin. Causes burns.

S: 16-26-27-45

Safety Statements: Keep away from sources of ignition - no

smoking. In case of contact with eyes, rinse immediately with

plenty of water and seek medical advice. Take off immediately

all contaminated clothing. In case of accident or if you feel

unwell, seek medical advice immediately (show the label where

possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  1 ppm

DOE Short Term Exposure Limit  6.6 ppm

DOE Ceiling Limit 33 ppm

Immediately Dangerous to Life and Health   110 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PIPERIDINE HYDROCHLORIDE    CAS # 6091447

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin. May cause irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  235 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: T

Indication of Danger: Toxic.

R: 25 37

Risk Statements: Toxic if swallowed. Irritating to respiratory

system.

S: 36/37/39 45

Safety Statements: Wear suitable protective clothing, gloves,

and eye/face protection. In case of accident or if you feel

unwell, seek medical advice immediately (show the label where

possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PIPERIDINEETHANOL (N-)    CAS # 3040446

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   2   0

ACUTE TOXICTY RISK INDEX 2.5 - LD50  1498.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material is irritating to

mucous membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  170.6 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22 36/37/38

Risk Statements: Harmful if swallowed. Irritating to eyes,

respiratory system and skin.

S: 36/37/39

Safety Statements: Wear suitable protective clothing, gloves,

and eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PIPERIDINEMETHANOL (3-)    CAS # 4606659

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be harmful

if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Material is extremely destructive to tissue of the

mucous membranes and upper respiratory tract, eyes, and skin. To

the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  235 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34

Risk Statements: Causes burns.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PIPERIDINEMETHANOL (4-)    CAS # 6457494

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be harmful

if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Material is extremely destructive to tissue of the

mucous membranes and upper respiratory tract, eyes, and skin. To

the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  230 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34

Risk Statements: Causes burns.

S: 26 27 36/37/39

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Take

off immediately all contaminated clothing. Wear suitable

protective clothing, gloves, and eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PIPERIDINEMETHANOL (N,BOC,4-)    CAS # 123855516

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   F   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   CORROSIVE

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PIPERIDINEPROPIONIC ACID (1-)    CAS # 26371073

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 37/39

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable gloves and eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PIPERIDONE MONOHYDRATE HYDROCHLORIDE (4-)    CAS # 40064344

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

Store at 2-8°C

May develop pressure. Open carefully.

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

2-8░C SPECIAL REQUIREMENTS    May develop pressure. Open carefully.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PIPERONAL    CAS # 120570

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 2.3 - LD50  2700.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes severe eye irritation.

Inhalation: May be harmful if inhaled. Material may be

Ingestion: Harmful if swallowed.

SENSITIZATION

Sensitization: Prolonged or repeated exposure may cause allergic

reactions in certain sensitive individuals.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

Oral

Rat

1,900 mg/kg

LD50

Oral

Mouse

LD50

IRRITATION DATA

Eyes

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

VAPOR PRESSURE .007100 mm Hg @ 20 °C

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: N PRODUCTS HAZARDOUS POLYMERIZATI

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, STORAGE: Keep tightly

closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

R: 38

Risk Statements: Irritating to skin.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PIPERONYL ALCOHOL    CAS # 495761

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  230 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PIPERONYLAMINE    CAS # 2620500

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Absorption into the body leads to the formation of methemoglobin

which in sufficient concentration causes cyanosis. Onset may be

delayed 2 to 4 hours or longer. To the best of our knowledge,

the chemical, physical, and toxicological properties have not

been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  230 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Acids, Acid chlorides, Acid anhydrides,  Chloroformates,

Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PIPERONYLOYL CHLORIDE    CAS # 25054539

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place. Store under nitrogen\. SPECIAL REQUIREMENTS  Moisture

sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34

Risk Statements: Causes burns.

S: 22 26 27 28 36/37/39 45

Safety Statements: Do not breathe dust. In case of contact with

eyes, rinse immediately with plenty of water and seek medical

advice. Take off immediately all contaminated clothing. After

contact with skin, wash immediately with plenty of water. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PIPES DISODIUM SALT    CAS # 76836027

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PIPSYL CHLORIDE    CAS # 98613   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   1   W

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Reacts violently with water

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

WASTE CHARACTERISTIC HAZARD:   REACTIVE

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Do not use water.

TOXIC EMISSIONS WHEN BURNED: Sulfur oxides Hydrogen chloride gas Hydrogen

iodide

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place\. SPECIAL REQUIREMENTS  Moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34

Risk Statements: Causes burns.

S: 26 27 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Take

off immediately all contaminated clothing. Wear suitable

protective clothing, gloves, and eye/face protection. In case of

accident or if you feel unwell, seek medical advice immediately

(show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PIROMIDIC ACID    CAS # 19562302

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 2 - LD50  5000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin. May cause irritation.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

d - Organic Acid/Flammable/Toxic

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER: foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

Store at 2-8°C

REACTIVE PROPERTIES

STORAGE Store at 2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

R: 20/21/22

Risk Statements: Harmful by inhalation, in contact with skin and

if swallowed.

S: 36

Safety Statements: Wear suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PIVALIC ACID    CAS # 75989

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  3   2   0

ACUTE TOXICTY RISK INDEX 2.7 - LD50   900.0 mg/Kg

INHALATION RISK INDEX   1.8 - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: Harmful if absorbed through skin.

Eye Contact: Causes burns.

Inhalation: May be harmful if inhaled. Material is extremely

destructive to the tissue of the mucous membranes and upper

respiratory tract.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

VAPOR PRESSURE .821 mm Hg @ 20 °C

FLASH POINT  147.2 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

d - Organic Acid/Flammable/Toxic

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Bases, Reducing  agents.

FIRE EXTINGUISHER:  Water spray.

Store at 2-8°C

Keep tightly closed. Keep away from heat and open flame. Store in a cool dry

place.

REACTIVE PROPERTIES

HANDLING: Avoid breathing dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame. Store in a cool dry place. Store at 2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 21/22 34

Risk Statements: Harmful in contact with skin and if swallowed.

Causes burns.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PLASMIN    CAS # 9001905

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  3   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material is irritating to

mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SENSITIZATION

Respiratory: May cause allergic respiratory reaction.

SIGNS AND SYMPTOMS OF EXPOSURE

Potentially biohazardous material.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Use extinguishing media appropriate to surrounding fire

conditions.

Store at -20°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

-20░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: B Xn

Indication of Danger: Biohazard. Harmful.

R: 36/37/38 42

Risk Statements: Irritating to eyes, respiratory system and

skin. May cause sensitization by inhalation.

S: 2 22 24 26 36/37 45

Safety Statements: Keep out of the reach of children. Do not

breathe dust. Avoid contact with skin. In case of contact with

eyes, rinse immediately with plenty of water and seek medical

advice. Wear suitable protective clothing and gloves. In case of

accident or if you feel unwell, seek medical advice immediately

(show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PLATINUM    CAS # 7440064

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  1   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SENSITIZATION

Sensitization: Prolonged or repeated exposure may cause allergic

reactions in certain sensitive individuals.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, an   toxicological

properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Alcohols.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Platinum oxide

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  1 mg/m3

DOE Short Term Exposure Limit  3 mg/m3

DOE Ceiling Limit 4 mg/m3

Immediately Dangerous to Life and Health   4 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PLATINUM    CAS # 88

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  1   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SENSITIZATION

Sensitization: Prolonged or repeated exposure may cause allergic

reactions in certain sensitive individuals.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, an   toxicological

properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Alcohols.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Platinum oxide

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PLATINUM OXIDE    CAS # 52785065

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   1   OX

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

IRRITATION DATA

skin

Rabbit

Remarks: No irritation effect

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Oxidizing Agent

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Organic materials, Finely powdered metals.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from combustible   materials, heat, sparks, and open flame.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: O Xi

Indication of Danger: Oxidizing. Irritant.

R: 8 36

Risk Statements: Contact with combustible material may cause

fire. Irritating to eyes.

S: 26

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice.

Immediately Dangerous to Life and Health  4 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PLATINUM(II) CHLORIDE    CAS # 10025657

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  3   0   1

ACUTE TOXICTY RISK INDEX 2.2 - LD50  3423.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be harmful

if inhaled.

Ingestion: May be harmful if swallowed.

SENSITIZATION

Sensitization: May cause allergic respiratory and skin reactions

SIGNS AND SYMPTOMS OF EXPOSURE

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Material is extremely destructive to tissue of the

mucous membranes and upper respiratory tract, eyes, and skin.

Exposure can cause: Dermatitis. Asthma. To the best of our

knowledge, the chemical, physical, and toxicological properties

have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Noncombustible. Use extinguishing media appropriate

tosurrounding fire conditions.

TOXIC EMISSIONS WHEN BURNED: Hydrogen chloride gas Chlorine

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34 42/43

Risk Statements: Causes burns. May cause sensitization by

inhalation and skin contact.

S: 26 27 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Take

off immediately all contaminated clothing. Wear suitable

protective clothing, gloves, and eye/face protection. In case of

accident or if you feel unwell, seek medical advice immediately

(show the label where possible).

Immediately Dangerous to Life and Health  4 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PLATINUM(II) DIAMMINE DICHLORIDE (CIS-)    CAS # 15663271

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   B   C   .   .   .   G   .   .   J   .   L

NFPA HAZARD CODES (H,F,R,O)  3   0   0

HUMAN TERATOGEN - DESIGNATED AREA MAY BE REQUIRED

EXTREMELY TOXIC - DESIGNATED AREA MAY BE REQUIRED

ACUTE TOXICTY RISK INDEX 4.3 - LD50    25.8 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes severe eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be fatal if swallowed.

SENSITIZATION

Sensitization: Prolonged or repeated exposure may cause allergic

reactions in certain sensitive individuals.

TARGET ORGAN(S) OR SYSTEM(S)

Damage to the liver. Kidneys. Ears. Blood. Peripheral nervous

system. Bone marrow. Testes. Female reproductive system.

SIGNS AND SYMPTOMS OF EXPOSURE

Can cause severe cumulative renal toxicity, bone marrow

suppression and ototoxicity. Damage to the liver. Prolonged

exposure can cause: Nausea, vomiting, diarrhea.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen  chloride gas Ammonia

Platinum oxide

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PLATINUM(IV) OXIDE    CAS # 1314154

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   1   OX

ACUTE TOXICTY RISK INDEX 1.8 - LD50  8000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Oxidizing Agent

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Organic materials, Finely powdered metals.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

TOXIC EMISSIONS WHEN BURNED: Platinum oxide

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, STORAGE: Keep

tightly closed. Keep away from combustible    materials, heat, sparks, and

open flame.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: O Xi

Indication of Danger: Oxidizing. Irritant.

R: 8 36

Risk Statements: Contact with combustible material may cause

fire. Irritating to eyes.

S: 26

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice.

Immediately Dangerous to Life and Health  4 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PLATINUM(IV) OXIDE MONOHYDRATE    CAS # 12137212

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   F   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   1   OX

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Oxidizing Agent

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

WASTE CHARACTERISTIC HAZARD:   TOXIC   CORROSIVE

INCOMPATIBILITIES:Organic materials, Finely powdered metals.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Platinum oxide

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from combustible    materials, heat, sparks, and open flame.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: O Xi

Indication of Danger: Oxidizing. Irritant.

R: 8 36

Risk Statements: Contact with combustible material may cause

fire. Irritating to eyes.

S: 26

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PLUMBAGIN    CAS # 481425

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  3   0   0

ACUTE TOXICTY RISK INDEX 3.9 - LD50    65.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases, Strong  reducing

agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: MERIZATION  Hazardous Polymerization: Will not

occurROUTE OF EXPOS

REACTIVE PROPERTIES

HANDLING    on clothing. Avoid prolonged or repeated exposure. STORAGE: Keep

tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: T

R: 25 34

Risk Statements: Toxic if swallowed. Causes burns.

S: 22 26 36/37/39 45

Safety Statements: Do not breathe dust. In case of contact with

advice. Wear suitable protective clothing, gloves, and eye/face

protection. In case of accident or if you feel unwell, seek

medical advice immediately (show the label where possible).

Indication of Danger: Toxic.

Risk Statements: Toxic if swallowed. Causes burns.

Safety Statements: Do not breathe dust. In case of contact with

eyes, rinse immediately with plenty of water and seek medical

protection. In case of accident or if you feel unwell, seek

medical advice immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYACRYLAMIDE    CAS # 9003058

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 2.7 - LD50  1000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Store in a cool dry place.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYAMIDE 6    CAS # 25038544

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   C   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

EXTREMELY TOXIC - DESIGNATED AREA MAY BE REQUIRED

ACUTE TOXICTY RISK INDEX 3 - LD50         mg/Kg

INHALATION RISK INDEX   <1  - LC50   3410.0

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Do not get in eyes, on skin, on STORAGE: Keep

tightly closed. Store in a cool dry place\. SPECIAL REQUIREMENTS  Moisture

sensitive.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  10 mg/m3

DOE Short Term Exposure Limit  30 mg/m3

DOE Ceiling Limit 200 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYANILINE    CAS # 25233301

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYBUFFER    CAS # 82658864

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at 2-8°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure.STORAGE: Keep tightly closed\. Store at

2-8°C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYBUFFER 74    CAS # 82658853

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at 2-8°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure.STORAGE: Keep tightly closed\. Store at

2-8°C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYBUFFER EXCHANGER 94

CAS # 106717300

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   3   1

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material is irritating to

mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Nerves. Liver. Heart.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

FLASH POINT  95 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at 2-8°C

Keep container closed. Keep away from heat, sparks, and open flame.

REACTIVE PROPERTIES

HANDLING: Avoid contact with eyes, skin, and clothing. Do   not breathe

vapor. Avoid prolonged or repeated exposure. STORAGE: Keep container closed.

Keep away from heat, sparks, and open flame\. Store at 2-8°C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 10 36/37/38

Risk Statements: Flammable. Irritating to eyes, respiratory

system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYDEOXYINOSINIC-DEOXYCYTIDYLIC ACID    CAS # 118578373

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. May cause irritation.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:   TOXIC

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

-20░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYDIMETHYLSILOXANE    CAS # 9016006

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE    5.0 mm Hg @ 20 °C

FLASH POINT  275 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Silicon oxide

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  7.5 mg/m3

DOE Short Term Exposure Limit  25 mg/m3

DOE Ceiling Limit 150 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYETHYLENE    CAS # 9002884

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

CONDITIONS AGGRAVATED BY EXPOSURE

Exposure to thermal decomposition products can cause "polymer

fume fever", a temporary flu-like condition which usually

appears several hours after exposure and passes within 36-48

hours.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Acrolein Formaldehyde

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22

Safety Statements: Do not breathe dust.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  10 mg/m3

DOE Short Term Exposure Limit  30 mg/m3

DOE Ceiling Limit 50 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYETHYLENE GLYCOL    CAS # 37225266

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE .010000 mm Hg @ 20 °C

FLASH POINT  525 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids, Strong bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  10 mg/m3

DOE Short Term Exposure Limit  30 mg/m3

DOE Ceiling Limit 500 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYETHYLENE GLYCOL DISTEARATE    CAS # 9005087

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  235 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYETHYLENE GLYCOL-B-POLYPROPYLENE GLYCOL    CAS # 9003116

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   1   0

ACUTE TOXICTY RISK INDEX 1.5 - LD50 15000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Ingestion can cause: Diarrhea. Weakness. To the best of our

knowledge, the chemical, physical, and toxicological properties

have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

VAPOR PRESSURE .300 mm Hg @ 20 °C

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 23 24/25

Safety Statements: Do not breathe vapor. Avoid contact with skin

and eyes.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  10 mg/m3

DOE Short Term Exposure Limit  30 mg/m3

DOE Ceiling Limit 200 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYETHYLENIMINE    CAS # 25987068

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  235 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Hygroscopic.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYGLUTAMIC ACID SODIUM (D-)    CAS # 30811791

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure may affect cell growth rate and morphology. To the best

of our knowledge, the chemical, physical, and toxicological

properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong acids, Strong bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

Store at -20°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. Store at

-20°C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYHISTIDINE (L-)    CAS # 26062486

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

Exposure may affect cell growth rate and morphology.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Materials to Avoid: S

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

Store at -20°C

Keep tightly closed. Store in a cool dry place.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place. Store at -20░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYHYDROXYBUTYRIC ACID    CAS # 29435481

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 2 - LD50  5000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

S

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

d - Organic Acid/Flammable/Toxic

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

STORAGE

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYLYSINE HYDROBROMIDE (D-)    CAS # 27964994

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure may affect cell growth rate and morphology. To the best

of our knowledge, the chemical, physical, and toxicological

properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong acids, Strong bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen bromide gas

Store at -20°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. Store at

-20░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22-24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYLYSINE HYDROBROMIDE (l-)    CAS # 25988630

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure may affect cell growth rate and morphology.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen bromide gas

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. Store at

-20°C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYLYSINE HYDROCHLORIDE (L-)    CAS # 26124787

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure may affect cell growth rate and morphology. To the best

properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam\. Protective Equipment: Wear self-contained breathing

apparatus and protective clothing to prevent contact with skin and eyes.

Specific Hazard(s): Emits t

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, STORAGE: Keep

tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYMALEIMIDE    CAS # 25721748

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

INHALATION RISK INDEX   <1  - LC50

SIGNS AND SYMPTOMS OF EXPOSURE

The chemical, physical, and toxicological properties of this

product have not been thoroughly investigated.

CONDITIONS AGGRAVATED BY EXPOSURE

While low molecular weight polyelectrolytes such as

carboxyimamidate show low toxicity and antitumor properties,

toxicity may increase with higher molecular weight. Bioactive

polymers and may cause liver damage. (ref:J Med. Chem., 25,

1060-1064, 1982; Pure & Appl. Chem., 46, 103-113, 1976)

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: and nitrogen oxides

Store at -20°C

REACTIVE PROPERTIES

STORAGE Store at -20░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYMETHYL METHACRYLATE    CAS # 9011147

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong acids, Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  5 mg/m3

DOE Short Term Exposure Limit  15 mg/m3

DOE Ceiling Limit 100 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYMETHYL VINYL ETHER/MALEIC ANHYDRIDE    CAS # 9011169

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   I   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 3.3 - LD50   268.0 mg/Kg

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: Toxic if absorbed through skin.

Eye Contact: Causes severe eye irritation.

Inhalation: Vapor or mist is irritating to the mucous membranes

and upper respiratory tract. Toxic if inhaled.

Ingestion: Toxic if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Female reproductive system. Eyes. Bone marrow. Blood.

SIGNS AND SYMPTOMS OF EXPOSURE

Inhalation of high concentrations of benzene may have an initial

stimulatory effect on the central nervous system characterized

by exhilaration, nervous excitation and/or giddiness,

depression, drowsiness, or fatigue. The victim may experience

tightness in the chest, breathlessness, and loss of

consciousness. Tremors, convulsions, and death due to

respiratory paralysis or circulatory collapse can occur in a few

minutes to several hours following severe exposures. Aspiration

of small amounts of liquid immediately causes pulmonary edema

and hemorrhage of pulmonary tissue. Direct skin contact may

cause erythema. Repeated or prolonged skin contact may result in

drying, scaling dermatitis, or development of secondary skin

infections. The chief target organ is the hematopoietic system.

Bleeding from the nose, gums, or mucous membranes and the

development of purpuric spots, pancytopenia, leukopenia,

thrombocytopenia, aplastic anemia, and leukemia may occur as the

condition progresses. The bone marrow may appear normal,

aplastic or hyperplastic, and may not correlate with peripheral

blood-forming tissues. The onset of effects of prolonged benzene

exposure may be delayed for many months or years after the

actual exposure has ceased. Narcotic effect. Exposure can cause:

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: T

Indication of Danger: Toxic.

R: 45-46-48/20/21/22

Risk Statements: May cause cancer. May cause heritable genetic

damage. Also harmful: danger of serious damage to health by

prolonged exposure through inhalation, in contact with skin and

if swallowed.

S: 53-36/37-45

Safety Statements: Restricted to professional users. Attention -

Avoid exposure - obtain special instructions before use. Wear

suitable protective clothing and gloves. In case of accident or

if you feel unwell, seek medical advice immediately (show the

label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYMYXIN B NONAPEPTIDE    CAS # 86408368

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

Store at 2-8°C

REACTIVE PROPERTIES

STORAGE Store at 2-8░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYMYXIN B SULFATE    CAS # 1405205

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

ACUTE TOXICTY RISK INDEX 2.3 - LD50  2548.4 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause: Stomach pains, vomiting, diarrhea. To the

best of our knowledge, the chemical, physical, and toxicological

properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Materials to Avoid: S

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxides

Store at 2-8°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22

Risk Statements: Harmful if swallowed.

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYOXYETHYLENE ETHER    CAS # 25322683

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

ACUTE TOXICTY RISK INDEX 1.2 - LD50 30000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE .010000 mm Hg @ 20 °C

FLASH POINT  525 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids, Strong bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  10 mg/m3

DOE Short Term Exposure Limit  60 mg/m3

DOE Ceiling Limit 500 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYOXYETHYLENE TRIDECYL ETHER    CAS # 24938918

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

IRRITATION DATA

skin

Rabbit

2,000 mg

4W

Remarks: Mild irritation effect

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  230 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYPHOSPHORIC ACID    CAS # 2466093

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 2.6 - LD50  1170.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

TARGET ORGAN(S) OR SYSTEM(S)

Bone marrow. Blood. Liver.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

May cause cyanosis (blue-gray coloring of skin and lips caused

by lack of oxygen). Symptoms of exposure may include burning

sensation, coughing, wheezing, laryngitis, shortness of breath,

headache, nausea, and vomiting. Inhalation may result in spasm,

inflammation and edema of the larynxand bronchi, chemical

pneumonitis, and pulmonary edema. Material is extremely

destructive to tissue of the mucous membranes and upper

respiratory tract, eyes, and skin.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Bases Reacts with water to generate heat and form posphoric

acid. The reaction is not violent

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Phosphoric acid Thermal decomposition may

produce toxic fumes of phosphorus oxides and/orphosphine

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Do not get in eyes, on skin, on   clothing. Avoid

prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34

Risk Statements: Causes burns.

S: 26-36/37/39-45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYPHOSPHORIC ACID    CAS # 8017161

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

TARGET ORGAN(S) OR SYSTEM(S)

Bone marrow. Blood. Liver.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

May cause cyanosis (blue-gray coloring of skin and lips caused

by lack of oxygen). Symptoms of exposure may include burning

sensation, coughing, wheezing, laryngitis, shortness of breath,

headache, nausea, and vomiting. Inhalation may result in spasm,

inflammation and edema of the larynxand bronchi, chemical

pneumonitis, and pulmonary edema. Material is extremely

destructive to tissue of the mucous membranes and upper

respiratory tract, eyes, and skin.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Bases Reacts with water to generate heat and form posphoric

acid. The reaction is not violent

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Phosphoric acid Thermal decomposition may

produce toxic fumes of phosphorus oxides and/orphosphine

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Do not get in eyes, on skin, on   clothing. Avoid

prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34

Risk Statements: Causes burns.

S: 26-36/37/39-45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  1.5 mg/m3

DOE Short Term Exposure Limit  5 mg/m3

DOE Ceiling Limit 40 mg/m3

Immediately Dangerous to Life and Health   200 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYPYRROLE    CAS # 51325032

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Vapor or mist is irritating to the eyes, mucous

membranes, and upper respiratory tract.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause: Headache and dizziness, potentially fatal

apparent hypersensitiveity syndrome. To the best of our

knowledge, the chemical, physical, and toxicological properties

have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  235 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxides

Store at 2-8°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Store at 2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYPYRROLE, DOPED    CAS # 30604810

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 37

Risk Statements: Irritating to respiratory system.

S: 22 36

Safety Statements: Do not breathe dust. Wear suitable protective

clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYSODIUM STYRENESULFONATE (4-)    CAS # 25704181

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 1.8 - LD50  8000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Sulfur oxides Sodium/sodium oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  150 mg/m3

DOE Short Term Exposure Limit  500 mg/m3

DOE Ceiling Limit 500 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYSTYRENE-CO-DIVINYL BENZENE    CAS # 69011149

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

-

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Alkylbenzenes vinylbenzenes naphthalene

benzaldehydes andphenol may also form

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.    -

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  10 mg/m3

DOE Short Term Exposure Limit  30 mg/m3

DOE Ceiling Limit 50 mg/m3

Immediately Dangerous to Life and Health   250 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYSTYRENE/MALEIC ANHYDRIDE    CAS # 9011136

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 1.4 - LD50 21000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER: WATER SPRAY.    CARBON DIOXIDE, DRY CHEMICAL POWDER OR

appropriate foam.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYVINYL FORMAL    CAS # 9003332

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Flammable

FLASH POINT  55.4 °F

Forms ignitable mixtures in air at room temperature - Danger of remote

ignition and flashback

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE

INCOMPATIBILITIES:Materials to Avoid: S

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Store in a cool dry place.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYVINYL FORMVAR    CAS # 850430

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 1.7 - LD50 10000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ORL-RAT LD50 >10G/KG. ORL-RBT LD50 >10G/KG.

ONLY SELECTED REGISTRY OF TOXIC EFFECTS OF CHEMICAL SUBSTANCES

(RTECS) DATA IS PRESENTED HERE. SEE ACTUAL ENTRY IN RTECS FOR

COMPLETE INFORMATION.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYVINYLCARBAZOLE    CAS # 25067598

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 2 - LD50  5000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Materials to Avoid: S

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Store in a cool dry place.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYVINYLIDENE FLUORIDE    CAS # 24937799

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

VAPOR PRESSURE   15.0 mm Hg @ 20 °C

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Silica, Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen fluoride

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22-24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYVINYLIDINE FLUORIDE    CAS # 9011170

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Titanium/titanium oxides, Silica.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen fluoride

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  10 mg/m3

DOE Short Term Exposure Limit  30 mg/m3

DOE Ceiling Limit 50 mg/m3

Immediately Dangerous to Life and Health   250 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYVINYLNAPHTHALENE (2-)    CAS # 28406566

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

User Exposure: Avoid inhalation. Avoid contact with eyes, skin, and

clothing. Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYVINYLPOLYPYRROLIDONE    CAS # 25249541

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Unexcreted particles may be phagocytized by cells of the

reticuloendothelial system and deposited in storage sites in the

liver, spleen, lung, and bone marrow resulting in the storage

disease thesaurosis. Severity and symptoms depend on storage

site and nature of the particle. Pathological changes are not

necessarily attributed to the thesaurosis, but in some cases an

inflammation or granulomatoma have occurred.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYVINYLPYRIDINE (4-)    CAS # 9017407

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 3.7 - LD50   100.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

a - Organic Base/Flammable/Toxic

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POLYVINYLPYRROLIDONE    CAS # 9003398

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

ACUTE TOXICTY RISK INDEX .7 - LD50******** mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SENSITIZATION

Sensitization: Will not occur

SIGNS AND SYMPTOMS OF EXPOSURE

Unexcreted particles may be phagocytized by cells of the

reticuloendothelial system and deposited in storage sites in the

liver, spleen, lung, and bone marrow resulting in the storage

disease thesaurosis. Severity and symptoms depend on storage

site and nature of the particle. Pathological changes are not

necessarily attributed to the thesaurosis, but in some cases an

inflammation or granulomatoma have occurred.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  100 mg/m3

DOE Short Term Exposure Limit  300 mg/m3

DOE Ceiling Limit 500 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PONCEAU S    CAS # 6226795

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POPSO FREE ACID    CAS # 68189435

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM    CAS # 7440097   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   3   1   W

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: May be harmful if inhaled. Material is extremely

destructive to the tissue of the mucous membranes and upper

respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Flammable

VAPOR PRESSURE .010000 mm Hg @ 20 °C

Reacts violently with water

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   REACTIVE   TOXIC

INCOMPATIBILITIES:Oxidizing agents Carbon monoxide, Carbon dioxide Reacts

violently with water., Material readily reacts with

FIRE EXTINGUISHER:  Use approved class D extinguishers or smother with

drysand, dry ground limestone, or dry clay Do not use water. Do not use

carbon dioxide   extinguisher on this material.

REACTIVE PROPERTIES

HANDLING: Avoid contact with eyes, skin, and clothing.   Avoid breathing

dust. Avoid prolonged or repeated exposure. STORAGE: Keep container closed.

Keep away from heat, sparks, and open flame\. SPECIAL REQUIREMENTS Handle and

store under inert gas.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: F C

Indication of Danger: Highly Flammable. Corrosive.

R: 14/15 34

Risk Statements: Reacts violently with water, liberating

extremely flammable gases. Causes burns.

S: 8 43 45

Safety Statements: Keep container dry. In case of fire, use

special powder for metal fire. Never use water. In case of

accident or if you feel unwell, seek medical advice immediately

(show the label where possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  2 mg/m3

DOE Short Term Exposure Limit  2 mg/m3

DOE Ceiling Limit 2 mg/m3

Immediately Dangerous to Life and Health   10 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM ACETATE    CAS # 127082

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   F   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 2.2 - LD50  3250.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   CORROSIVE

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Dry chemical powder.

REACTIVE PROPERTIES

HANDLING    and clothing. Avoid prolonged or repeated exposure. STORAGE: Keep

tightly closed. SPECIAL REQUIREMENTS  Hygroscopic.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  12.5 mg/m3

DOE Short Term Exposure Limit  40 mg/m3

DOE Ceiling Limit 250 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM ANTIMONY TARTRATE HYDRATE    CAS # 28300745

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 3.6 - LD50   115.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: Toxic if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Liver. Kidneys. Heart.

SIGNS AND SYMPTOMS OF EXPOSURE

Potassium antimony tartrate is the most potent trivalent

antimony compound. Trivalent antimony compounds are more toxic

than the pentavalent because they are excreted slowly. The most

serious adverse effects are on the heart and liver along with

coughing, chest and abdominal pain, vomiting, fainting, and

collapse. Less immediate adverse effects include:

Gastrointestinal disturbances. Headache. Dizziness. Weakness.

Damage to the kidneys.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Mineral acids, Strong bases, Carbonates, Lead, Silver

salts, Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Antimony/antimony oxides Potassium oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: Xn N

Indication of Danger: Harmful. Dangerous for the environment.

R: 20/22 51/53

Risk Statements: Harmful by inhalation and if swallowed. Toxic

to aquatic organisms, may cause long-term adverse effects in the

aquatic environment.

S: 61

Safety Statements: Avoid release to the environment. Refer to

special instructions/safety data sheets.

Immediately Dangerous to Life and Health  50 mg/m3

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  1.31 mg/m3

DOE Short Term Exposure Limit  3.92 mg/m3

DOE Ceiling Limit 6.53 mg/m3

Immediately Dangerous to Life and Health   131 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM BICARBONATE    CAS # 298146

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   C   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

EXTREMELY TOXIC - DESIGNATED AREA MAY BE REQUIRED

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Noncombustible. Use extinguishing media appropriate

tosurrounding fire conditions.

TOXIC EMISSIONS WHEN BURNED: Potassium oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  12.5 mg/m3

DOE Short Term Exposure Limit  35 mg/m3

DOE Ceiling Limit 60 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM BIPHTHALATE                                 CAS # 877257

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 2.2 - LD50  3200.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER: WATER SPRAY.    CARBON DIOXIDE, DRY CHEMICAL POWDER OR

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: IDE

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM BITARTRATE    CAS # 868144

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  150 mg/m3

DOE Short Term Exposure Limit  500 mg/m3

DOE Ceiling Limit 500 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM BORATE    CAS # 12229139

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   1   W

INHALATION RISK INDEX   <1  - LC50

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Reacts violently with water

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

WASTE CHARACTERISTIC HAZARD:   REACTIVE

FIRE EXTINGUISHER: USE APPROVED CLASS D EXTINGUISHERS OR SMOTHER WITH DRY

SAND, DRY    GROUND LIMESTONE OR DRY CLAY.    DO NOT USE WATER.

TOXIC EMISSIONS WHEN BURNED: HAZARDOUS COMBUSTION OR DECOMPOSITION PRODUCTS

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM BORATE    CAS # 1332770

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   1   W

INHALATION RISK INDEX   <1  - LC50

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Reacts violently with water

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

WASTE CHARACTERISTIC HAZARD:   REACTIVE

FIRE EXTINGUISHER: USE APPROVED CLASS D EXTINGUISHERS OR SMOTHER WITH DRY

SAND, DRY    GROUND LIMESTONE OR DRY CLAY.    DO NOT USE WATER.

TOXIC EMISSIONS WHEN BURNED: HAZARDOUS COMBUSTION OR DECOMPOSITION PRODUCTS

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM BOROHYDRIDE    CAS # 13762511   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   I   J   K   .

NFPA HAZARD CODES (H,F,R,O)  3   0   1   W

ACUTE TOXICTY RISK INDEX 5 - LD50   167.0 mg/Kg

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50     46.0

ROUTE OF EXPOSUR skin Contact: Causes burns.

skin Absorption: Toxic if absorbed through skin.

Eye Contact: Causes burns.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be harmful

if inhaled.

Ingestion: Toxic if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Lungs. Nerves.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Reacts violently with water

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

WASTE CHARACTERISTIC HAZARD:   REACTIVE   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Acids, Alcohols, Strong bases.

FIRE EXTINGUISHER:  Use approved class D extinguishers or smother with

drysand, dry ground limestone, or dry clay. Do not use water.

TOXIC EMISSIONS WHEN BURNED: Liberates flammable/explosive hydrogen gas

Borane/boron oxides Potassium oxidesHazardous Decomposition Products Formed

Upon Contact with Water:   Hydrogen gas

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.   Unsuitable: Do not allow water to enter container.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: T

Indication of Danger: Toxic.

R: 24/25 14/15 34

Risk Statements: Toxic in contact with skin and if swallowed.

Reacts violently with water, liberating extremely flammable

gases. Causes burns.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM BROMATE    CAS # 7758012   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    A   .   .   .   .   .   .   .   .   .   K   .

NFPA HAZARD CODES (H,F,R,O)  2   0   1   OX

SPECIAL CARCINOGEN - DESIGNATED AREA MAY BE REQUIRED

CHRONIC TOXICITY RISK INDEX 3.7 - TD50     9.8 mg/Kg

ACUTE TOXICTY RISK INDEX 3.5 - LD50   157.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: Toxic if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Kidneys. Ears. Liver. Central nervous system.

SIGNS AND SYMPTOMS OF EXPOSURE

Absorption into the body leads to the formation of methemoglobin

which in sufficient concentration causes cyanosis. Onset may be

delayed 2 to 4 hours or longer.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

Oxidizing Agent

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong reducing agents, Finely powdered metals.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen bromide gas Oxygen

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from combustible    materials, heat, sparks, and open flame.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: O T

Indication of Danger: Oxidizing. Toxic.

R: 45 9 25

Risk Statements: May cause cancer. Explosive when mixed with

combustible material. Also toxic if swallowed.

S: 53 45

Safety Statements: Restricted to professional users. Attention -

Avoid exposure - obtain special instructions before use. In case

of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .1 mg/m3

DOE Short Term Exposure Limit  .3 mg/m3

DOE Ceiling Limit 60 mg/m3

Immediately Dangerous to Life and Health   60 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM BROMIDE    CAS # 7558023

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   I   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Central nervous system. Eyes.

SIGNS AND SYMPTOMS OF EXPOSURE

Can cause CNS depression. Potassium bromide on prolonged contact

with moist skin can produce severe irritation or burns   Prolonged

inhalation of dust can produce bronchitis. Ingestion

of large quantities can cause irritability, confusion, tremors,

acne-like skin eruptions, memory loss, headache, slurred speech,

and anorexia. Prolonged exposure can cause: Blurred vision.

Bromide rashes, especially of the face, and resembling acne and

furunculosis, often occur when bromide inhalation or

administration is prolonged. To the best of our knowledge, the

chemical, physical, and toxicological properties have not been

thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

VAPOR PRESSURE .010000 mm Hg @ 20 °C

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids, Heavy metal salts,

Aluminum, Potassium.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen bromide gas

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM BROMIDE    CAS # 7758023

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   I   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 2.2 - LD50  3070.0 mg/Kg

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Central nervous system. Eyes.

SIGNS AND SYMPTOMS OF EXPOSURE

Can cause CNS depression. Potassium bromide on prolonged contact

with moist skin can produce severe irritation or burns   Prolonged

inhalation of dust can produce bronchitis. Ingestion

of large quantities can cause irritability, confusion, tremors,

acne-like skin eruptions, memory loss, headache, slurred speech,

and anorexia. Prolonged exposure can cause: Blurred vision.

Bromide rashes, especially of the face, and resembling acne and

furunculosis, often occur when bromide inhalation or

administration is prolonged. To the best of our knowledge, the

chemical, physical, and toxicological properties have not been

thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

VAPOR PRESSURE .010000 mm Hg @ 20 °C

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids, Heavy metal salts,

Aluminum, Potassium.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen bromide gas

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  12.5 mg/m3

DOE Short Term Exposure Limit  40 mg/m3

DOE Ceiling Limit 250 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM BUTOXIDE (TERT-)    CAS # 865474   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   3   1   W

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes severe burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes severe burns.

Inhalation: May be harmful if inhaled. Material is extremely

destructive to the tissue of the mucous membranes and upper

respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Material is extremely destructive to tissue of the

mucous membranes and upper respiratory tract, eyes, and skin. To

the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Flammable

VAPOR PRESSURE .0900 mm Hg @ 20 °C

Reacts violently with water

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   REACTIVE   TOXIC

INCOMPATIBILITIES:Acids, Water, Reducing agents, Oxygen, Alcohols,

Chlorinated solvents, Halogens, Ketones.

FIRE EXTINGUISHER:  Dry chemical powder. Do not use water. Do not use carbon

dioxideextinguisher on this material. FIREFIGHTIN   Protective Equipment:

Wear self-contained breathing apparatus and protective clothing to prevent

contact with

TOXIC EMISSIONS WHEN BURNED: Potassium oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from combustible   materials, heat, sparks, and open flame. Store in a cool

dry   place\. Incompatible Materials: Do not allow contact with water

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: F C

Indication of Danger: Highly Flammable. Corrosive.

R: 11 14 35

Risk Statements: Highly flammable. Reacts violently with water.

Causes severe burns.

S: 7/9 16 26 36/37/39 43 45

Safety Statements: Keep container tightly closed and in

well-ventilated place. Keep away from sources of ignition - no

smoking. In case of contact with eyes, rinse immediately with

plenty of water and seek medical advice. Wear suitable

protective clothing, gloves, and eye/face protection. In case of

fire, use special powder for metal fire. Never use water. In

case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  1.5 mg/m3

DOE Short Term Exposure Limit  5 mg/m3

DOE Ceiling Limit 35 mg/m3

Immediately Dangerous to Life and Health   150 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM CARBONATE    CAS # 584087

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 2.4 - LD50  1870.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents Avoid contact with  acid.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22-36/37/38

Risk Statements: Harmful if swallowed. Irritating to eyes,

respiratory system and skin.

S: 26-36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  10 mg/m3

DOE Short Term Exposure Limit  20 mg/m3

DOE Ceiling Limit 150 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM CHLORATE    CAS # 3811049

DEPARTMENT OF HOMELAND SECURITY CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   2   OX

ACUTE TOXICTY RISK INDEX 1.8 - LD50  7000.0 mg/Kg

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Harmful if inhaled.

Ingestion: Harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Blood. Liver. Kidneys.

SIGNS AND SYMPTOMS OF EXPOSURE

Anemia. Absorption into the body leads to the formation of

methemoglobin which in sufficient concentration causes cyanosis.

Onset may be delayed 2 to 4 hours or longer. Symptoms of

overexposure to potassium chlorate include nausea, vomiting,

abdominal pain, diarrhea, anemia (hemolytic) and hemorrhage.

Methemoglobin formation results from catalytic oxidation of the

blood. Anuria, damage to the liver, convulsions due to central

nervous system effects, and coma may also occur. To the best of

our knowledge, the chemical, physical, and toxicological

properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

Explosive

Oxidizing Agent

SEGREGATION: SHELF # 3

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

k - Explosive/Unstable

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   REACTIVE

INCOMPATIBILITIES:Strong reducing agents, Finely powdered metals, Strong

acids, Alcohols, Organic materials.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriatfoam.

TOXIC EMISSIONS WHEN BURNED: Oxygen

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from combustible   materials, heat, sparks, and open flame.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATIO   Symbol of Danger: O Xn N

Indication of Danger: Oxidizing. Harmful. Dangerous for the

environment.

R: 9 20/22 51/53

Risk Statements: Explosive when mixed with combustible material.

Harmful by inhalation and if swallowed. Toxic to aquatic

organisms, may cause long-term adverse effects in the aquatic

environment.

S: 13 16 27 61

Safety Statements: Keep away from food, drink, and animal

feedingstuffs. Keep away from sources of ignition - no smoking.

Take off immediately all contaminated clothing. Avoid release to

the environment. Refer to special instructions/safety data

sheets.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  12.5 mg/m3

DOE Short Term Exposure Limit  40 mg/m3

DOE Ceiling Limit 300 mg/m3

Immediately Dangerous to Life and Health   350 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM CHLORIDE    CAS # 7447407

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

ACUTE TOXICTY RISK INDEX 2.3 - LD50  2600.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Hyperkalaemia, nausea, vomiting, abdominal pain, diarrhea or

constipation, paraesthesia, thirst, dizziness, skin rash,

pruritus, weakness, muscle cramps, and minor psychiatric or

visual changes. ORL-mus LD50: 56 mg/kg * ORL-rat LD50: 36 to 85

mg/kg (depending on strain)

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong acids, Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen chloride gas Potassium  oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  1.5 mg/m3

DOE Short Term Exposure Limit  5 mg/m3

DOE Ceiling Limit 15 mg/m3

Immediately Dangerous to Life and Health   15 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM CHROMATE    CAS # 7789006   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   H   .   J   K   L

NFPA HAZARD CODES (H,F,R,O)  3   0   1   OX

ACUTE TOXICTY RISK INDEX 2.9 - LD50   580.6 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material is irritating to

mucous membranes and upper respiratory tract.

Ingestion: Toxic if swallowed.

SENSITIZATION

skin: May cause allergic skin reaction.

TARGET ORGAN(S) OR SYSTEM(S)

Lungs. Kidneys. Liver. Blood. Eyes. Skin.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Oxidizing Agent

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Organic materials, Finely powdered metals, Strong oxidizing

agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

FIREFIGHTIN   Protective Equipment: Wear self-contained breathing apparatus

and protective clothing to prevent contact with skin and eyes.   Specific

Hazard(s): Emits toxic fu

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from combustible   materials, heat, sparks, and open flame.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T N

Indication of Danger: Toxic. Dangerous for the environment.

R: 49 46 36/37/38 43 50/53

Risk Statements: May cause cancer by inhalation. May cause

heritable genetic damage. Irritating to eyes, respiratory system

and skin. May cause sensitization by skin contact. Very toxic to

aquatic organisms, may cause long-term adverse effects in the

aquatic environment.

S: 53 45 60 61

Safety Statements: Restricted to professional users. Attention -

Avoid exposure - obtain special instructions before use. In case

of accident or if you feel unwell, seek medical advice

immediately (show the label where possible). This material and

its container must be disposed of as hazardous waste. Avoid

release to the environment. Refer to special instructions/safety

data sheets.

Immediately Dangerous to Life and Health  15 mg/m3

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .0187 mg/m3

DOE Short Term Exposure Limit  .75 mg/m3

DOE Ceiling Limit 6 mg/m3

Immediately Dangerous to Life and Health   56 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM CYANATE    CAS # 590283

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 2.3 - LD50  2712.9 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. May cause irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER:  Noncombustible. Use extinguishing media appropriate

tosurrounding fire conditions.

REACTIVE PROPERTIES

HANDLING: Avoid breathing dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22

Risk Statements: Harmful if swallowed.

S: 24/25

Safety Statements: Avoid contact with skin and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM CYANIDE    CAS # 151508

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   C   D   E   .   .   .   I   J   K   .

NFPA HAZARD CODES (H,F,R,O)  3   0   0

EXTREMELY TOXIC - DESIGNATED AREA MAY BE REQUIRED

ACUTE TOXICTY RISK INDEX 5 - LD50     5.0 mg/Kg

NEUROTOXIC - RISK INDEX   5.0

INHALATION HAZARD  INHALATION RISK INDEX   <1  - LC50

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:   TOXIC

FIRE EXTINGUISHER: appropriate foam.    DO NOT USE CARBON DIOXIDE

EXTINGUISHER ON THIS MATERIAL.

TOXIC EMISSIONS WHEN BURNED: M AIR  HAZARDOUS COMBUSTION OR DECOMPOSITIO

REACTIVE PROPERTIES

VIOLENT EXPLOSIONS OCCUR IF MELTED WITH NITRATE, NITRITE OR CHLORATE SALTS.

CYANIDE SALTS OF CADMIUM, COBALT, COPPER, LEAD, NICKEL AND ZINC REACTS

WITHINCANDESCENCE WITH MAGNESIUM. REACTS EXPLOSIVELY WITH NITRIC ACID.

Immediately Dangerous to Life and Health  25 mg/m3

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  5 mg/m3

DOE Short Term Exposure Limit  5 mg/m3

DOE Ceiling Limit 5 mg/m3

Immediately Dangerous to Life and Health   62.6 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM D-GLUCONATE    CAS # 299274

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 1.7 - LD50 10380.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause: Ingestion of large amounts may cause nausea,

vomiting, and diarrhea. I082 inhibits beta receptors in cardiac,

bronchial, and vascular musculature

CONDITIONS AGGRAVATED BY EXPOSURE

Alkalosis, an abnormal condition of increased alkalinity of the

blood and tissues.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM DICHROMATE    CAS # 7778509   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   E   .   G   H   .   J   K   L

NFPA HAZARD CODES (H,F,R,O)  4   1   1   OX

ACUTE TOXICTY RISK INDEX 3.2 - LD50   327.6 mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be fatal if absorbed through skin.

Eye Contact: Causes burns.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be fatal

if inhaled.

Ingestion: May be fatal if swallowed.

SENSITIZATION

Respiratory: May cause allergic respiratory reaction.

skin: May cause allergic skin reaction.

TARGET ORGAN(S) OR SYSTEM(S)

Blood. Kidneys. Lungs.

SIGNS AND SYMPTOMS OF EXPOSURE

Inhalation of dichromate dusts can cause ulceration and

perforation of the nasal septum. Contact with breaks in the skin

can cause ulceration (chrome sores). Other symptoms of exposure

include erosion and discoloration of the teeth, nephritis,

epigastric pain (inflammation and ulceration of the

gastrointestinal tract). Material is extremely destructive to

tissue of the mucous membranes and upper respiratory tract,

eyes, and skin. Inhalation may result in spasm, inflammation and

edema of the larynxand bronchi, chemical pneumonitis, and

pulmonary edema. Coma. Cyanosis. Vomiting. Ingestion can cause:

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

Oxidizing Agent

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Hydrazine, Finely powdered metals Avoid contact with acid.,

Organic materials

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriatfoam.

TOXIC EMISSIONS WHEN BURNED: Chromium (VI) oxide Potassium oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from combustible   materials, heat, sparks, and open flame. Store in a cool

dry   place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: O-T+-N

Indication of Danger: Oxidizing. Very toxic. Dangerous for the

environment.

R: 45-46-60-61-8-21-25-26-34-42/43-48/23-50/53

Risk Statements: May cause cancer. May cause heritable genetic

damage. May impair fertility. May cause harm to the unbor   child. Contact

with combustible material may cause fire. Also

harmful in contact with skin. Also toxic if swallowed. Also very

toxic by inhalation. Causes burns. May cause sensitization by

inhalation and skin contact. Also toxic: danger of serious

damage to health by prolonged exposure through inhalation. Very

toxic to aquatic organisms, may cause long-term adverse effects

in the aquatic environment.

S: 53-45-60-61

Safety Statements: Avoid exposure - obtain special instructions

before use. In case of accident or if you feel unwell, seek

medical advice immediately (show the label where possible). This

material and its container must be disposed of as hazardous

waste. Avoid release to the environment. Refer to special

instructions/safety data sheets.

Immediately Dangerous to Life and Health  15 mg/m3

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .0141 mg/m3

DOE Short Term Exposure Limit  1.5 mg/m3

DOE Ceiling Limit 10 mg/m3

Immediately Dangerous to Life and Health   42.4 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM DICYANOAURATE(I)    CAS # 13967505

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   E   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   0   0

INHALATION HAZARD  INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be fatal if absorbed through skin.

Eye Contact: Causes eye irritation.

Inhalation: May be fatal if inhaled. Material is irritating to

mucous membranes and upper respiratory tract.

Ingestion: May be fatal if swallowed.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong acids, Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen cyanide Nitrogen oxides Potassium

oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: T

Indication of Danger: Toxic.

R: 23/24/25 36/37/38

Risk Statements: Toxic by inhalation, in contact with skin and

if swallowed. Irritating to eyes, respiratory system and skin.

S: 45 26 36/37/39 22

Safety Statements: In case of accident or if you feel unwell,

seek medical advice immediately (show the label where possible).

In case of contact with eyes, rinse immediately with plenty of

water and seek medical advice. Wear suitable protective

clothing, gloves, and eye/face protection. Do not breathe dust.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM DIHYDROGEN CITRATE    CAS # 866831

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Potassium oxides

REACTIVE PROPERTIES

HANDLIN   User Exposure: Avoid inhalation. Avoid contact with eyes, skin, and

clothing. Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

Store in a cool dry place.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM DISULFITE    CAS # 16731558

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin. High concentrations are extremely destructive to

tissues of the mucous membranes and upper respiratory tract,

eyes, and skin.

SENSITIZATION

Sensitization: May cause allergic respiratory and skin reactions

SIGNS AND SYMPTOMS OF EXPOSURE

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Exposure can cause: Coughing, chest pains, difficulty

in breathing. Stomach pains, vomiting, diarrhea.

CONDITIONS AGGRAVATED BY EXPOSURE

Warning: certain individuals with preexisting respiratory

conditions such as asthma may experience hypersensitiveity to

sulfites and sulfur dioxide. The symptoms include

bronchoconstriction, bronchospasm, gastrointestinal

disturbances, flushing, hypotension, tingling sensation

urticaria/angioedema, and shock

IARC CARCINOGEN LIST

Rating: Group 3

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Dry chemical powder.

TOXIC EMISSIONS WHEN BURNED: Sulfur oxides

REACTIVE PROPERTIES

User Exposure: Do not breathe dust. Do not get in eyes, on skin, on

clothing. Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

Store in a cool dry place\. Incompatible Materials: Avoid contact with acid.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 31 36/37/38

Risk Statements: Contact with acids liberates toxic gas.

Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  20 mg/m3

DOE Short Term Exposure Limit  60 mg/m3

DOE Ceiling Limit 400 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM ETHOXIDE    CAS # 917588   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   3   1   W

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: May be harmful if inhaled. Material is extremely

destructive to the tissue of the mucous membranes and upper

respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Inhalation may result in spasm, inflammation and edema

of the larynxand bronchi, chemical pneumonitis, and pulmonary

edema. Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Flammable

Reacts violently with water

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   REACTIVE   TOXIC

INCOMPATIBILITIES:Acids, Acid chlorides, Material generates ethanol on

contact with water or moisture, Generates caustic   potassium solution upon

hydrolysis, Acid anhydrides, Oxidizing   agents, Reducing agents.

FIRE EXTINGUISHER:  Use approved class D extinguishers or smother with

drysand, dry ground limestone, or dry clay. Do not use water, foam, or carbon

dioxide.

TOXIC EMISSIONS WHEN BURNED: Potassium oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid prolonged or repeated   exposure. Do not

get in eyes, on skin, on clothing. STORAGE: Keep tightly closed. Keep away

from heat, sparks, and open flame\. Incompatible Materials: Do not allow

contact with water  Special REQUIREMENTS Handle and store under inert gas.

Reacts violently with water.   Air sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: F-C

Indication of Danger: Highly Flammable. Corrosive.

R: 11-14-34

Risk Statements: Highly flammable. Reacts violently with water.

Causes burns.

S: 8-16-26-43-45

Safety Statements: Keep container dry. Keep away from sources of

ignition - no smoking. In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. In

case of fire, use dry powder. Never use water. In case of

accident or if you feel unwell, seek medical advice immediately

(show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM ETHYL XANTHOGENATE    CAS # 140896

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   1   0

ACUTE TOXICTY RISK INDEX 2.5 - LD50  1700.0 mg/Kg

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material is irritating to   mucous

membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Nerves. Liver.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause: Nausea, headache, and vomiting. To the best

of our knowledge, the chemical, physical, and toxicological

properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids, Strong bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22 36/37/38

Risk Statements: Harmful if swallowed. Irritating to eyes,

respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM FERRATE(VI)    CAS # 39469868

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   1   OX

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Oxidizing Agent

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Organic materials, Finely powdered metals.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Potassium oxides Iron oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from combustible    materials, heat, sparks, and open flame. Store under

nitrogen\. SPECIAL REQUIREMENTS  Store under inert gas. Moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: O

Indication of Danger: Oxidizing.

R: 8

Risk Statements: Contact with combustible material may cause

fire.

S: 17 36

Safety Statements: Keep away from combustible material. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM FERRICYANIDE    CAS # 13746662

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 1.7 - LD50  9580.6 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, an   toxicological

properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong acids, Strong oxidizing agents, Ammonia,

Hydrochloric acid, Avoid contact with sodium nitrite and   chromium trioxide,

Heating to decomposition or contact with acids   or acid vapors can liberate

poisonous cyanide vapors.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen cyanide

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

R: 32

Risk Statements: Contact with acids liberates very toxic gas.

Immediately Dangerous to Life and Health  25 mg/m3

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  5.9 mg/m3

DOE Short Term Exposure Limit  17.7 mg/m3

DOE Ceiling Limit 30 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM FERROCYANIDE    CAS # 14459951

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Blood.

SIGNS AND SYMPTOMS OF EXPOSURE

May cause cyanosis (blue-gray coloring of skin and lips caused

by lack of oxygen).

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Avoid contact with acid., Strong oxidizing agents, Strong

acids

FIRE EXTINGUISHER:  Noncombustible. Use extinguishing media appropriate

tosurrounding fire conditions.

TOXIC EMISSIONS WHEN BURNED: Hydrogen cyanide Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

R: 32

Risk Statements: Contact with acids liberates very toxic gas.

S: 22-24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

Immediately Dangerous to Life and Health  25 mg/m3

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  13.5 mg/m3

DOE Short Term Exposure Limit  40.6 mg/m3

DOE Ceiling Limit 67.6 mg/m3

Immediately Dangerous to Life and Health   67.6 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM FLUORIDE    CAS # 13455215

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   E   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  3   0   0

ACUTE TOXICTY RISK INDEX 3.3 - LD50   245.0 mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   <1  - LC50

skin Contact: Causes burns.

skin Absorption: Toxic if absorbed through skin.

Eye Contact: Causes burns.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. Toxic if

inhaled.

Ingestion: Toxic if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Kidneys. Teeth. G.I. System. Nerves. Bones. Heart.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

Symptoms of fluoride overexposure may include salivation,

nausea, vomiting, abdominal pain, fever, and labored breathing.

Fluoride ion can reduce serum calcium levels possibly causing

fatal hypocalcemia. Prolonged exposure to fluoride dusts,

vapors, or mists results in perforation of the nasal septum.

Chronic effects include excessive calcification of the bones,

ligaments, and tendons. Material is extremely destructive to

tissue of the mucous membranes and upper respiratory tract,

eyes, and skin. Inhalation may result in spasm, inflammation and

edema of the larynxand bronchi, chemical pneumonitis, and

pulmonary edema. Symptoms of exposure may include burning

sensation, coughing, wheezing, laryngitis, shortness of breath,

headache, nausea, and vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen fluoride

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Stored

under argon. Store in a   cool dry place\. SPECIAL REQUIREMENTS  Hygroscopic.

Store under inert gas.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T

Indication of Danger: Toxic.

R: 23/24/25

Risk Statements: Toxic by inhalation, in contact with skin and

if swallowed.

S: 26-45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. In

case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  12.4 mg/m3

DOE Short Term Exposure Limit  37.2 mg/m3

DOE Ceiling Limit 61.9 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM FLUORIDE    CAS # 7789233

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   E   .   G   .   I   J   K   .

NFPA HAZARD CODES (H,F,R,O)  3   0   0

ACUTE TOXICTY RISK INDEX 3.3 - LD50   245.0 mg/Kg

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION HAZARD  INHALATION RISK INDEX   2.3 - LC50

skin Contact: Causes burns.

skin Absorption: Toxic if absorbed through skin.

Eye Contact: Causes burns.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. Toxic if

inhaled.

Ingestion: Toxic if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Kidneys. Teeth. G.I. System. Nerves. Bones. Heart.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

Symptoms of fluoride overexposure may include salivation,

nausea, vomiting, abdominal pain, fever, and labored breathing.

Fluoride ion can reduce serum calcium levels possibly causing

fatal hypocalcemia. Prolonged exposure to fluoride dusts,

vapors, or mists results in perforation of the nasal septum.

Chronic effects include excessive calcification of the bones,

ligaments, and tendons. Material is extremely destructive to

tissue of the mucous membranes and upper respiratory tract,

eyes, and skin. Inhalation may result in spasm, inflammation and

edema of the larynxand bronchi, chemical pneumonitis, and

pulmonary edema. Symptoms of exposure may include burning

sensation, coughing, wheezing, laryngitis, shortness of breath,

headache, nausea, and vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

VAPOR PRESSURE***** mm Hg @ 20 °C

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen fluoride

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Stored

under argon. Store in a   cool dry place\. SPECIAL REQUIREMENTS  Hygroscopic.

Store under inert gas.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T

Indication of Danger: Toxic.

R: 23/24/25

Risk Statements: Toxic by inhalation, in contact with skin and

if swallowed.

S: 26-45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. In

case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

Immediately Dangerous to Life and Health  250 mg/m

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  7.65 mg/m3

DOE Short Term Exposure Limit  22.9 mg/m3

DOE Ceiling Limit 38.2 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM HEXACHLOROPLATINATE(IV)    CAS # 16921305

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   G   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  3   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be harmful

if inhaled.

Ingestion: Toxic if swallowed.

SENSITIZATION

Sensitization: May cause allergic respiratory and skin reactions

SIGNS AND SYMPTOMS OF EXPOSURE

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Material is extremely destructive to tissue of the

mucous membranes and upper respiratory tract, eyes, and skin.

Exposure can cause: Dermatitis. Asthma.

IRRITATION DATA

skin

Rabbit

Remarks: No irritation effect

Eyes

Rabbit

Remarks: Severe irritation effect

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Acids.

FIRE EXTINGUISHER:  Dry chemical powder. Do not use water.

TOXIC EMISSIONS WHEN BURNED: Hydrogen chloride gas Chlorine

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T

Indication of Danger: Toxic.

R: 25 41 42/43

Risk Statements: Toxic if swallowed. Risk of serious damage to

eyes. May cause sensitization by inhalation and skin contact.

S: 22 26 36/37/39 45

Safety Statements: Do not breathe dust. In case of contact with

eyes, rinse immediately with plenty of water and seek medical

advice. Wear suitable protective clothing, gloves, and eye/face

protection. In case of accident or if you feel unwell, seek

medical advice immediately (show the label where possible).

Immediately Dangerous to Life and Health  4 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM HEXAFLUOROPHOSPHATE    CAS # 17084138

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be harmful

if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Material is extremely destructive to tissue of the

mucous membranes and upper respiratory tract, eyes, and skin. To

the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Dry chemical powder.

TOXIC EMISSIONS WHEN BURNED: Hydrogen fluoride Thermal decomposition may

produce toxic fumes of phosphorus oxides and/orphosphine

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34

Risk Statements: Causes burns.

S: 26 27 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Take

off immediately all contaminated clothing. Wear suitable

protective clothing, gloves, and eye/face protection. In case of

accident or if you feel unwell, seek medical advice immediately

(show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM HEXAMETHYLDISILAZIDE    CAS # 40949948   HAZARDOUS CHEMICAL OF

CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   0   1   W

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be harmful

if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Material is extremely destructive to tissue of the

mucous membranes and upper respiratory tract, eyes, and skin. To

the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Reacts violently with water

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

WASTE CHARACTERISTIC HAZARD:   REACTIVE   TOXIC

INCOMPATIBILITIES:Water, Strong oxidizing agents.

FIRE EXTINGUISHER:  Dry chemical powder.  Do not use water.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Silicon oxide Potassium oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place. Handle    and store under nitrogen\. Incompatible

Materials: Do not allow contact with water       SPECIAL REQUIREMENTS Handle

and store under inert gas. Reacts violently with water.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 14 34

Risk Statements: Reacts violently with water. Causes burns.

S: 26 36/37/39 43 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of fire, use dry powder. Never use water. In case of

accident or if you feel unwell, seek medical advice immediately

(show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM HYDRIDE    CAS # 7693267   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   2   W

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Inhalation may result in spasm, inflammation and edema

of the larynxand bronchi, chemical pneumonitis, and pulmonary

edema. Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  235 °F

Reacts violently in air

Reacts violently with water

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   REACTIVE

INCOMPATIBILITIES:Chlorinated solvents, Oxidizing agents,  Alcohols, Acids

Reacts violently with water.

FIRE EXTINGUISHER:  Use approved class D extinguishers or smother with dry

sand, dry ground limestone, or dry clay.  Do not use water.

TOXIC EMISSIONS WHEN BURNED: Liberates flammable/explosive  hydrogen gas

REACTIVE PROPERTIES

HANDLING: Avoid breathing dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat, sparks, and open flame. Store in a cool dry place. Handle and

store under    nitrogen\. Incompatible Materials: Do not allow contact with

water

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: F-C

Indication of Danger: Highly Flammable. Corrosive.

R: 11-14/15-34

Risk Statements: Highly flammable. Reacts violently with water,

liberating extremely flammable gases. Causes burns.

S: 16-26-27-36/37/39-45

Safety Statements: Keep away from sources of ignition - no

smoking. In case of contact with eyes, rinse immediately with

plenty of water and seek medical advice. Take off immediately

all contaminated clothing. Wear suitable protective clothing,

gloves, and eye/face protection. In case of accident or if you

feel unwell, seek medical advice immediately (show the label

where possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  1.5 mg/m3

DOE Short Term Exposure Limit  5 mg/m3

DOE Ceiling Limit 35 mg/m3

Immediately Dangerous to Life and Health   150 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM HYDROGEN PHOSPHATE    CAS # 7758114

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Causes a fall in blood pressure. Abdominal pain, nausea,

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Potassium oxides Thermal  decomposition may

produce toxic fumes of phosphorus oxides and/or phosphine

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36

Risk Statements: Irritating to eyes.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  10 mg/m3

DOE Short Term Exposure Limit  30 mg/m3

DOE Ceiling Limit 50 mg/m3

Immediately Dangerous to Life and Health   250 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM HYDROGEN SULFATE    CAS # 7646937

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   0   0

ACUTE TOXICTY RISK INDEX 2.3 - LD50  2340.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be harmful

if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema   Symptoms of

exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Material is extremely destructive to tissue of the

mucous membranes and upper respiratory tract, eyes, and skin. To

the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong bases, Strong oxidizing agents, Alcohols.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34 37

Risk Statements: Causes burns. Irritating to respiratory system.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  10 mg/m3

DOE Short Term Exposure Limit  30 mg/m3

DOE Ceiling Limit 200 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM HYDROGENFLUORIDE    CAS # 7789299

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract.

Ingestion: Toxic if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Kidneys.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Ingestion of large quantities of potassium ion may

cause lowered blood pressure, coma, and death. Symptoms of

fluoride overexposure may include salivation, nausea, vomiting,

abdominal pain, fever, and labored breathing. Fluoride ion can

reduce serum calcium levels possibly causing fatal hypocalcemia.

Prolonged exposure to fluoride dusts, vapors, or mists results

in perforation of the nasal septum. Chronic effects include

excessive calcification of the bones, ligaments, and tendons.

ACGIH CARCINOGEN LIST

Rating: A4

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Noncombustible. Use extinguishing media appropriate to

surrounding fire conditions.

TOXIC EMISSIONS WHEN BURNED: Hydrogen fluoride

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T-C

Indication of Danger: Toxic. Corrosive.

R: 25-34

Risk Statements: Toxic if swallowed. Causes burns.

S: 22-26-37-45

Safety Statements: Do not breathe dust. In case of contact with

eyes, rinse immediately with plenty of water and seek medical

advice. Wear suitable gloves. In case of accident or if you feel

unwell, seek medical advice immediately (show the label where

possible).

Immediately Dangerous to Life and Health  250 mg/m

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  5.14 mg/m3

DOE Short Term Exposure Limit  15.4 mg/m3

DOE Ceiling Limit 25.7 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM HYDROXIDE    CAS # 1310583   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   F   G   H   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  3   0   1   W

ACUTE TOXICTY RISK INDEX 3.2 - LD50   273.0 mg/Kg

INHALATION RISK INDEX   1.1 - LC50

ROUTE OF EXPOSURE

skin Contact: Causes severe burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes severe burns.

Inhalation: May be harmful if inhaled. Material is extremely

destructive to the tissue of the mucous membranes and upper

respiratory tract.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Material is extremely destructive to tissue of the

mucous membranes and upper respiratory tract, eyes, and skin. To

the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

VAPOR PRESSURE .0300 mm Hg @ 20 °C

Reacts violently with water

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

c - Inorganic Base/Oxidizer/Toxic

WASTE CHARACTERISTIC HAZARD:   REACTIVE   TOXIC   CORROSIVE

INCOMPATIBILITIES:Nitro compounds, Zinc, Organic materials, Magnesium,

Copper, Water, Aluminum.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Do not use water.

TOXIC EMISSIONS WHEN BURNED: Hydrogen gas  Potassium oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place\. Incompatible Materials: Absorbs carbon dioxide from air\.

SPECIAL REQUIREMENTS  Air sensitive. Very hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 22-35

Risk Statements: Harmful if swallowed. Causes severe burns   S:

26-36/37/39-45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  2 mg/m3

DOE Short Term Exposure Limit  2 mg/m3

DOE Ceiling Limit 2 mg/m3

Immediately Dangerous to Life and Health   150 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM IODATE    CAS # 7758056

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   1   OX

INHALATION RISK INDEX   <1  - LC50

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material is irritating to

mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Bone marrow. Blood. Thyroid.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause nausea, vomiting, diarrhea, and skin rash.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Oxidizing Agent

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Incompatibility: mixtures of iodates with finely divided

aluminum, arsenic, copper, carbon, phosphorous (red   or white) sulfur;

hydrides of alkali and alkaline earth metals;   sulfides of antimony,

arsenic, copper or tin, metal cyanides,   thiocyan

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen iodide

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from combustible   materials, heat, sparks, and open flame.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: O-Xi

Indication of Danger: Oxidizing. Irritant.

R: 8-36/37/38

Risk Statements: Contact with combustible material may cause

fire. Irritating to eyes, respiratory system and skin.

S: 17-26

Safety Statements: Keep away from combustible material. In case

of contact with eyes, rinse immediately with plenty of water and

seek medical advice.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  20 mg/m3

DOE Short Term Exposure Limit  60 mg/m3

DOE Ceiling Limit 60 mg/m3

Immediately Dangerous to Life and Health   60 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM IODIDE    CAS # 7681110   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   B   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  0   0   0

HUMAN TERATOGEN - DESIGNATED AREA MAY BE REQUIRED

REPRODUCTIVE RISK INDEX   3.3

ACUTE TOXICTY RISK INDEX 2.4 - LD50  1862.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SENSITIZATION

Sensitization: Prolonged or repeated exposure may cause allergic

reactions in certain sensitive individuals.

TARGET ORGAN(S) OR SYSTEM(S)

Thyroid.

SIGNS AND SYMPTOMS OF EXPOSURE

Prolonged exposure to iodides may produce iodism in sensitive

individuals. Symptoms of exposure include: skin rash, running

nose, headache and irritation of the mucous membrane. For severe

cases the skin may show pimples, boils, hives, blisters and

black and blue spots. Iodides are readily diffused across the

placenta. Neonatal deaths from respiratory distress secondary to

goiter have been reported. Iodides have been known to cause

drug-induced fevers, which are usually of short duration.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

VAPOR PRESSURE .0300 mm Hg @ 20 °C

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong reducing agents, Nickel, Strong acids,  and their

alloys, Steel, Aluminum, Alkali metals, Brass,    Magnesium, Zinc, Cadmium,

Copper.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen iodide Potassium  oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store

under argon. Store in a    cool dry place\. SPECIAL REQUIREMENTS  Air, light,

and moisture sensitive. Store under inert gas.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .25 mg/m3

DOE Short Term Exposure Limit  .75 mg/m3

DOE Ceiling Limit 6 mg/m3

Immediately Dangerous to Life and Health   300 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM METAPERIODATE    CAS # 7790218

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   1   OX

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin. High concentrations are extremely destructive to

tissues of the mucous membranes and upper respiratory tract,

eyes, and skin.

SIGNS AND SYMPTOMS OF EXPOSURE

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Oxidizing Agent

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong reducing agents, Finely powdered metals, Amines.

FIRE EXTINGUISHER:  Water spray.

REACTIVE PROPERTIES

HANDLING: Avoid breathing dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure.STORAGE: Keep tightly closed. Keep away

from combustible   materials, heat, sparks, and open flame. Store in a cool

dry   place\. SPECIAL REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: O Xi

Indication of Danger: Oxidizing. Irritant.

R: 8 36/37/38

Risk Statements: Contact with combustible material may cause

fire. Irritating to eyes, respiratory system and skin.

S: 17 26 36

Safety Statements: Keep away from combustible material. In case

of contact with eyes, rinse immediately with plenty of water and

seek medical advice. Wear suitable protective clothing.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  1 mg/m3

DOE Short Term Exposure Limit  3 mg/m3

DOE Ceiling Limit 20 mg/m3

Immediately Dangerous to Life and Health   100 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM NADH (BETA-)    CAS # 104809327

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   1

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Nerves. Liver. Eyes. Kidneys. Heart.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

2-8░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM NITRATE    CAS # 7757791

DEPARTMENT OF HOMELAND SECURITY CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   1   OX

ACUTE TOXICTY RISK INDEX 2.1 - LD50  3750.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Central nervous system. Blood.

SIGNS AND SYMPTOMS OF EXPOSURE

Absorption into the body leads to the formation of methemoglobin

which in sufficient concentration causes cyanosis. Onset may be

delayed 2 to 4 hours or longer.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Oxidizing Agent

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong reducing agents, Finely powdered  metals, Strong

acids, Organic materials.

FIRE EXTINGUISHER:  Water spray.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Potassium  oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from combustible    materials, heat, sparks, and open flame\. SPECIAL

REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: O

Indication of Danger: Oxidizing.

R: 8

Risk Statements: Contact with combustible material may cause

fire.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  1 mg/m3

DOE Short Term Exposure Limit  3.5 mg/m3

DOE Ceiling Limit 20 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM NITRITE    CAS # 7758090   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   C   .   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   0   1   OX

EXTREMELY TOXIC - DESIGNATED AREA MAY BE REQUIRED

ACUTE TOXICTY RISK INDEX 4.2 - LD50    31.2 mg/Kg

INHALATION RISK INDEX   <1  - LC50  26350.0

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: Toxic if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Produces methemoglobin, nausea, vomiting, rapid pulse, headache,

and visual disturbances. To the best of our knowledge, the

chemical, physical, and toxicological properties have not been

thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Oxidizing Agent

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong reducing agents, Finely powdered  metals, Strong

acids.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not get in eyes, on skin, on clothing. STORAGE: Keep tightly

closed. Keep away from combustible    materials, heat, sparks, and open

flame.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: O T N

Indication of Danger: Oxidizing. Toxic. Dangerous for the

environment.

R: 8 25 50

Risk Statements: Contact with combustible material may cause

fire. Toxic if swallowed. Very toxic to aquatic organisms.

S: 45 61

Safety Statements: In case of accident or if you feel unwell,

seek medical advice immediately (show the label where possible).

Avoid release to the environment. Refer to special

instructions/safety data sheets.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .04 mg/m3

DOE Short Term Exposure Limit  .1 mg/m3

DOE Ceiling Limit .75 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM NITROSODISULFONATE     CAS # 14293700   HAZARDOUS CHEMICAL OF

CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   3   1   W

ACUTE TOXICTY RISK INDEX 3.7 - LD50   100.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

FLASH POINT  77 °F

Forms ignitable mixtures in air at room temperature - Danger of remote

ignition and flashback

Reacts violently with water

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   REACTIVE   TOXIC

INCOMPATIBILITIES:Materials to Avoid: A

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxidesROUTE OF EXPOSURE

Store at 2-8°C

Do not allow contact with water Avoid contact with acid.

REACTIVE PROPERTIES

User Exposure: Avoid inhalation. Avoid contact with eyes, skin, and

clothing. Avoid prolonged or repeated exposure. STORAGE    Incompatible

Materials: Do not allow contact with water Avoid    contact with acid. Store

at 2-8░C SPECIAL REQUIREMENTS

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 14-20/21/22

Risk Statements: Reacts violently with water. Harmful by

S: 36

Safety Statements: Wear suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM PERCHLORATE    CAS # 7778747   HAZARDOUS CHEMICAL OF CONCERN

DEPARTMENT OF HOMELAND SECURITY CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   1   OX

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation   Inhalation: May be harmful if

inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Blood. Thyroid.

SIGNS AND SYMPTOMS OF EXPOSURE

Potassium perchlorate has produced fatal aplastic anemia along

with other blood disorders which include agranulocytosis,

thrombocytopenia, and leucopenia. It reduces the ability of the

thryoid to uptake iodide and stimulates the release of inorganic

iodide that has been taken up previously. Absorption into the

body leads to the formation of methemoglobin which in sufficient

concentration causes cyanosis. Onset may be delayed 2 to 4 hours

or longer.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

Oxidizing Agent

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong reducing agents, Finely powdered metals, Strong

acids, Organic materials, Forms explosive mixtures   with:, Alcohols.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from combustible   materials, heat, sparks, and open flame\. SPECIAL

REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: O Xn

Indication of Danger: Oxidizing. Harmful.

R: 9 22

Risk Statements: Explosive when mixed with combustible material.

Harmful if swallowed.

S: 13 22 27

Safety Statements: Keep away from food, drink, and animal

feedingstuffs. Do not breathe dust. Take off immediately all

contaminated clothing.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  125 mg/m3

DOE Short Term Exposure Limit  400 mg/m3

DOE Ceiling Limit 500 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM PERMANGANATE    CAS # 7722647

DEPARTMENT OF HOMELAND SECURITY CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   1   OX

ACUTE TOXICTY RISK INDEX 2.6 - LD50  1090.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Men exposed to manganese dusts showed a decrease in fertility.

Chronic manganese poisoning primarily involves the central

nervous system. Early symptoms include languor, sleepiness and

weakness in the legs. A stolid mask-like appearance of the face,

emotional disturbances such as uncontrollable laughter and a

spastic gait with tendency to fall in walking are findings in

more advanced cases. High incidence of pneumonia has been found

in workers exposed to the dust or fume of some manganese

compounds. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Oxidizing Agent

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong reducing agents, Finely powdered metals, Peroxides,

Zinc, Copper.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not get in eyes, on skin, on clothing. Avoid   prolonged or

repeated exposure. STORAGE: Keep tightly closed. Keep away from combustible

materials, heat, sparks, and open flame.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: O Xn N

Indication of Danger: Oxidizing. Harmful. Dangerous for the

environment   R: 8 22 50/53

Risk Statements: Contact with combustible material may cause

fire. Harmful if swallowed. Very toxic to aquatic organisms, may

cause long-term adverse effects in the aquatic environment.

S: 60 61

Safety Statements: This material and its container must be

disposed of as hazardous waste. Avoid release to the

environment. Refer to special instructions/safety data sheets.

Immediately Dangerous to Life and Health  500 mg/m3

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .575 mg/m3

DOE Short Term Exposure Limit  8.63 mg/m3

DOE Ceiling Limit 14.4 mg/m3

Immediately Dangerous to Life and Health   125 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM PEROXODISULFATE    CAS # 7727211   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   J   K   L

NFPA HAZARD CODES (H,F,R,O)  3   0   1   OX

ACUTE TOXICTY RISK INDEX 2.8 - LD50   802.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material is irritating to

mucous membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

SENSITIZATION

Sensitization: May cause allergic respiratory and skin reactions

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Oxidizing Agent

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Organic materials, Strong reducing agents,  Finely powdered

metals, Strong bases, Alcohols.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Sulfur oxides Oxygen

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from combustible    materials, heat, sparks, and open flame\. SPECIAL

REQUIREMENTS  Moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: O Xn

Indication of Danger: Oxidizing. Harmful.

R: 8 22 36/37/38 42/43

Risk Statements: Contact with combustible material may cause

fire. Harmful if swallowed. Irritating to eyes, respiratory

system and skin. May cause sensitization by inhalation and skin

contact.

S: 22 24 26 37

Safety Statements: Do not breathe dust. Avoid contact with skin.

In case of contact with eyes, rinse immediately with plenty of

water and seek medical advice. Wear suitable gloves.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .1 mg/m3

DOE Short Term Exposure Limit  .3 mg/m3

DOE Ceiling Limit 60 mg/m3

Immediately Dangerous to Life and Health   350 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM PEROXOMONOSULFATE    CAS # 37222665

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  0   0   1   OX

ACUTE TOXICTY RISK INDEX 2.8 - LD50   802.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Oxidizing Agent

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

WASTE CHARACTERISTIC HAZARD:   TOXIC

FIRE EXTINGUISHER: WATER SPRAY.

TOXIC EMISSIONS WHEN BURNED: MBUSTION OR DECOMPOSITION PRODUCTS  TOXIC FUME

REACTIVE PROPERTIES

REACTS VIGOROUSLY WITH HYDRAZINE. DRY MATERIAL DECOMPOSES RAPIDLY ABOVE 100

CLIBERATING OXYGEN WHILE WET MATERIAL REQUIRES A TEMPERATURE OF ONLY 50 C

FORDECOMPOSITION. A VIGOROUS SELF-SUSTAINING FIRE RESULTED FROM THE CONTACT

OFTWO FLAKES OF MOIST POTASSIUM HYDROXIDE. THE FIRE WAS EXTINGUISHABLE

WITHWATER, BUT NOT WITH CARBON DIOXIDE OR DRY POWDER. EXPLOSIONS OCCUR WHEN

POTASSIUM SULFATE AND SODIUM SULFATE ARE MELTED WITHALUMINUM AND ALSO WHEN

BARIUM SULFATE AND CALCIUM SULFATE ARE REDUCED BYALUMINUM. EXPLOSIVE WHEN

MIXED WITH MAGNESIUM.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  10 mg/m3

DOE Short Term Exposure Limit  30 mg/m3

DOE Ceiling Limit 50 mg/m3

Immediately Dangerous to Life and Health   250 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM PEROXYMONOSULFATE    CAS # 70693628

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  3   0   1   OX

ACUTE TOXICTY RISK INDEX 3 - LD50  2000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50   5000.0

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: May be harmful if inhaled. Material is extremely

destructive to the tissue of the mucous membranes and upper

respiratory tract.

Ingestion: Harmful if swallowed.

SENSITIZATION

Sensitization: Prolonged or repeated exposure may cause allergic

reactions in certain sensitive individuals.

SIGNS AND SYMPTOMS OF EXPOSURE

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Material is extremely destructive to tissue of the

mucous membranes and upper respiratory tract, eyes, and skin. To

the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Oxidizing Agent

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong reducing agents, Finely powdered  metals, Strong

bases, Halogens, Halides, Cyanides.

TOXIC EMISSIONS WHEN BURNED: Potassium oxides Sulfuric acid  Sulfur oxides

Oxygen

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 22 34

Risk Statements: Harmful if swallowed. Causes burns.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM PHOSPHATE DIBASIC    CAS # 16788571

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Potassium oxides Phosphorous oxides

REACTIVE PROPERTIES

User Exposure: Avoid inhalation. Avoid contact with eyes, skin, and

clothing. Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  300 mg/m3

DOE Short Term Exposure Limit  500 mg/m3

DOE Ceiling Limit 500 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM PHOSPHATE DIBASIC    CAS # 16788751

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Potassium oxides Phosphorous oxides

REACTIVE PROPERTIES

User Exposure: Avoid inhalation. Avoid contact with eyes, skin, and

clothing. Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM PHOSPHATE MONOBASIC    CAS # 7778770

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Phosphorous oxides Potassium oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Hygroscopic.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  10 mg/m3

DOE Short Term Exposure Limit  30 mg/m3

DOE Ceiling Limit 50 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM PHOSPHATE TRIBASIC    CAS # 7778532

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 2 - LD50  4500.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes severe eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 38 41

Risk Statements: Irritating to skin. Risk of serious damage to

eyes.

S: 26 39

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

eye/face protection.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  10 mg/m3

DOE Short Term Exposure Limit  30 mg/m3

DOE Ceiling Limit 50 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM PHOSPHATE TRIBASIC MONOHYDRATE    CAS # 27176109

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 2 - LD50  4500.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes severe eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 41

Risk Statements: Risk of serious damage to eyes.

S: 26 39

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM POLYSULFIDE    CAS # 1312738

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Inhalation may result in spasm, inflammation and edema

of the larynxand bronchi, chemical pneumonitis, and pulmonary

edema. Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  328 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Potassium oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

Unsuitable: Heat\. Incompatible Materials: Avoid contact with acid. Nitrogen

oxides       SPECIAL REQUIREMENTS    Avoid contact with acid.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: C-N

Indication of Danger: Corrosive. Dangerous for the environment.

R: 31-34-50

Risk Statements: Contact with acids liberates toxic gas. Causes

burns. Very toxic to aquatic organisms.

S: 26-45-61

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. In

case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible). Avoid release to

the environment. Refer to special instructions/safety data

sheets.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM POLYSULFIDE    CAS # 37199669

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Inhalation may result in spasm, inflammation and edema

of the larynxand bronchi, chemical pneumonitis, and pulmonary

edema. Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  328 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Potassium oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

Unsuitable: Heat\. Incompatible Materials: Avoid contact with acid. Nitrogen

oxides       SPECIAL REQUIREMENTS    Avoid contact with acid.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: C-N

Indication of Danger: Corrosive. Dangerous for the environment.

R: 31-34-50

Risk Statements: Contact with acids liberates toxic gas. Causes

burns. Very toxic to aquatic organisms.

S: 26-45-61

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. In

case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible). Avoid release to

the environment. Refer to special instructions/safety data

sheets.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM PYROPHOSPHATE    CAS # 7320345

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  10 mg/m3

DOE Short Term Exposure Limit  30 mg/m3

DOE Ceiling Limit 500 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM SELENATE    CAS # 7790592

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   E   .   .   .   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION HAZARD  INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be fatal if inhaled, swallowed, or absorbed

through skin. Causes eye and skin irritation.

TARGET ORGAN(S) OR SYSTEM(S)

Liver. Kidneys.

SIGNS AND SYMPTOMS OF EXPOSURE

Chronic contact with selenium compounds may cause garlic odor of

breath and sweat, dermatitis, and moderate emotional

instability. To the best of our knowledge, the chemical,

physical, and toxicological properties have not been thoroughly

investigated. Acute selenium poisoning produces central nervous

system effects, which include nervousness, convulsions, and

drowsiness. Other signs of intoxication can include skin

eruptions, lassitude, gastrointestinal distress, teeth that are

discolored or decayed, odorous ("garlic") breath, and partial

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Noncombustible. Use extinguishing media appropriate

tosurrounding fire conditions.

TOXIC EMISSIONS WHEN BURNED: Selenium/selenium oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid all contact. STORAG   Suitable: Keep tightly closed. Store in a cool

dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T N

Indication of Danger: Toxic. Dangerous for the environment.

R: 23/25 33 50/53

Risk Statements: Toxic by inhalation and if swallowed. Danger of

cumulative effects. Very toxic to aquatic organisms, may cause

long-term adverse effects in the aquatic environment   S: 20/21 28 45 60

61

Safety Statements: When using do not eat, drink, or smoke. After

contact with skin, wash immediately with plenty of soap-suds. In

case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible). This material and

its container must be disposed of as hazardous waste. Avoid

release to the environment. Refer to special instructions/safety

data sheets.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .56 mg/m3

DOE Short Term Exposure Limit  1.68 mg/m3

DOE Ceiling Limit 2.8 mg/m3

Immediately Dangerous to Life and Health   2.8 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM SODIUM TARTRATE TETRAHYDRATE    CAS # 304596

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER: WATER SPRAY.    CARBON DIOXIDE, DRY CHEMICAL POWDER OR

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: IDE

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  20 mg/m3

DOE Short Term Exposure Limit  60 mg/m3

DOE Ceiling Limit 400 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM SODIUM TARTRATE TETRAHYDRATE    CAS # 6381595

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  20 mg/m3

DOE Short Term Exposure Limit  60 mg/m3

DOE Ceiling Limit 400 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM SULFATE    CAS # 7778805

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

ACUTE TOXICTY RISK INDEX 1.9 - LD50  6600.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause: Gastrointestinal disturbances. To the best

of our knowledge, the chemical, physical, and toxicological

properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  2 mg/m3

DOE Short Term Exposure Limit  6 mg/m3

DOE Ceiling Limit 40 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM SULFITE    CAS # 10117381

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   1

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong acids, Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Potassium oxides Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place. Store under nitrogen\. SPECIAL REQUIREMENTS  Air and

moisture sensitive. Store under inert gas.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  10 mg/m3

DOE Short Term Exposure Limit  30 mg/m3

DOE Ceiling Limit 50 mg/m3

Immediately Dangerous to Life and Health   250 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM TELLURITE    CAS # 7790581

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   I   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 3.9 - LD50    58.8 mg/Kg

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Nerves. Kidneys.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Materials to Avoid: S

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at 2-8°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

2-8░C

Immediately Dangerous to Life and Health  25 mg/m3

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .199 mg/m3

DOE Short Term Exposure Limit  .597 mg/m3

DOE Ceiling Limit 3.5 mg/m3

Immediately Dangerous to Life and Health   15 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM TERATHIONATE    CAS # 13932133

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material is irritating to

mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Sulfur oxides Potassium oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM TETRABORATE TETRAHYDRATE    CAS # 12045782

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   F   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 2.1 - LD50  3700.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Central nervous system. Kidneys. Testes.

SIGNS AND SYMPTOMS OF EXPOSURE

Toxicity reported for borates in humans: ingestion or absorption

may cause nausea, vomiting, diarrhea, abdominal cramps,

anderythematous lesions on the skin and mucous membranes. Other

symptoms include: circulatory collapse, tachycardia, cyanosis,

delirium, convulsions, and coma. Death has been reported to

occur in infants from less than 5 grams and in adults from 5 to

20 grams.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC   CORROSIVE

INCOMPATIBILITIES:Potassium, Acid anhydrides.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Boron oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM TETRACHLOROPALLADATE    CAS # 10025986

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   1

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Noncombustible. Use extinguishing media appropriate to

surrounding fire conditions.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM TETRACHLOROPLATINATE(II)    CAS # 10025997

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   C   .   E   .   G   .   .   J   K   L

NFPA HAZARD CODES (H,F,R,O)  3   0   1

EXTREMELY TOXIC - DESIGNATED AREA MAY BE REQUIRED

ACUTE TOXICTY RISK INDEX 4.6 - LD50    13.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes severe eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: Toxic if swallowed.

SENSITIZATION

Sensitization: May cause allergic reaction.

SIGNS AND SYMPTOMS OF EXPOSURE

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Exposure can cause: Dermatitis. Asthma.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Acids.

FIRE EXTINGUISHER:  Dry chemical powder. Do not use water.

TOXIC EMISSIONS WHEN BURNED: Hydrogen chloride gas Potassium oxides Platinum

oxide

REACTIVE PROPERTIES

HANDLING: Avoid breathing dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T

Indication of Danger: Toxic.

R: 25 38 41 42/43

Risk Statements: Toxic if swallowed. Irritating to skin. Risk of

serious damage to eyes. May cause sensitization by inhalation

and skin contact.

S: 22 26 36/37/39 45

Safety Statements: Do not breathe dust. In case of contact with

eyes, rinse immediately with plenty of water and seek medical

advice. Wear suitable protective clothing, gloves, and eye/face

protection. In case of accident or if you feel unwell, seek

medical advice immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM TETRAFLUOROBORATE    CAS # 14075537

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   F   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 1.9 - LD50  5854.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   CORROSIVE

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen fluoride Boron oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place. Handle   and store under nitrogen\. SPECIAL REQUIREMENTS

Moisture sensitive. Handle and store under inert gas.

Immediately Dangerous to Life and Health  250 mg/m

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  4.14 mg/m3

DOE Short Term Exposure Limit  124 mg/m3

DOE Ceiling Limit 207 mg/m3

Immediately Dangerous to Life and Health   414 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM TETRAOXALATE DIHYDRATE    CAS # 6100205

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: Harmful if absorbed through skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

The chemical, physical, and toxicological properties of this

product have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Potassium oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 21/22

Risk Statements: Harmful in contact with skin and if swallowed.

S: 24/25

Safety Statements: Avoid contact with skin and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM THIOCYANATE    CAS # 333200

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   I   J   K   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

ACUTE TOXICTY RISK INDEX 2.8 - LD50   854.0 mg/Kg

NEUROTOXIC - RISK INDEX   5.0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: Harmful if absorbed through skin.

Eye Contact: May cause eye irritation.

Inhalation: Harmful if inhaled. Material may be irritating to

mucous membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Central nervous system. Cardiovascular system. Thyroid.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause: Nausea, headache, and vomiting. To the best

of our knowledge, the chemical, physical, and toxicological

properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Acids, Strong bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxides Potassium Hydrogen

cyanide

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Light and moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 20/21/22-32-52/53

Risk Statements: Harmful by inhalation, in contact with skin and

if swallowed. Contact with acids liberates very toxic gas.

Harmful to aquatic organisms, may cause long-term adverse

effects in the aquatic environment.

S: 13-61

Safety Statements: Keep away from food, drink, and animal

feedingstuffs. Avoid release to the environment. Refer to

special instructions/safety data sheets.

Immediately Dangerous to Life and Health  25 mg/m3

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  10 mg/m3

DOE Short Term Exposure Limit  35 mg/m3

DOE Ceiling Limit 60 mg/m3

Immediately Dangerous to Life and Health   60 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSIUM TRIMETHYLSILANOLATE    CAS # 10519967

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   0   1

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be harmful

if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, an

vomiting. Material is extremely destructive to tissue of the

mucous membranes and upper respiratory tract, eyes, and skin. To

the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Silicon oxide

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Handle

and store under nitrogen\. SPECIAL REQUIREMENTS  Moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34

Risk Statements: Causes burns.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# POTASSSIUM FORMATE    CAS # 590294

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 1.4 - LD50 17741.9 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid contact and inhalation. Do not get in eyes, on skin, on

clothing. STORAGE: Keep tightly closed. Store in a cool dry place\. SPECIAL

REQUIREMENTS Very hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  20 mg/m3

DOE Short Term Exposure Limit  60 mg/m3

DOE Ceiling Limit 500 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PRASEODYMIUM    CAS # 7440100   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   2

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Reacts violently in air

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

WASTE CHARACTERISTIC HAZARD:   REACTIVE

INCOMPATIBILITIES:Halogens, Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Dry chemical powder.

REACTIVE PROPERTIES

HANDLING: Use protective clothing, gloves, and mask. Avoid   contact and

inhalation. STORAGE: Store under argon\. SPECIAL REQUIREMENTS  Air and

moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: F

Indication of Danger: Highly Flammable.

R: 17

Risk Statements: Spontaneously flammable in air.

S: 17

Safety Statements: Keep away from combustible material.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  1.5 mg/m3

DOE Short Term Exposure Limit  5 mg/m3

DOE Ceiling Limit 35 mg/m3

Immediately Dangerous to Life and Health   150 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PRASEODYMIUM NITRATE    CAS # 10361805

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   1   OX

ACUTE TOXICTY RISK INDEX 2.4 - LD50  1859.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin. High concentrations are extremely destructive to

tissues of the mucous membranes and upper respiratory tract,

eyes, and skin.

SIGNS AND SYMPTOMS OF EXPOSURE

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Oxidizing Agent

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong reducing agents, Finely powdered metals, Strong

acids.

FIRE EXTINGUISHER:  Water spray.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid breathing dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from combustible   materials, heat, sparks, and open flame. Store in a cool

dry   place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: O Xn

Indication of Danger: Oxidizing. Harmful.

R: 8 22 36/37/38

Risk Statements: Contact with combustible material may cause

fire. Harmful if swallowed. Irritating to eyes, respiratory

system and skin.

S: 17 26 27 36/37/39

Safety Statements: Keep away from combustible material. In case

of contact with eyes, rinse immediately with plenty of water and

seek medical advice. Take off immediately all contaminated

clothing. Wear suitable protective clothing, gloves, and

eye/face protection.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  7.5 mg/m3

DOE Short Term Exposure Limit  20 mg/m3

DOE Ceiling Limit 150 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PRASEODYMIUM(III) NITRATE HEXAHYDRATE    CAS # 15878770

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   1   OX

ACUTE TOXICTY RISK INDEX 2.2 - LD50  3500.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin. High concentrations are extremely destructive to

tissues of the mucous membranes and upper respiratory tract,

eyes, and skin.

SIGNS AND SYMPTOMS OF EXPOSURE

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Oxidizing Agent

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong reducing agents, Finely powdered metals, Strong

acids.

FIRE EXTINGUISHER:  Water spray.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

User Exposure: Avoid breathing dust. Do not get in eyes, on skin, on

clothing. Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

Keep away from combustible   materials, heat, sparks, and open flame. Store

in a cool dry   place\. SPECIAL REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: O Xi

Indication of Danger: Oxidizing. Irritant.

R: 8

Risk Statements: Contact with combustible material may cause

fire.

S: 17 26 27 36/37/39

Safety Statements: Keep away from combustible material. In case

of contact with eyes, rinse immediately with plenty of water and

seek medical advice. Take off immediately all contaminated

clothing. Wear suitable protective clothing, gloves, and

eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PREDNISONE    CAS # 53032   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   B   .   .   E   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   1   OX

HUMAN TERATOGEN - DESIGNATED AREA MAY BE REQUIRED

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

Oxidizing Agent

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents Avoid contact with

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxides Hydrogen chloride

gas

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure.STORAGE: Keep container closed. Store in

a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 63

S: 45 36/37/39

Safety Statements: In case of accident or if you feel unwell,

seek medical advice immediately (show the label where possible).

Wear suitable protective clothing, gloves, and eye/face

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PREGNENOLONE    CAS # 145131

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   0   0

INHALATION RISK INDEX   <1  - LC50

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

Some steroids show carcinogenic and teratogenic activity. To the

best of our knowledge, the chemical, physical, and toxicological

properties have not been thoroughly investigated.

The toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

User Exposure: Do not breathe vapor. Avoid contact with eyes, skin, and

clothing. Avoid prolonged or repeated exposure. Do not    use if skin is cut

or scratched. Wash thoroughly after handling.     Suitable: Keep tightly

closed. Keep away from heat, sparks, and open flame.           Store under

inert gas. Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PRIMULINE    CAS # 8064606

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: and nitrogen oxides Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22

Risk Statements: Harmful if swallowed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROCION BLUE MX-R    CAS # 13324204

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 1.7 - LD50  8980.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxides Hydrogen chloride

gas

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, andclothing. Avoid

prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in a cool

dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROCION RED MX-5B    CAS # 17804498

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 1.8 - LD50  7460.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen chloride gas Nitrogen oxides Sulfur

oxides

REACTIVE PROPERTIES

HANDLING skin, and clothing. Avoid prolonged or repeated exposure. STORAGE:

Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 36 42/43

Risk Statements: Irritating to eyes. May cause sensitization by

inhalation and skin contact.

S: 22 26 36/37 45

Safety Statements: Do not breathe dust. In case of contact with

eyes, rinse immediately with plenty of water and seek medical

advice. Wear suitable protective clothing and gloves. In case of

accident or if you feel unwell, seek medical advice immediately

(show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROCION YELLOW    CAS # 59112786

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROFLAVINE HEMISULFATE    CAS # 1811285

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Avoid contact and inhalation. Do not get in eyes, on skin, on

clothing. STORAGE: Keep tightly closed. Store in a cool dry place\. SPECIAL

REQUIREMENTS  Light sensitive. Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROGESTERONE    CAS # 57830   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   B   .   .   .   .   .   .   .   J   .   L

NFPA HAZARD CODES (H,F,R,O)  3   0   0

HUMAN TERATOGEN - DESIGNATED AREA MAY BE REQUIRED

ACUTE TOXICTY RISK INDEX 2 - LD50  5000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SENSITIZATION

Sensitization: Causes photosensitiveity. Exposure to light can

result in allergic reactions resulting in dermatologic lesions,

which can vary from sunburnlike responses to edematous,

vesiculated lesions, or bullae

TARGET ORGAN(S) OR SYSTEM(S)

Female reproductive system. Male reproductive system. Central

nervous system. Liver.

SIGNS AND SYMPTOMS OF EXPOSURE

Causes anabolic and androgenic effects. Exposure can cause liver

disturbances, jaundice, nausea, vomiting, chills, diarrhea,

abdominal fullness, excitation, insomnia, confusion, acne, blood

lipid changes, increased serum cholesterol, retention of sodium,

chloride, water, potassium, phosphates, calcium and nitrogen,

oedema, increased vascularity of the skin, and growth of the

bones. In women, exposure can result in the suppression of

ovarian activity and menstruation. Continued exposure can

produce hirsutism, hoarseness or deepening of the voice, and

atrophy of the breasts and endometrial tissue. In men, exposure

may suppress spermatogenesis. To the best of our knowledge, the

chemical, physical, and toxicological properties have not been

thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Light sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 40

Risk Statements: Limited evidence of a carcinogenic effect.

S: 36/37

Safety Statements: Wear suitable protective clothing and gloves.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROLIDASE    CAS # 9025325

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SENSITIZATION

Respiratory: May cause allergic respiratory reaction.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Materials to Avoid: S

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at -20°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

-20░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 36/37/38 42

Risk Statements: Irritating to eyes, respiratory system and

skin. May cause sensitization by inhalation.

S: 22 24 26 36/37

Safety Statements: Do not breathe dust. Avoid contact with skin.

In case of contact with eyes, rinse immediately with plenty of

water and seek medical advice. Wear suitable protective clothing

and gloves.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROLINAMIDE (L-)    CAS # 7531524

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

-

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides-

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.    -

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROLINE (BOC-)    CAS # 15761394

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROLINE (D-)    CAS # 344252

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROLINE (DL-)    CAS # 609369

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROLINE (FMOC,L-)    CAS # 71989316

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Materials to Avoid: S

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

Store at 2-8°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

2-8░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROLINE (L-)    CAS # 147853

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

d - Organic Acid/Flammable/Toxic

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

Immediately Dangerous to Life and Health  1000 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROLINE BENZYL ESTER HCL (L-)    CAS # 16652714

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, STORAGE: Keep

tightly closed. Keep away from combustible    materials, heat, sparks, and

open flame\. SPECIAL REQUIREMENTS  Store under inert gas. Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROLINE IMINOPEPTIDASE    CAS # 9025405

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. May cause irritation.

SENSITIZATION

Sensitization: Prolonged or repeated exposure may cause allergic

reactions in certain sensitive individuals.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at -20°C

REACTIVE PROPERTIES

STORAGE Store at -20░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROLINE METHYL ESTER HYDROCHLORIDE (L-)    CAS # 2133406

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place. Store under nitrogen\. SPECIAL REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/38

Risk Statements: Irritating to eyes and skin.

S: 26 37/39

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable gloves and eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PRONASE E    CAS # 9036060

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  1   0   0

ACUTE TOXICTY RISK INDEX 2.2 - LD50  3290.0 mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SENSITIZATION

Sensitization: Prolonged or repeated exposure may cause allergic

reactions in certain sensitive individuals.

TARGET ORGAN(S) OR SYSTEM(S)

Lungs.

SIGNS AND SYMPTOMS OF EXPOSURE

Repeated exposure can cause: Asthma.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at -20°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

-20░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPANE    CAS # 74986

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   .   .   .   .   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   4   0

NEUROTOXIC - RISK INDEX   2.0

INHALATION HAZARD  INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract. Can

cause rapid suffocation.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Inhalation of propane at concentrations sufficent to exclude an

adequate supply of oxygen to the lungs can result in dizziness,

drowsiness, and eventual unconsciousness. It has a narcotic

action and acts as a depressant on the central nervous system.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Gas

Flammable

VAPOR PRESSURE 6065.6 mm Hg @ 20 °C

FLASH POINT  -155.2 °F

Store in an explosion-proof refrigerator or tightly stoppered in a

well-ventilated area

SEGREGATION: SHELF # 3

STORAGE GROUP(S):

h - Flammable/Pyrophoric Gas

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Use water spray or fog nozzle to keep cylinder cool. Move

cylinder away from fire if there is no risk.

REACTIVE PROPERTIES

HANDLING: Do not breathe gas. Do not get in eyes, on skin, on clothing. Avoid

prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away from

heat, sparks, and open flame. Use with equipment rated for cylinder pressure,

and    of compatible materials of construction. Close valve when not in

use and when empty. Make sure cylinder is properly secured when    in use or

stored Cylinder temperature should not exceed 125░F    (52░C).    Unsuitable:

Store away from heat and direct sunlight       SPECIAL REQUIREMENTS

Contents under pressure.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: F+

Indication of Danger: Extremely Flammable.

R: 12

Risk Statements: Extremely flammable.

S: 9 16

Safety Statements: Keep container in a well-ventilated place.

Keep away from sources of ignition - no smoking.

Immediately Dangerous to Life and Health  2100 ppm

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  1000 ppm

DOE Short Term Exposure Limit  5500 ppm

DOE Ceiling Limit 17000 ppm

Immediately Dangerous to Life and Health   33000 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPANEDIOL (1,2-)    CAS # 57556

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

ACUTE TOXICTY RISK INDEX 1.4 - LD50 20000.0 mg/Kg

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause: Gastrointestinal disturbances. Nausea,

headache, and vomiting. CNS depression.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE .0800 mm Hg @ 20 °C

FLASH POINT  217.4 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Acid chlorides, Acid anhydrides, Oxidizing  agents,

Chloroformates, Reducing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Hygroscopic.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  10 mg/m3

DOE Short Term Exposure Limit  10 mg/m3

DOE Ceiling Limit 10 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPANEDIOL (1,3-)    CAS # 504632

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

ACUTE TOXICTY RISK INDEX 1.4 - LD50 20000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause: Gastrointestinal disturbances. Nausea,

headache, and vomiting. To the best of our knowledge, the

chemical, physical, and toxicological properties have not been

thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE .1000 mm Hg @ 20 °C

FLASH POINT  267.8 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Acid chlorides, Acid anhydrides, Oxidizing agents,

Chloroformates, Reducing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 23 24/25

Safety Statements: Do not breathe vapor. Avoid contact with skin

and eyes.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  20 mg/m3

DOE Short Term Exposure Limit  60 mg/m3

DOE Ceiling Limit 400 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPANEDIOL DI-P-TOSYLATE (1,3-)    CAS # 5469669

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPANEDISULFONIC ACID (1,3-)    CAS # 21668779

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin.

TARGET ORGAN(S) OR SYSTEM(S)

Central nervous system. Ears. Liver. Kidneys. Blood.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Can cause CNS depression. Exposure can cause:

Gastrointestinal disturbances.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPANEDITHIOL (1,3-)    CAS # 109808

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   2   0

ACUTE TOXICTY RISK INDEX 2.7 - LD50  1070.0 mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   2.1 - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Vapor or mist is irritating to the eyes, mucous

membranes, and upper respiratory tract.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause: Nausea, headache, and vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE    2.7 mm Hg @ 20 °C

FLASH POINT  141.8 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Bases, Oxidizing agents, Reducing agents,  Alkali metals.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Water may be effective for cooling, but may not    effect extinguishment.

TOXIC EMISSIONS WHEN BURNED: Sulfur oxides Hydrogen sulfide gas

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPANESULFONYL CHLORIDE (1-)    CAS # 10147361   HAZARDOUS CHEMICAL OF

CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   1   W

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  206.6 °F

Reacts violently with water

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   REACTIVE

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases,  Alcohols.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Sulfur oxides Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame. Store in a cool dry place\. SPECIAL REQUIREMENTS

Moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 14 34 36

Risk Statements: Reacts violently with water. Causes burns.

Irritating to eyes.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPANOL (1-)    CAS # 71238

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   .   .   G   H   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   3   0

ACUTE TOXICTY RISK INDEX 2.4 - LD50  1870.0 mg/Kg

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION HAZARD  INHALATION RISK INDEX   2.5 - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes severe eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SENSITIZATION

Sensitization: Will not occur

TARGET ORGAN(S) OR SYSTEM(S)

Nerves. Liver.

SIGNS AND SYMPTOMS OF EXPOSURE

Can cause CNS depression. Prolonged exposure can cause: Narcotic

effect. Drying, cracking, or irritation of the skin.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

VAPOR PRESSURE   14.9 mm Hg @ 20 °C

FLASH POINT  72 °F

Forms ignitable mixtures in air at room temperature - Danger of remote

ignition and flashback

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid breathing vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep container closed. Keep

away from heat, sparks, and open flame.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: F Xi

Indication of Danger: Highly Flammable. Irritant.

R: 11 41 67

Risk Statements: Highly flammable. Risk of serious damage to

eyes. Vapors may cause drowsiness and dizziness.

S: 7 16 24 26 39

Safety Statements: Keep container tightly closed. Keep away from

sources of ignition - no smoking. Avoid contact with skin. In

case of contact with eyes, rinse immediately with plenty of

water and seek medical advice. Wear eye/face protection.

Immediately Dangerous to Life and Health  800 ppm

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  200 ppm

DOE Short Term Exposure Limit  250 ppm

DOE Ceiling Limit 250 ppm

Immediately Dangerous to Life and Health   800 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPANOL (2-)    CAS # 67630

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   .   .   .   H   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   3   0

ACUTE TOXICTY RISK INDEX 2 - LD50  5045.0 mg/Kg

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION HAZARD  INHALATION RISK INDEX   2.5 - LC50  39249.0

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Nerves. Kidneys. Cardiovascular system. G.I. System.

Overexposure may cause mild, reversible liver effects.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

Drowsiness. Narcotic effect. Nausea, headache, and vomiting.

Prolonged exposure can cause: Can cause CNS depression.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

VAPOR PRESSURE   33.0 mm Hg @ 20 °C

FLASH POINT  53 °F

Forms ignitable mixtures in air at room temperature - Danger of remote

ignition and flashback

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

INCOMPATIBILITIES:Aluminum, Halogens, Acid anhydrides, Acids, Oxidizing

agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat, sparks, and open flame. Store in a cool dry place. Handle and

store under   nitrogen\. SPECIAL REQUIREMENTS Handle and store under inert

gas. Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: F-Xi

Indication of Danger: Highly Flammable. Irritant.

R: 11-36-67

Risk Statements: Highly flammable. Irritating to eyes. Vapors

may cause drowsiness and dizziness.

S: 7-16-24/25-26

Safety Statements: Keep container tightly closed. Keep away from

sources of ignition - no smoking. Avoid contact with skin and

eyes. In case of contact with eyes, rinse immediately with

plenty of water and seek medical advice.

OSHA REGULATORY LIMITS

OSHA Permissible Exposure Limit 400 ppm

OSHA Short Term Exposure Limit  500 ppm

ACGIH RECOMMENDED LIMITS

ACGIH Threshold Limit Value 200 ppm

ACGIH Short Term Exposure Limit 400 ppm

Immediately Dangerous to Life and Health  2000 ppm

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  400 ppm

DOE Short Term Exposure Limit  400 ppm

DOE Ceiling Limit 2000 ppm

Immediately Dangerous to Life and Health   2000 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPARGYL ALCOHOL    CAS # 107197

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   C   D   E   F   G   .   I   J   K   .

NFPA HAZARD CODES (H,F,R,O)  4   3   0

EXTREMELY TOXIC - DESIGNATED AREA MAY BE REQUIRED

ACUTE TOXICTY RISK INDEX 4.4 - LD50    20.0 mg/Kg

NEUROTOXIC - RISK INDEX   5.0

INHALATION HAZARD  INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: Readily absorbed through skin. May be fatal if

absorbed through skin.

Eye Contact: Causes burns. Lachrymator.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be fatal

if inhaled.

Ingestion: May be fatal if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Liver. Kidneys.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

FLASH POINT  91.4 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC   CORROSIVE

INCOMPATIBILITIES:Strong acids, Strong bases, Amines,  Phosphorous pentoxide,

Isocyanate vapor, Oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Carbon Dioxide, dry chemical powder, or appropriate    foam. Water can be

applied as a spray or fog and if properly    applied is capable of

extinguishing the fi

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

STORAGE: Keep tightly closed. Keep away from heat, sparks, and open flame.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T-N

Indication of Danger: Toxic. Dangerous for the environment.

R: 10-23/24/25-34-51/53

Risk Statements: Flammable. Toxic by inhalation, in contact with

skin and if swallowed. Causes burns. Toxic to aquatic organisms,

may cause long-term adverse effects in the aquatic environment.

S: 26-28-36-45-61

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. After

contact with skin, wash immediately with plenty of soap-suds.

Wear suitable protective clothing. In case of accident or if you

feel unwell, seek medical advice immediately (show the label

where possible). Avoid release to the environment. Refer to

special instructions/safety data sheets.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  1 ppm

DOE Short Term Exposure Limit  1.5 ppm

DOE Ceiling Limit 12.5 ppm

Immediately Dangerous to Life and Health   60 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPARGYL BENZENESULFONATE    CAS # 6165759

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  212 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place. Store under nitrogen.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPARGYL BROMIDE    CAS # 106967   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  3   3   1

ACUTE TOXICTY RISK INDEX 4 - LD50    53.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation. Lachrymator.

Inhalation: May be harmful if inhaled. Material is irritating to

mucous membranes and upper respiratory tract.

Ingestion: Toxic if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Central nervous system.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause: Headache.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

FLASH POINT  64.4 °F

Forms ignitable mixtures in air at room temperature - Danger of remote

ignition and flashback

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Bases, Peroxides Copper, Copper

alloys Mercury/mercury oxides., Silver/silver oxides

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen bromide gas

Store at 2-8°C

Keep tightly closed. Keep away from heat, sparks, and open flame.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat, sparks, and open flame\. Store at 2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: F T

Indication of Danger: Highly Flammable. Toxic.

R: 11 25 36/37/38

Risk Statements: Highly flammable. Toxic if swallowed.

Irritating to eyes, respiratory system and skin.

S: 16 26 36/37/39 45

Safety Statements: Keep away from sources of ignition - no

smoking. In case of contact with eyes, rinse immediately with

plenty of water and seek medical advice. Wear suitable

protective clothing, gloves, and eye/face protection. In case of

accident or if you feel unwell, seek medical advice immediately

(show the label where possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .03 mg/m3

DOE Short Term Exposure Limit  .03 mg/m3

DOE Ceiling Limit .03 mg/m3

Immediately Dangerous to Life and Health   20 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPARGYLAMINE HYDROCHLORIDE    CAS # 15430521

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be fatal if inhaled, swallowed, or absorbed

through skin. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Acids, Acid chlorides, Acid anhydrides,  Oxidizing agents.

FIRE EXTINGUISHER:  Dry chemical powder. Carbon dioxide, dry chemical powder,

or appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: T

Indication of Danger: Toxic.

R: 36/37/38 25

Risk Statements: Irritating to eyes, respiratory system and

skin. Toxic if swallowed.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPARGYLOXY PHTHALIMIDE (N-)    CAS # 4616631

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPENE    CAS # 115071

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  1   4   0

ACUTE TOXICTY RISK INDEX 2.7 - LD50   910.0 mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   4.7 - LC50

ROUTE OF EXPOSURE

skin Contact: Can cause severe frostbite.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract. Can

cause rapid suffocation.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Acts as a simple asphyxiant. Symptoms of overexposure include

dizziness, disorientation, headache, excitation, central nervous

system depression and anesthesia. Exposure can cause: CNS

depression.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Gas

Flammable

VAPOR PRESSURE 6209.0 mm Hg @ 20 °C

FLASH POINT  -162 °F

Store in an explosion-proof refrigerator or tightly stoppered in a

well-ventilated area

SEGREGATION: SHELF # 3

STORAGE GROUP(S):

h - Flammable/Pyrophoric Gas

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Use water spray or fog nozzle to keep cylinder cool.Move

cylinder away from fire if there is no risk.

REACTIVE PROPERTIES

HANDLING: Do not breathe gas. Do not get in eyes, on skin, on clothing. Avoid

prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away from

heat, sparks, and open flame. Use with equipment rated for cylinder pressure,

and   of compatible materials of construction. Close valve when not in   use

and when empty. Make sure cylinder is properly secured when   in use or

stored Cylinder temperature should not exceed 125░F   (52░C).   Unsuitable:

Store away from heat and direct sunlight     SPECIAL REQUIREMENTS   Contents

under pressure.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: F+

Indication of Danger: Extremely Flammable.

R: 12

Risk Statements: Extremely flammable.

S: 9 16 33

Safety Statements: Keep container in a well-ventilated place.

Keep away from sources of ignition - no smoking. Take

precautionary measures against static discharges.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  500 ppm

DOE Short Term Exposure Limit  1500 ppm

DOE Ceiling Limit 10000 ppm

Immediately Dangerous to Life and Health   20000 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPENYLMAGNESIUM BROMIDE (1-)    CAS # 14092047   HAZARDOUS CHEMICAL OF

CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   3   2   W

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin.

TARGET ORGAN(S) OR SYSTEM(S)

Kidneys. Liver. Central nervous system.

SIGNS AND SYMPTOMS OF EXPOSURE

Narcotic effect. Coughing, chest pains, difficulty in breathing.

Nausea, dizziness, and headache. Unconsciousness. Exposure can

cause: Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Can cause CNS depression.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

Explosive

FLASH POINT  -4 °F

Store in an explosion-proof refrigerator or tightly stoppered in a

well-ventilated area

Forms explosive unstable peroxides

Test periodically or discard

Reacts violently with water

SEGREGATION: SHELF # 3

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

k - Explosive/Unstable

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   REACTIVE

INCOMPATIBILITIES:Alcohols, Acids, Water.

FIRE EXTINGUISHER:  Dry chemical powder.  Do not use water.

TOXIC EMISSIONS WHEN BURNED: Hydrogen bromide gas Carbon  monoxide

Store at 2-8°C

Keep tightly closed. Keep away from heat, sparks, and open flame. Do not

allow contact with water

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat, sparks, and open flame\. Incompatible Materials: Do not allow

contact with water Store at 2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: F-C

Indication of Danger: Highly Flammable. Corrosive.

R: 11-14/15-19-20/21/22-34

Risk Statements: Highly flammable. Reacts violently with water,

liberating extremely flammable gases. May form explosive

peroxides. Harmful by inhalation, in contact with skin and if

swallowed. Causes burns.

S: 16-26-27-36/37/39-45

Safety Statements: Keep away from sources of ignition - no

smoking. In case of contact with eyes, rinse immediately with

plenty of water and seek medical advice. Take off immediately

all contaminated clothing. Wear suitable protective clothing,

gloves, and eye/face protection. In case of accident or if you

feel unwell, seek medical advice immediately (show the label

where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPIDIUM IODIDE    CAS # 25535164

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

TOXICITY DAT

Subcutaneous

Mouse

16 MG/KG

LD50

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen iodide

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

the dark\. SPECIAL REQUIREMENTS  Light sensitive. Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPIOLALDEHYDE DIETHYL ACETAL    CAS # 10160879

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   3   1

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Vapor or mist is irritating to the eyes, mucous

membranes, and upper respiratory tract.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

FLASH POINT  87.8 °F

Forms ignitable mixtures in air at room temperature - Danger of remote

ignition and flashback

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE

INCOMPATIBILITIES:Strong acids, Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at 2-8°C

Keep tightly closed. Keep away from heat, sparks, and open flame.

Moisture sensitive.

REACTIVE PROPERTIES

HANDLING: Avoid contact and inhalation. Do not get in eyes, on skin, on

clothing. STORAGE: Keep tightly closed. Keep away from heat, sparks, and open

flame. Store at 2-8░C SPECIAL REQUIREMENTS  Moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 10 36/37/38

Risk Statements: Flammable. Irritating to eyes, respiratory

system and skin.

S: 16 26 36

Safety Statements: Keep away from sources of ignition - no

smoking. In case of contact with eyes, rinse immediately with

plenty of water and seek medical advice. Wear suitable

protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPIOLIC ACID    CAS # 471250

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   G   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  0   2   0

ACUTE TOXICTY RISK INDEX 3.7 - LD50   100.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: May be fatal if inhaled, swallowed, or absorbed

through skin.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  136.4 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

INCOMPATIBILITIES:Bases, Oxidizing agents, Reducing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Water spray.

Store at 2-8°C

Keep tightly closed. Keep away from heat and open flame.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame. Store at 2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: T

Indication of Danger: Toxic.

R: 24/25 34

Risk Statements: Toxic in contact with skin and if swallowed.

Causes burns.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPIONALDEHYDE    CAS # 123386   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   3   0

ACUTE TOXICTY RISK INDEX 2.5 - LD50  1410.0 mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   3.5 - LC50  21800.0

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: May be harmful if inhaled. Material is extremely

destructive to the tissue of the mucous membranes and upper

respiratory tract.

Ingestion: Harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Liver.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Exposure can cause: Nausea, dizziness, and headache.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

VAPOR PRESSURE  258.0 mm Hg @ 20 °C

FLASH POINT  -16 °F

Store in an explosion-proof refrigerator or tightly stoppered in a

well-ventilated area

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

INCOMPATIBILITIES:Oxidizing agents, Strong bases, Strong reducing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Water spray.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Open carefully. STORAGE: Keep tightly closed. Keep away from heat, sparks,

and open flame. Store under nitrogen\. SPECIAL REQUIREMENTS   May develop

pressure. Open carefully.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: F Xi

Indication of Danger: Highly Flammable. Irritant.

R: 11 36/37/38

Risk Statements: Highly flammable. Irritating to eyes,

respiratory system and skin.

S: 9 16 29

Safety Statements: Keep container in a well-ventilated place.

Keep away from sources of ignition - no smoking. Do not empty

into drains.

ACGIH RECOMMENDED LIMITS

ACGIH Threshold Limit Value 20 ppm

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  20 ppm

DOE Short Term Exposure Limit  45 ppm

DOE Ceiling Limit 260 ppm

Immediately Dangerous to Life and Health   840 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPIONAMIDE    CAS # 79050

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

VAPOR PRESSURE .0390 mm Hg @ 20 °C

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Acids, Bases, Strong reducing

agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Do not get in eyes, on skin, on   clothing.

STORAGE: Keep tightly closed. Store in a cool dry place.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPIONIC ACID    CAS # 79094

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   2   0

ACUTE TOXICTY RISK INDEX 3 - LD50  2600.0 mg/Kg

INHALATION RISK INDEX   1.8 - LC50   4900.0

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: Readily absorbed through skin. Harmful if

absorbed through skin.

Eye Contact: Causes burns.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be harmful

if inhaled.

Ingestion: May be harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Liver. Kidneys. Blood. Central nervous system.

SIGNS AND SYMPTOMS OF EXPOSURE

Nausea, dizziness, and headache. Blood effects. May cause an

asthmatic-like bronchitis. Symptoms of exposure may include

burning sensation, coughing, wheezing, laryngitis, shortness of

breath, headache, nausea, and vomiting. Inhalation may result in

spasm, inflammation and edema of the larynxand bronchi, chemical

pneumonitis, and pulmonary edema. Material is extremely

destructive to tissue of the mucous membranes and upper

respiratory tract, eyes, and skin.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE    2.4 mm Hg @ 20 °C

FLASH POINT  129.2 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

d - Organic Acid/Flammable/Toxic

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34

Risk Statements: Causes burns.

S: 23-36-45

Safety Statements: Do not breathe vapor. Wear suitable

protective clothing. In case of accident or if you feel unwell,

seek medical advice immediately (show the label where possible).

OSHA REGULATORY LIMITS

OSHA Permissible Exposure Limit 10 ppm

ACGIH RECOMMENDED LIMITS

ACGIH Threshold Limit Value 10 ppm

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  10 ppm

DOE Short Term Exposure Limit  10 ppm

DOE Ceiling Limit 15 ppm

Immediately Dangerous to Life and Health   350 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPIONIC ANHYDRIDE    CAS # 123626

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   1   0

ACUTE TOXICTY RISK INDEX 2.3 - LD50  2360.0 mg/Kg

INHALATION RISK INDEX   2.0 - LC50

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE    3.5 mm Hg @ 20 °C

FLASH POINT  208.4 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Alcohols.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam\. Protective Equipment: Wear self-contained breathing

apparatus and protective clothing to prevent contact with skin and eyes.

Specific Hazard(s): Emits t

REACTIVE PROPERTIES

HANDLING skin, on clothing. Avoid prolonged or repeated exposure. STORAGE:

Keep tightly closed. Keep away from heat and open flame. Store in a cool dry

place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

Risk Statements: Causes burns.

S: 26 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. In

immediately (show the label where possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  10 mg/m3

DOE Short Term Exposure Limit  30 mg/m3

DOE Ceiling Limit 200 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPIONITRILE    CAS # 107120

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   C   D   E   .   .   .   I   J   K   .

NFPA HAZARD CODES (H,F,R,O)  4   3   0

EXTREMELY TOXIC - DESIGNATED AREA MAY BE REQUIRED

ACUTE TOXICTY RISK INDEX 5 - LD50    39.0 mg/Kg

NEUROTOXIC - RISK INDEX   5.0

INHALATION HAZARD  INHALATION RISK INDEX   4.1 - LC50    113.6

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be fatal if absorbed through skin. Readily

absorbed through skin.

Eye Contact: Causes eye irritation.

Inhalation: Harmful if inhaled. Material may be irritating to

mucous membranes and upper respiratory tract.

Ingestion: Toxic if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Blood. Central nervous system.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause: Lung irritation. Cyanosis. CNS depression.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

VAPOR PRESSURE   34.7 mm Hg @ 20 °C

FLASH POINT  43 °F

Forms ignitable mixtures in air at room temperature - Danger of remote

ignition and flashback

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

INCOMPATIBILITIES:Strong acids, Strong bases, Strong oxidizing agents, Strong

reducing agents.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Emits hydrogen cyanide gas in

contact with acids

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat, sparks, and open flame. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: F T+

Indication of Danger: Highly Flammable. Very toxic.

R: 11 20 25 27 36

Risk Statements: Highly flammable. Harmful by inhalation. Toxic

if swallowed. Very toxic in contact with skin. Irritating to

eyes.

S: 9 16 28 36/37 45

Safety Statements: Keep container in a well-ventilated place.

Keep away from sources of ignition - no smoking. After contac   with skin,

wash immediately with plenty of soap and water. Wear

suitable protective clothing and gloves. In case of accident or

if you feel unwell, seek medical advice immediately (show the

label where possible).

Immediately Dangerous to Life and Health  25 mg/m3

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  6 ppm

DOE Short Term Exposure Limit  6 ppm

DOE Ceiling Limit 7 ppm

Immediately Dangerous to Life and Health   37 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPIONYL CHLORIDE    CAS # 79038   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   3   1   W

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns. Lachrymator.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be harmful

if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

FLASH POINT  42.8 °F

Forms ignitable mixtures in air at room temperature - Danger of remote

ignition and flashback

Reacts violently with water

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   REACTIVE   TOXIC

INCOMPATIBILITIES:Alcohols, Oxidizing agents, Strong bases.

FIRE EXTINGUISHER:  Carbon dioxide.  Do not use water.

TOXIC EMISSIONS WHEN BURNED: Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat, sparks, and open flame.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: F C

Indication of Danger: Highly Flammable. Corrosive.

R: 11 14 34

Risk Statements: Highly flammable. Reacts violently with water.

Causes burns.

S: 9 16 26 45

Safety Statements: Keep container in a well-ventilated place.

Keep away from sources of ignition - no smoking. In case of

contact with eyes, rinse immediately with plenty of water and

seek medical advice. In case of accident or if you feel unwell,

seek medical advice immediately (show the label where possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .075 ppm

DOE Short Term Exposure Limit  .2 ppm

DOE Ceiling Limit 1.5 ppm

Immediately Dangerous to Life and Health   7.5 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPIONYLTHIOCHOLINE IODIDE    CAS # 1866735

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

Eye Contact: Causes severe eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide. Dry chemical powder. Water spray.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen iodide Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place\. SPECIAL REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38-41

Risk Statements: Irritating to eyes, respiratory system and

skin. Risk of serious damage to eyes.

S: 26-36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPIOPHENONE    CAS # 93550

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   1   0

ACUTE TOXICTY RISK INDEX 2 - LD50  4490.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE .152 mm Hg @ 20 °C

FLASH POINT  210.2 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPRANOLOL HYDROCHLORIDE    CAS # 318989

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 3 - LD50   466.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: Harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Cardiovascular system.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22

Risk Statements: Harmful if swallowed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPYL ACETATE (1-)    CAS # 109604

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   .   .   .   H   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   3   0

ACUTE TOXICTY RISK INDEX 1.7 - LD50  9370.0 mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   2.2 - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Vapor or mist is irritating to the eyes, mucous

membranes, and upper respiratory tract.

SIGNS AND SYMPTOMS OF EXPOSURE

Prolonged exposure can cause: Narcotic effect.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

VAPOR PRESSURE   25.0 mm Hg @ 20 °C

FLASH POINT  57.2 °F

Forms ignitable mixtures in air at room temperature - Danger of remote

ignition and flashback

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE

INCOMPATIBILITIES:Oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Carbon Dioxide, dry chemical powder, or appropriate    foam. Water can be

applied as a spray or fog and if properly    applied is capable of

extinguishing the fi

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat, sparks, and open flame.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: F Xi

Indication of Danger: Highly Flammable. Irritant.

R: 11 36 66 67

Risk Statements: Highly flammable. Irritating to eyes. Repeated

exposure may cause skin dryness or cracking. Vapors may cause

drowsiness and dizziness.

S: 16 26 29 33

Safety Statements: Keep away from sources of ignition - no

smoking. In case of contact with eyes, rinse immediately with

plenty of water and seek medical advice. Do not empty into

drains. Take precautionary measures against static discharges.

Immediately Dangerous to Life and Health  1700 ppm

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  200 ppm

DOE Short Term Exposure Limit  250 ppm

DOE Ceiling Limit 1000 ppm

Immediately Dangerous to Life and Health   1700 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPYL ASTRA BLUE IODIDE    CAS # 199620183

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

Chronic copper poisoning is typified by hepatic cirrhosis, brain

damage and demyelination, kidney defects, and copper deposition

in the cornea as exemplified by humans with Wilson's disease. It

has also been reported that copper poisoning has lead to

hemolytic anemia and accelerates arteriosclerosis. To the best

of our knowledge, the chemical, physical, and toxicological

properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxides Hydrogen iodide

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Store in a cool dry place\. SPECIAL

REQUIREMENTS  Light sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 37/39

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable gloves and eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPYL GALLATE    CAS # 121799

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  1   2   0

ACUTE TOXICTY RISK INDEX 2.4 - LD50  2100.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Absorption: May be harmful if absorbed through the skin.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: Harmful if swallowed.

Multiple Routes: Causes eye and skin irritation.

SENSITIZATION

skin: May cause allergic skin reaction.

TOXICITY DAT   Oral

Rat

3,000 mg/kg

LD50

Oral

Mouse

2,000 mg/kg

LD50

Oral

Rat

2100 mg/kg

LD50

Intraperitoneal

Rat

380 MG/KG

LD50

Remarks: Behavioral:Convulsions or effect on seizure threshold.

Lungs, Thorax, or Respiration:Dyspnea. Lungs, Thorax, or

Respiration:Other changes.

Oral

Mouse

1700 mg/kg

LD50

Oral

Rabbit

2750 mg/kg

LD50

Oral

Pig

> 6000 mg/kg

LD50

Remarks: Kidney, Ureter, Bladder:Other changes.

Oral

Hamster

2480 mg/kg

LD50

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  187 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids, Strong bases, Strong

reducing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Water spray.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Do not get in eyes, on skin, on   clothing. Avoid

prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in a cool

dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22 43

Risk Statements: Harmful if swallowed. May cause sensitization

by skin contact.

S: 24 37

Safety Statements: Avoid contact with skin. Wear suitable gloves.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPYL ISOCYANATE (N-)    CAS # 110781

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   G   .   .   J   K   L

NFPA HAZARD CODES (H,F,R,O)  3   3   0

ACUTE TOXICTY RISK INDEX 3.2 - LD50   316.2 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: Harmful if absorbed through skin.

Eye Contact: Causes severe eye irritation. Lachrymator.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. Harmful if inhaled.

Ingestion: Toxic if swallowed.

SENSITIZATION

Sensitization: May cause allergic reaction.

skin: May cause allergic skin reaction.

SIGNS AND SYMPTOMS OF EXPOSURE

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Repeated exposure may cause asthma. Prolonged contact

can cause:

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

FLASH POINT  23 °F

Store in an explosion-proof refrigerator or tightly stoppered in a

well-ventilated area

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

INCOMPATIBILITIES:Water, Alcohols, Strong bases, Amines, Acids,  Strong

oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide. Dry chemical powder.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat, sparks, and open flame. Store under nitrogen.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: F Xn

Indication of Danger: Highly Flammable. Harmful.

R: 11 20/21/22 37/38 41 42/43

Risk Statements: Highly flammable. Harmful by inhalation, in

contact with skin and if swallowed. Irritating to respiratory

system and skin. Risk of serious damage to eyes. May cause

sensitization by inhalation and skin contact.

S: 23 36/37/39 26 45

Safety Statements: Do not breathe vapor. Wear suitable

protective clothing, gloves, and eye/face protection. In case of

contact with eyes, rinse immediately with plenty of water and

seek medical advice. In case of accident or if you feel unwell,

seek medical advice immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPYLAGAROSE    CAS # 39347143

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at 2-8°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. Store at

2-8°C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPYLAMINE    CAS # 107108

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   .   F   G   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  3   4   1

ACUTE TOXICTY RISK INDEX 3.1 - LD50   370.0 mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   4.0 - LC50   2500.0

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: Harmful if absorbed through skin. Readily

absorbed through skin.

Eye Contact: Causes burns.

Inhalation: Harmful if inhaled. Material is extremely

destructive to the tissue of the mucous membranes and upper

respiratory tract.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

VAPOR PRESSURE  247.6 mm Hg @ 20 °C

FLASH POINT  -22 °F

Store in an explosion-proof refrigerator or tightly stoppered in a

well-ventilated area

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

a - Organic Base/Flammable/Toxic

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC   CORROSIVE

INCOMPATIBILITIES:Acids, Acid chlorides, Acid anhydrides, Strong oxidizing

agents.

FIRE EXTINGUISHER:  For small (incipient) fires, use media such as"alcohol"

foam, dry chemical, or carbon dioxide. For large   fires, apply water from as

far as possible. Use very large   quantities (flooding) of water applied as a

mist or spray; solid

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat, sparks, and open flame\. SPECIAL REQUIREMENTS   May develop

pressure. Open carefully.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: F-C

Indication of Danger: Highly Flammable. Corrosive.

R: 11-20/21/22-34

Risk Statements: Highly flammable. Harmful by inhalation, in

contact with skin and if swallowed. Causes burns.

S: 7/9-16-26-36-45

Safety Statements: Keep container tightly closed and in

well-ventilated place. Keep away from sources of ignition - no

smoking. In case of contact with eyes, rinse immediately with

plenty of water and seek medical advice. Wear suitable

protective clothing. In case of accident or if you feel unwell,

seek medical advice immediately (show the label where possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  15 ppm

DOE Short Term Exposure Limit  50 ppm

DOE Ceiling Limit 250 ppm

Immediately Dangerous to Life and Health   250 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPYLANILINE (2-)    CAS # 1821392

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

ACUTE TOXICTY RISK INDEX 2.4 - LD50  1960.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Vapor or mist is irritating to the eyes, mucous

membranes, and upper respiratory tract.

SIGNS AND SYMPTOMS OF EXPOSURE

Absorption into the body leads to the formation of methemoglobin

which in sufficient concentration causes cyanosis. Onset may be

delayed 2 to 4 hours or longer. To the best of our knowledge,

the chemical, physical, and toxicological properties have not

been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  208 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid contact and inhalation. Do not get in eyes, on skin, on

clothing. STORAGE: Keep tightly closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 37/39

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable gloves and eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPYLBENZODIOXABOROLE (2:1,3,2-)    CAS # 40218493

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   2   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  151 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Boron oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34

Risk Statements: Causes burns.

S: 26 27 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Take

off immediately all contaminated clothing. Wear suitable

protective clothing, gloves, and eye/face protection. In case of

accident or if you feel unwell, seek medical advice immediately

(show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPYLCYCLOPROPANE METHYLAMINE (N-)    CAS # 26389606

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   3   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Vapor or mist is irritating to the eyes, mucous

membranes, and upper respiratory tract.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

FLASH POINT  87 °F

Forms ignitable mixtures in air at room temperature - Danger of remote

ignition and flashback

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids, Acid  chlorides,

Acid anhydrides.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat, sparks, and open flame. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 10 36/37/38

Risk Statements: Flammable. Irritating to eyes, respiratory

system and skin.

S: 16 26 36/37/39

Safety Statements: Keep away from sources of ignition - no

smoking. In case of contact with eyes, rinse immediately with

plenty of water and seek medical advice. Wear suitable

protective clothing, gloves, and eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPYLENE CARBONATE    CAS # 108327

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   1   0

ACUTE TOXICTY RISK INDEX 3 - LD50 29100.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50   5000.0

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause: Nausea, headache, and vomiting. CNS

depression. To the best of our knowledge, the chemical,

physical, and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE .130 mm Hg @ 20 °C

FLASH POINT  270 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place. Handle   and store under nitrogen\. SPECIAL REQUIREMENTS

Moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36

Risk Statements: Irritating to eyes.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .125 mg/m3

DOE Short Term Exposure Limit  .35 mg/m3

DOE Ceiling Limit 2.5 mg/m3

Immediately Dangerous to Life and Health   12.5 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPYLENE OXIDE    CAS # 75569   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   E   .   G   .   I   J   K   .

NFPA HAZARD CODES (H,F,R,O)  3   4   0

ACUTE TOXICTY RISK INDEX 3.1 - LD50   380.0 mg/Kg

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION HAZARD  INHALATION RISK INDEX   4.1 - LC50   1278.7

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: Harmful if absorbed through skin. Readily

absorbed through skin.

Eye Contact: Causes burns.

Inhalation: Harmful if inhaled. Material is extremely

destructive to the tissue of the mucous membranes and upper

respiratory tract.

Ingestion: Harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Central nervous system.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Can cause CNS depression.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

VAPOR PRESSURE  444.1 mm Hg @ 20 °C

FLASH POINT  -34.6 °F

Store in an explosion-proof refrigerator or tightly stoppered in a

well-ventilated area

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

INCOMPATIBILITIES:Oxidizing agents Copper, Copper alloys, Strong acids,

Strong bases, Peroxides, Alkali, Amines.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Carbon Dioxide, dry chemical powder, or appropriate   foam. Water can be

applied as a spray or fog and if properly applied is capable of extinguishing

the fire by

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep container closed. Keep

away from heat, sparks, and open flame\. SPECIAL REQUIREMENTS   May develop

pressure. Open carefully. Heat sensitive. Cool to   0░C before opening.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: F+ T

Indication of Danger: Extremely Flammable. Toxic.

R: 45 46 12 20/21/22 36/37/38

Risk Statements: May cause cancer. May cause heritable genetic

damage. Extremely flammable. Harmful by inhalation, in contact

with skin and if swallowed. Irritating to eyes, respiratory

system and skin.

S: 53 45

Safety Statements: Restricted to professional users. Attention -

Avoid exposure - obtain special instructions before use. In case

of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

OSHA REGULATORY LIMITS

OSHA Permissible Exposure Limit 20 ppm

Immediately Dangerous to Life and Health  400 ppm

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  20 ppm

DOE Short Term Exposure Limit  73 ppm

DOE Ceiling Limit 290 ppm

Immediately Dangerous to Life and Health   870 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPYLENEDIAMINETETRAACETIC ACID    CAS # 4408815

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

d - Organic Acid/Flammable/Toxic

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases Copper,  Copper

alloys, Nickel.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

STORAGE: Keep tightly closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPYLPENTANOIC ACID (2-)    CAS # 99661

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   1   0

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material is irritating to

mucous membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Liver. Nerves. Blood.

SIGNS AND SYMPTOMS OF EXPOSURE

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  232 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Bases, Oxidizing agents, Reducing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22 36/37/38

Risk Statements: Harmful if swallowed. Irritating to eyes,

respiratory system and skin.

S: 26

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROPYNE (1-)    CAS # 74997

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   H   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   4   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation. Can cause severe

frostbite.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material is irritating to

mucous membranes and upper respiratory tract. Can cause rapid

suffocation.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Prolonged exposure can cause: Narcotic effect. symptoms of

overexposure may include rapid respiration, diminished mental

alertness, faulty judgement, rapid fatigue, nausea, loss of

consciousness, coma, and possibly death.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Gas

Flammable

VAPOR PRESSURE   82.7 mm Hg @ 20 °C

FLASH POINT  -60 °F

Store in an explosion-proof refrigerator or tightly stoppered in a

well-ventilated area

SEGREGATION: SHELF # 3

STORAGE GROUP(S):

h - Flammable/Pyrophoric Gas

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Oxygen.

FIRE EXTINGUISHER:  Use water spray or fog nozzle to keep cylinder cool. Move

cylinder away from fire if there is no risk.

REACTIVE PROPERTIES

HANDLING: Do not breathe gas. Do not get in eyes, on skin, on clothing. Avoid

prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away from

heat, sparks, and open flame. Use with equipment rated for cylinder pressure,

and    of compatible materials of construction. Close valve when not in

use and when empty. Make sure cylinder is properly secured when    in use or

stored Cylinder temperature should not exceed 125░F    (52░C).    Unsuitable:

Store away from heat and direct sunlight       SPECIAL REQUIREMENTS

Contents under pressure.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: F Xi

Indication of Danger: Highly Flammable. Irritant.

R: 11 37

Risk Statements: Highly flammable. Irritating to respiratory

system.

S: 16 36 38

Safety Statements: Keep away from sources of ignition - no

smoking. Wear suitable protective clothing. In case of

insufficient ventilation, wear suitable respiratory equipment.

Immediately Dangerous to Life and Health  1700 ppm

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  1000 ppm

DOE Short Term Exposure Limit  1700 ppm

DOE Ceiling Limit 1700 ppm

Immediately Dangerous to Life and Health   1700 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROTAMINE    CAS # 9012004

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  1   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SENSITIZATION

Sensitization: Prolonged or repeated exposure may cause allergic

reactions in certain sensitive individuals.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause a sensation of warmth, flushing of the skin,

hypotension, slow pulse rate, and difficulty in breathing.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Sulfur oxides Nitrogen oxides

Store at -20°C

REACTIVE PROPERTIES

STORAGE Store at -20░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROTAMINE SULFATE    CAS # 53597254

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Materials to Avoid: S

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 24/25 22

Safety Statements: Avoid contact with skin and eyes. Do not

breathe dust.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROTAMINE SULFATE    CAS # 9007312

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Materials to Avoid: S

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROTEASE    CAS # 9073783

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 2.2 - LD50  3290.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Absorption: May be harmful if absorbed through the skin.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. Harmful if inhaled.

Ingestion: May be harmful if swallowed.

Multiple Routes: Causes irritation.

SENSITIZATION

Sensitization: May cause allergic respiratory and skin reactions

TARGET ORGAN(S) OR SYSTEM(S)

Lungs.

SIGNS AND SYMPTOMS OF EXPOSURE

Repeated exposure can cause: Asthma.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at -20°C

REACTIVE PROPERTIES

STORAGE   Store at -20░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 36/37/38 42/43

Risk Statements: Irritating to eyes, respiratory system and

skin. May cause sensitization by inhalation and skin contact.

S: 22 26 36/37

Safety Statements: Do not breathe dust. In case of contact with

eyes, rinse immediately with plenty of water and seek medical

advice. Wear suitable protective clothing and gloves.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROTEINASE K    CAS # 39450016

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  3   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SENSITIZATION

Sensitization: May cause allergic respiratory and skin reactions

TARGET ORGAN(S) OR SYSTEM(S)

Lungs.

SIGNS AND SYMPTOMS OF EXPOSURE

Repeated exposure can cause: Asthma.

CONDITIONS AGGRAVATED BY EXPOSURE

Proteinase K (Endopeptidase K) is an enzyme derived from

bacteria that selectively breaks down proteins wherever a serine

molecule occurs in a protein. The toxicological properties have

not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at -20°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

-20░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 36/37/38 42

Risk Statements: Irritating to eyes, respiratory system and

skin. May cause sensitization by inhalation.

S: 22 24 26 36/37

Safety Statements: Do not breathe dust. Avoid contact with skin.

In case of contact with eyes, rinse immediately with plenty of

water and seek medical advice. Wear suitable protective clothing

and gloves.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROTOPORPHYRIN IX    CAS # 553128

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

Store at 2-8°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. Store at

2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PROTOPORPHYRIN IX DISODIUM    CAS # 50865015

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 1.5 - LD50 16129.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

Store at 2-8°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. Store at

2-8░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PSEUDOEPHEDRINE (1S,2S-)    CAS # 90824

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 2.9 - LD50   660.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin. Causes eye and skin irritation.

TARGET ORGAN(S) OR SYSTEM(S)

Central nervous system. Heart.

SIGNS AND SYMPTOMS OF EXPOSURE

The chemical, physical, and toxicological properties of this

product have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 20/21/22 36/37/38

Risk Statements: Harmful by inhalation, in contact with skin and

if swallowed. Irritating to eyes, respiratory system and skin.

S: 26 37/39

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable gloves and eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PULLULAN    CAS # 9057027

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. May cause irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PUMICE    CAS # 1332098

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PUMP OIL    CAS # 64742650

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  0   0   1   OX

ACUTE TOXICTY RISK INDEX 2 - LD50  5000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Oxidizing Agent

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T

Indication of Danger: Toxic.

R: 45

Risk Statements: May cause cancer.

S: 53 45

Safety Statements: Avoid exposure - obtain special instructions

before use. In case of accident or if you feel unwell, seek

medical advice immediately (show the label where possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  100 mg/m3

DOE Short Term Exposure Limit  300 mg/m3

DOE Ceiling Limit 500 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PURINE    CAS # 120730

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

INHALATION RISK INDEX   <1  - LC50

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Purine is a very strong metabolism antagonist and inhibits the

growth of cells and tissues.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

investigated.

Intraperitoneal

Rat

800 MG/KG

LD50

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Materials to Avoid: S

FIRE EXTINGUISHER: appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on STORAGE: Keep tightly

closed. Keep away from heat, sparks, and open flame. Store in a cool dry

place. Store under nitrogen. SPECIAL REQUIREMENTS  Store under inert gas. Air

sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PUROMYCIN DIHYDROCHLORIDE    CAS # 58582

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

ACUTE TOXICTY RISK INDEX 2.3 - LD50  2322.6 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Liver. Pancreas. Kidneys.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

Store at -20°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

-20░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22

Risk Statements: Harmful if swallowed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PUTRESCINE DIHYDROCHLORIDE    CAS # 333937

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 2.7 - LD50  1000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYBOP    CAS # 128625525

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen fluoride Phosphorous

oxides

Store at 2-8°C

Keep tightly closed.

Light sensitive. Moisture sensitive.

REACTIVE PROPERTIES

HANDLING: Avoid contact with eyes, skin, and clothing.    Avoid prolonged or

repeated exposure. STORAGE: Keep tightly closed. Store at 2-8░C SPECIAL

REQUIREMENTS  Light sensitive. Moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRAZINE    CAS # 290379   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   2   2

ACUTE TOXICTY RISK INDEX 2.2 - LD50  2730.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

Explosive

FLASH POINT  131 °F

Forms explosive unstable peroxides

Test periodically or discard

SEGREGATION: SHELF # 3

STORAGE GROUP(S):

k - Explosive/Unstable

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   REACTIVE   TOXIC

INCOMPATIBILITIES:Acids, Oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 3   Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRAZOLE    CAS # 288131

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 2.7 - LD50  1010.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Teratogenic in rats, producing a high incidence of ocular and

urinary bladder anomalies. Rats fed pyrazole for 24 days showed

retarded growth, increased liver weight, changes in the

mitochondria and smooth endoplasmic reticulum,

hyperbilirubinemia, and hypoglycemia. Rats dying after lethal

doses (6-11 days) showed extensive centrolobular necrosis with

inflammatory reactions in the parenchyma, and fatty changes in

surviving cells.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Acids, Oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22 36/37/38 52

Risk Statements: Harmful if swallowed. Irritating to eyes,

respiratory system and skin. Harmful to aquatic organisms.

S: 26 36/37 61

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing and gloves. Avoid release to the

environment. Refer to special instructions/safety data sheets.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRAZOLE CARBAMIDINE HYDROCHLORIDE (1H:1-)    CAS # 4023023

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRENE    CAS # 129000

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   F   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   1   0

ACUTE TOXICTY RISK INDEX 5 - LD50 16000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50    170.0

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Readily absorbed through skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Blood. Liver. Kidneys.

SIGNS AND SYMPTOMS OF EXPOSURE

Inhalation studies in animals have caused hepatic, pulmonary,

and intragastric pathologic changes. Neutrophil, leukocyte, and

erythrocyte levels also decreased. Cutaneous application caused

hyperemia, weight loss, and hematopoietic changes and

dermatitis. Chronic effects: leukocytosis.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  392 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   CORROSIVE

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: N

Indication of Danger: Dangerous for the environment.

R: 50/53

Risk Statements: Very toxic to aquatic organisms, may cause

long-term adverse effects in the aquatic environment.

S: 60-61

Safety Statements: This material and its container must be

disposed of as hazardous waste. Avoid release to the

environment. Refer to special instructions/safety data sheets.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  2.5 mg/m3

DOE Short Term Exposure Limit  7.5 mg/m3

DOE Ceiling Limit 15 mg/m3

Immediately Dangerous to Life and Health   15 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRETHRINS    CAS # 8003347

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   J   K   L

NFPA HAZARD CODES (H,F,R,O)  3   2   0

ACUTE TOXICTY RISK INDEX 3.4 - LD50   200.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: Harmful if absorbed through skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. Harmful if inhaled.

Ingestion: Harmful if swallowed.

SENSITIZATION

Respiratory: May cause allergic respiratory reaction.

skin: May cause allergic skin reaction.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  167 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at 2-8°C

Keep tightly closed. Keep away from heat and open flame.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Keep away from heat and open flame. Store at

2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn N

Indication of Danger: Harmful. Dangerous for the environment.

R: 20/21/22 50/53

Risk Statements: Harmful by inhalation, in contact with skin and

if swallowed. Very toxic to aquatic organisms, may cause

long-term adverse effects in the aquatic environment.

S: 13 60 61

Safety Statements: Keep away from food, drink, and animal

feedingstuffs. This material and its container must be disposed

of as hazardous waste. Avoid release to the environment. Refer

to special instructions/safety data sheets.

Immediately Dangerous to Life and Health  5000 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDAZINE    CAS # 289805

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   2   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. May cause irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  185 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Water spray.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid breathing vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 23 24/25

Safety Statements: Do not breathe vapor. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDINE    CAS # 110861

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   E   F   G   .   I   J   K   .

NFPA HAZARD CODES (H,F,R,O)  3   3   0

ACUTE TOXICTY RISK INDEX 3 - LD50   891.0 mg/Kg

NEUROTOXIC - RISK INDEX   3.0

INHALATION HAZARD  INHALATION RISK INDEX   2.8 - LC50  10000.0

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: Harmful if absorbed through skin. Readily

absorbed through skin.

Eye Contact: Causes severe eye irritation.

Inhalation: Harmful if inhaled. Material is irritating to mucous

membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Bone marrow. Nerves. Liver. Kidneys.

SIGNS AND SYMPTOMS OF EXPOSURE

Dose-related side effects may include:anorexia, nausea,

vomiting, headache, dizziness, tachycarida, nervousness,

insomnia, skin disorders Symptoms of exposure may include

burning sensation, coughing, wheezing, laryngitis, shortness of

breath, headache, nausea, and vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

VAPOR PRESSURE   16.0 mm Hg @ 20 °C

FLASH POINT  62.6 °F

Forms ignitable mixtures in air at room temperature - Danger of remote

ignition and flashback

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

a - Organic Base/Flammable/Toxic

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC   CORROSIVE

INCOMPATIBILITIES:Strong acids, Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Carbon Dioxide, dry chemical powder, or appropriate   foam. Water can be

applied as a spray or fog and if properly applied is capable of extinguishing

the fire by

TOXIC EMISSIONS WHEN BURNED: and nitrogen oxides Cyanides

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep container closed. Keep

away from heat, sparks, and open flame. Packaged under nitrogen in SURE/SEAL

bottles.as.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: F-Xn

Indication of Danger: Highly Flammable. Harmful.

R: 11-20/21/22

Risk Statements: Highly flammable. Harmful by inhalation, in

contact with skin and if swallowed.

S: 26-28

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. After

contact with skin, wash immediately with plenty of soap-suds.

OSHA REGULATORY LIMITS

OSHA Permissible Exposure Limit 5 ppm

ACGIH RECOMMENDED LIMITS

ACGIH Threshold Limit Value 1 ppm

Immediately Dangerous to Life and Health  1000 ppm

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  5 ppm

DOE Short Term Exposure Limit  15 ppm

DOE Ceiling Limit 200 ppm

Immediately Dangerous to Life and Health   1000 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDINE BORONIC ACID (3-)    CAS # 1692257

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDINE BORONIC ACID (4-)    CAS # 1692155

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Borane/boron oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDINE HYDROCHLORIDE    CAS # 628137

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 2.5 - LD50  1600.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin. Causes eye and skin irritation.

TARGET ORGAN(S) OR SYSTEM(S)

Liver. Kidneys.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Avoid contact and inhalation. Do not get in eyes, on skin, on

clothing. STORAGE: Keep tightly closed. Store in a cool dry place\. SPECIAL

REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 20/21/22 36/37/38

Risk Statements: Harmful by inhalation, in contact with skin and

if swallowed. Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDINE OXIDE    CAS # 694597

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   1   0

ACUTE TOXICTY RISK INDEX 2.6 - LD50  1369.9 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  289.4 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place. Store under nitrogen\. SPECIAL REQUIREMENTS  Store under

inert gas. Very hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38   Risk Statements: Irritating to eyes, respiratory system and

skin.

S: 26

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDINE PROPIONIC ACID (4-)    CAS # 3724194

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDINEALDOXIME (4-)    CAS # 696548

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  230 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids, Acid chlorides, Acid

anhydrides.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

User Exposure: Avoid inhalation. Do not get in eyes, on skin, on

clothing. STORAGE: Keep tightly closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 20/22 36/37/38

Risk Statements: Harmful by inhalation and if swallowed.

Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDINEALDOXIME (SYN-2-)    CAS # 2110147

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 1.6 - LD50 12086.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 20/21/22-36/37/38

Risk Statements: Harmful by inhalation, in contact with skin and

if swallowed. Irritating to eyes, respiratory system and skin.

S: 26-36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDINEALDOXIME METHIODIDE (2-)    CAS # 94633

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

ACUTE TOXICTY RISK INDEX 2 - LD50  4838.7 mg/Kg

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled.

Ingestion: Harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Nerves.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases, Acid  chlorides,

Acid anhydrides.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen iodide

Store at 2-8°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22

Risk Statements: Harmful if swallowed.

S: 36

Safety Statements: Wear suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDINECARBOXAIDEHYDE N-OXIDE (4-)    CAS # 7216424

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDINECARBOXALDEHYDE (2-)    CAS # 1121604

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   2   0

ACUTE TOXICTY RISK INDEX 2.9 - LD50   585.0 mg/Kg

INHALATION RISK INDEX   2.0 - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Material is irritating to

mucous membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE .900 mm Hg @ 20 °C

FLASH POINT  129.2 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

Store at 2-8°C

Keep tightly closed. Keep away from heat and open flame. Store under

nitrogen.

Store under inert gas.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame. Store under nitrogen\. Store at 2-8░C SPECIAL

REQUIREMENTS  Store under inert gas.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 10 22 36/37/38

Risk Statements: Flammable. Harmful if swallowed. Irritating to

eyes, respiratory system and skin.

S: 7 26 36

Safety Statements: Keep container tightly closed. In case of

contact with eyes, rinse immediately with plenty of water and

seek medical advice. Wear suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDINECARBOXALDEHYDE (3-)    CAS # 500221

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   3   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material is irritating to

mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

FLASH POINT  95 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Water spray.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

Store at 2-8°C

Heat sensitive. Handle and store under inert gas. Store under inert gas.

Keep tightly closed. Keep away from heat and open flame. Store under

nitrogen.

REACTIVE PROPERTIES

HANDLING: Avoid breathing vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame. Store under nitrogen. Store at 2-8░C SPECIAL

REQUIREMENTS    Heat sensitive. Handle and store under inert gas. Store under

inert gas.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 10 37/38

Risk Statements: Flammable. Irritating to respiratory system and

skin.

S: 26 39

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDINECARBOXALDEHYDE (4-)    CAS # 872855

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   2   0

ACUTE TOXICTY RISK INDEX 2.5 - LD50  1600.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  129.2 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases, Strong acids, Strong

reducing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

Store at 2-8°C

Keep tightly closed. Keep away from heat and open flame. Store under

nitrogen.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Do not get in eyes, on skin, on   clothing.

STORAGE: Keep tightly closed. Keep away from heat and open flame. Store under

nitrogen\. Store at 2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 10 36/37/38

Risk Statements: Flammable. Irritating to eyes, respiratory

system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  12.5 mg/m3

DOE Short Term Exposure Limit  40 mg/m3

DOE Ceiling Limit 250 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDINEDICARBOXYLIC ACID (2,6-)    CAS # 499832

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  370.4 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Acids, Bases, Oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

User Exposure: Do not breathe dust. Avoid contact with eyes, skin, and

clothing. Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  10 mg/m3

DOE Short Term Exposure Limit  30 mg/m3

DOE Ceiling Limit 50 mg/m3

Immediately Dangerous to Life and Health   250 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDINESULFONIC ACID (3-)    CAS # 636737

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 3 - LD50   432.9 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: Harmful if inhaled or swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Acids, Bases, Oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

STORAGE: Keep tightly closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34

Risk Statements: Causes burns.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDINIUM CHLOROCHROMATE    CAS # 26299149

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   .   J   K   L

NFPA HAZARD CODES (H,F,R,O)  3   0   1   OX

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SENSITIZATION

skin: May cause allergic skin reaction.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Oxidizing Agent

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Avoid contact with metals., Strong acids,  Strong reducing

agents, Alcohols

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas Chromium

(VI) oxide

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.    Unsuitable: Do not store near, nor allow contact with,

clothing    and other combustible material\. SPECIAL REQUIREMENTS  Moisture

sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: O-T-N

Indication of Danger: Oxidizing. Toxic. Dangerous for the

environment.

R: 49-8-43-50/53

Risk Statements: May cause cancer by inhalation. Contact with

combustible material may cause fire. May cause sensitization by

skin contact. Very toxic to aquatic organisms, may cause

long-term adverse effects in the aquatic environment.

S: 53-45-60-61

Safety Statements: Restricted to professional users. Attention -

Avoid exposure - obtain special instructions before use. In case

of accident or if you feel unwell, seek medical advice

immediately (show the label where possible). This material and

its container must be disposed of as hazardous waste. Avoid

release to the environment. Refer to special instructions/safety

data sheets.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDINIUM DICHROMATE    CAS # 20039376   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   H   .   J   K   L

NFPA HAZARD CODES (H,F,R,O)  3   0   1   OX

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SENSITIZATION

skin: May cause allergic skin reaction.

TARGET ORGAN(S) OR SYSTEM(S)

Kidneys. Lungs.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Oxidizing Agent

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

e - Oxidizer/Organic Peroxide

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Organic materials, Finely powdered metals.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from combustible   materials, heat, sparks, and open flame.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: O-T-N

Indication of Danger: Oxidizing. Toxic. Dangerous for the

environment.

R: 49-8-43-50/53

Risk Statements: May cause cancer by inhalation. Contact with

combustible material may cause fire. May cause sensitization by

skin contact. Very toxic to aquatic organisms, may cause

long-term adverse effects in the aquatic environment.   S: 53-45-60-61

Safety Statements: Restricted to professional users. Attention -

Avoid exposure - obtain special instructions before use. In case

of accident or if you feel unwell, seek medical advice

immediately (show the label where possible). This material and

its container must be disposed of as hazardous waste. Avoid

release to the environment. Refer to special instructions/safety

data sheets.

Immediately Dangerous to Life and Health  15 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDINIUM TOLUENESULFONATE (PARA-)    CAS # 24057281

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxides

REACTIVE PROPERTIES

User Exposure: Do not breathe dust. Avoid contact with eyes, skin, and

clothing. Avoid prolonged or repeated exposure. STORAGE: Keep tightly

closed\. SPECIAL REQUIREMENTS  Store under inert gas. Moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDOXAL HYDROCHLORIDE    CAS # 65225

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 2.4 - LD50  2150.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen chloride gas Nitrogen  oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDOXAL PHOSPHATE MONOHYDRATE (5'-)    CAS # 41468251

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 2 - LD50  5900.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases, Strong acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Phosphorous oxides

Store at 2-8°C

Keep container closed.

Light sensitive.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep container closed\. Store

at 2-8°C SPECIAL REQUIREMENTS  Light sensitive.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDOXAMINE DIHYDROCHLORIDE    CAS # 524367

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 1.8 - LD50  7500.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: Causes eye and skin irritation. May be harmful

by inhalation, ingestion, or skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

Store at -20°C

Keep tightly closed. Store in a cool dry place.

REACTIVE PROPERTIES

HANDLING: Avoid contact and inhalation. Do not get in eyes, on skin, on

clothing. STORAGE: Keep tightly closed. Store in a cool dry place\. Store at

-20°C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26-36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDOXINE    CAS # 65236

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 2.1 - LD50  4000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

People who have consumed more than 2000 mg on a daily basis have

experienced a toxic sensory neuropathy. Withdrawal from 200 mg

daily has produced dependency symptoms. To the best of our knowledge, the

chemical, physical, and toxicological properties

have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26-36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDOXINE HYDROCHLORIDE    CAS # 58560

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 2.1 - LD50  4000.0 mg/Kg

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Overexposure by ingestion may result in nervousness, tremors,

and insomnia. To the best of our knowledge, the chemical,

physical, and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDYL BISPHENYLSULFONIC ACID MONOSODIUM    CAS # 69898459

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION HAZARD  INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause: Dermatitis. Pulmonary edema. Effects may be

delayed. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDYL PIPERAZINE (1:2-)    CAS # 34803662

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Vapor or mist is irritating to the eyes, mucous

membranes, and upper respiratory tract.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  230 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids, Acid  chlorides,

Acid anhydrides.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDYL SULFOFURYLTRIAZINE (BIS-)    CAS # 79551147

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxides Sodium oxide

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDYLACETIC ACID (4-)    CAS # 6622919

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Acids, Bases, Oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

STORAGE: Keep tightly closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 37/39

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable gloves and eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDYLACETIC ACID HYDROCHLORIDE (2-)    CAS # 16179978

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Acids, Bases, Oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

STORAGE: Keep tightly closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDYLACETIC ACID HYDROCHLORIDE (3-)    CAS # 6419369

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Acids, Bases, Oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

STORAGE: Keep tightly closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDYLACRYLIC ACID (TRANS,3:3-)    CAS # 19337974

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Acids, Bases, Oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at 2-8°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDYLAZONAPHTHOL (1,2,2-)    CAS # 85858

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong bases, Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDYLAZORESORCINOL    CAS # 1141599

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDYLOXIDE TERTBUTYLNITRONE    CAS # 66893810   HAZARDOUS CHEMICAL OF

CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

Store at 2-8°C

Keep tightly closed. Store in a cool dry place.

REACTIVE PROPERTIES

User Exposure: Avoid inhalation. Avoid contact with eyes, skin, and

clothing. Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

Store in a cool dry place\. Store at 2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22-24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIDYLPIPERAZINE (1:4-)    CAS # 1008919

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   0   1

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be harmful

if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Material is extremely destructive to tissue of the

mucous membranes and upper respiratory tract, eyes, and skin. To

the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34

Risk Statements: Causes burns.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRIMIDINE CARBOXALDEHYDE    CAS # 10070925

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   1

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

VAPOR PRESSURE .280 mm Hg @ 20 °C

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store

under nitrogen.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYROCATECHOL VIOLET    CAS # 115413

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Sulfur oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYROGALLOL    CAS # 87661

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   I   J   K   L

NFPA HAZARD CODES (H,F,R,O)  3   0   0

ACUTE TOXICTY RISK INDEX 3.4 - LD50   174.0 mg/Kg

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION HAZARD

INHALATION RISK INDEX   2.5 - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: Harmful if absorbed through skin.

Eye Contact: Causes burns.

Inhalation: Harmful if inhaled. Material is extremely

destructive to the tissue of the mucous membranes and upper

respiratory tract.

Ingestion: Harmful if swallowed.

SENSITIZATION

skin: May cause allergic skin reaction.

TARGET ORGAN(S) OR SYSTEM(S)

Blood. Liver. Kidneys.

SIGNS AND SYMPTOMS OF EXPOSURE

Studies in animals have shown edema and hyperemia of the lungs,

moderate fatty degeneration, round cell infiltration, and

necrosis of the liver. The kidney showed hyperemia, necrosis of

the epithelium, granular pigmentation, and glomerular nephritis.

The heart showed separation of fibers of the myocardium,

interfibrillary hemorrhages, infiltration of the endocardium,

lesions of the endothelium, and fibrinous deposits in the

valves. Changes in bone marrow and myeloid changes in the spleen

were observed. Changes in muscle included the disappearance of

nuclei in striated muscle, loss of striation, and the swelling

of the muscle plasma which coagulates and decomposes. Symptoms

of exposure may include burning sensation, coughing, wheezing,

laryngitis, shortness of breath, headache, nausea, and vomiting.

Absorption into the body leads to the formation of methemoglobin

which in sufficient concentration causes cyanosis. Onset may be

delayed 2 to 4 hours or longer.

CONDITIONS AGGRAVATED BY EXPOSURE

Pyrogallol has a tremendous affinity for oxygen of the blood

causing death by respiratory failure. Intoxication signs are

vomiting, hypothermia, weakness, muscular incoordination, fine

tremores, loss of reflexes, diarrhea, and coma.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

VAPOR PRESSURE    1.2 mm Hg @ 20 °C

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

a - Organic Base/Flammable/Toxic

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Materials to Avoid: O

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid breathing dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS Handle and store under inert gas. Air and light sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 20/21/22 68 52/53

Risk Statements: Harmful by inhalation, in contact with skin and

if swallowed. Possible risk of irreversible effects. Harmful to

aquatic organisms, may cause long-term adverse effects in the

aquatic environment.

S: 36/37 61

Safety Statements: Wear suitable protective clothing and gloves.

Avoid release to the environment. Refer to special

instructions/safety data sheets.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .75 mg/m3

DOE Short Term Exposure Limit  2.5 mg/m3

DOE Ceiling Limit 15 mg/m3

Immediately Dangerous to Life and Health   25 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRONIN B    CAS # 2150483

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause: Nausea, headache, and vomiting. To the best

of our knowledge, the chemical, physical, and toxicological

properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Do not get in eyes, on skin, on   clothing.

STORAGE: Keep tightly closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRONIN Y    CAS # 92320

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYROPHOSPHATE TETRASODIUM    CAS # 7722885

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 2.1 - LD50  3770.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Ingestion can cause: Nausea, vomiting, diarrhea. To the best of

our knowledge, the chemical, physical, and toxicological

properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Use extinguishing media appropriate to surrounding fire    conditions.

TOXIC EMISSIONS WHEN BURNED: Sodium/sodium oxides  Phosphorous oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  5 mg/m3

DOE Short Term Exposure Limit  15 mg/m3

DOE Ceiling Limit 25 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRROLE    CAS # 109977   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   F   G   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   2   2

ACUTE TOXICTY RISK INDEX 3.6 - LD50   137.0 mg/Kg

INHALATION HAZARD

INHALATION RISK INDEX   3.1 - LC50   2270.0

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes severe eye irritation.

Inhalation: Harmful if inhaled. Material may be irritating to

mucous membranes and upper respiratory tract.

Ingestion: Toxic if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Liver. Heart. Kidneys.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

Explosive

VAPOR PRESSURE    6.0 mm Hg @ 20 °C

FLASH POINT  96.8 °F

Forms explosive unstable peroxides

Test periodically or discard

SEGREGATION: SHELF # 3

STORAGE GROUP(S):

a - Organic Base/Flammable/Toxic

k - Explosive/Unstable

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   REACTIVE   TOXIC   CORROSIVE

INCOMPATIBILITIES:Avoid contact with acid., Acid chlorides, Acid anhydrides,

Oxidizing agents

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam

TOXIC EMISSIONS WHEN BURNED: Thermal decomposition ma produce  and nitrogen

oxides

Store at 2-8°C

Keep tightly closed. Keep away from heat, sparks, and open flame. Store under

nitrogen.

Store under inert gas. Air, light, and moisture sensitive.

REACTIVE PROPERTIES

HANDLING: Avoid contact with eyes, skin, and clothing. Do   not breathe

vapor. Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

Keep away from heat, sparks, and open flame. Store under nitrogen\. Store at

2-8░C SPECIAL REQUIREMENTS  Store under inert gas. Air, light, and moisture

sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: T

Indication of Danger: Toxic.

R: 10 20 25 41

Risk Statements: Flammable. Harmful by inhalation. Toxic if

swallowed. Risk of serious damage to eyes.

S: 26 37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable gloves and eye/face protection. In case of accident or

if you feel unwell, seek medical advice immediately (show the

label where possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .035 ppm

DOE Short Term Exposure Limit  .1 ppm

DOE Ceiling Limit .75 ppm

Immediately Dangerous to Life and Health   4 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRROLE BORONIC ACID (1-N-BOC:2-)    CAS # 135884310

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRROLE CARBOXYALDEHYDE    CAS # 1003298

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   1   1

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  224 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong reducing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at 2-8°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. Store at

2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRROLECARBOXYLIC ACID (2-)    CAS # 634979

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

Store at 2-8°C

Keep tightly closed. Store in a cool dry place. Store under nitrogen.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Store in a cool dry place. Store under

nitrogen. Store at 2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRROLIDINE    CAS # 119237648

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  235 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place. Store under nitrogen.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRROLIDINE    CAS # 123751

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   .   .   G   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  2   3   0

ACUTE TOXICTY RISK INDEX 5 - LD50   433.0 mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   3.4 - LC50    403.0

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns. Lachrymator.

Inhalation: Harmful if inhaled. Material is extremely

destructive to the tissue of the mucous membranes and upper

respiratory tract.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Nausea, dizziness, and headache. Exposure can cause: Symptoms of

exposure may include burning sensation, coughing, wheezing,

laryngitis, shortness of breath, headache, nausea, and vomiting.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Flammable

VAPOR PRESSURE   48.8 mm Hg @ 20 °C

FLASH POINT  37.4 °F

Store in an explosion-proof refrigerator or tightly stoppered in a

well-ventilated area

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

a - Organic Base/Flammable/Toxic

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

INCOMPATIBILITIES:Acids, Acid chlorides, Acid anhydrides, Strong oxidizing

agents Carbon dioxide.

FIRE EXTINGUISHER:  For small (incipient) fires, use media such as"alcohol"

foam, dry chemical, or carbon dioxide. For large   fires, apply water from as

far as possible. Use very large   quantities (flooding) of water applied as a

mist or spray; solid

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep container closed. Keep

away from heat, sparks, and open flame. Store in a cool dry place. Handle and

Store under nitrogen\. SPECIAL REQUIREMENTS Handle and store under inert gas.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: F-C

Indication of Danger: Highly Flammable. Corrosive.

R: 11-20/22-35

Risk Statements: Highly flammable. Harmful by inhalation and if

swallowed. Causes severe burns.

S: 16-26-36/37/39-45

Safety Statements: Keep away from sources of ignition - no

smoking. In case of contact with eyes, rinse immediately with

plenty of water and seek medical advice. Wear suitable

protective clothing, gloves, and eye/face protection. In case of

accident or if you feel unwell, seek medical advice immediately

(show the label where possible).

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  5 mg/m3

DOE Short Term Exposure Limit  15 mg/m3

DOE Ceiling Limit 100 mg/m3

Immediately Dangerous to Life and Health   500 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRROLIDINE METHANOL (2-)    CAS # 23356969

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   2   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  186.8 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Acid chlorides, Acid anhydrides.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

Store at 2-8°C

Air sensitive.

Keep tightly closed. Keep away from heat and open flame. Store in a cool dry

place.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Do not get in eyes, on skin, on   clothing.

STORAGE: Keep tightly closed. Keep away from heat and open flame. Store in a

cool dry place\. Store at 2-8°C SPECIAL REQUIREMENTS  Air sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRROLIDINECARBODITHIOIC ACID (1-)    CAS # 5108963

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases, Strong acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Sulfur oxides

Store at 2-8°C

Air and light sensitive. Handle and store under inert gas.

Keep tightly closed. Store in a cool dry place. Handle and store under

nitrogen.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place. Handle   and store under nitrogen\. Store at 2-8░C SPECIAL

REQUIREMENTS  Air and light sensitive. Handle and store under inert gas.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRROLIDINOCYCLOPENTENE    CAS # 7148074

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   2   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Vapor or mist is irritating to the eyes, mucous

membranes, and upper respiratory tract.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  116 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Water spray.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

Store at 2-8°C

Keep tightly closed. Keep away from heat and open flame.

REACTIVE PROPERTIES

HANDLING: Avoid breathing vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame\. Store at 2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 10 36/37/38

Risk Statements: Flammable. Irritating to eyes, respiratory

system and skin.

S: 16 26 36/37/39

Safety Statements: Keep away from sources of ignition - no

smoking. In case of contact with eyes, rinse immediately with

plenty of water and seek medical advice. Wear suitable

protective clothing, gloves, and eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRROLIDINONE (2-)    CAS # 616455

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   1   0

ACUTE TOXICTY RISK INDEX 4.8 - LD50  5000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  235 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids, Strong  bases,

Strong reducing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Hygroscopic.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  1 ppm

DOE Short Term Exposure Limit  3 ppm

DOE Ceiling Limit 20 ppm

Immediately Dangerous to Life and Health   40 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRROLIDINYLMETHYL PYRROLIDINE (1,2-)    CAS # 51207660

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   2   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Vapor or mist is irritating to the eyes, mucous

membranes, and upper respiratory tract.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  190 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Keep away from heat and open flame. Store in a

cool dry place.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRROLIDONE CARBOXYLIC ACID (L,2:5-)    CAS # 98793

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRROLINE (3-)    CAS # 109966

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   3   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns. Lachrymator.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be harmful

if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

FLASH POINT  -.4 °F

Store in an explosion-proof refrigerator or tightly stoppered in a

well-ventilated area

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat, sparks, and open flame\. SPECIAL REQUIREMENTS  Air sensitive.

Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: F C

Indication of Danger: Highly Flammable. Corrosive.

R: 11 34

Risk Statements: Highly flammable. Causes burns.

S: 16 26 36/37/39 45

Safety Statements: Keep away from sources of ignition - no

smoking. In case of contact with eyes, rinse immediately with

plenty of water and seek medical advice. Wear suitable

protective clothing, gloves, and eye/face protection. In case of

accident or if you feel unwell, seek medical advice immediately

(show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRROLINECARBOXYLIC ACID (5-)    CAS # 108321375

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   F   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes irritation.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   CORROSIVE

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

Store at 2-8°C

REACTIVE PROPERTIES

STORAGE Store at 2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRROLYL BENZOIC ACID (4:1H:1-)    CAS # 22106338

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 37/39

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable gloves and eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRUVIC ACID    CAS # 127173

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   2   0

ACUTE TOXICTY RISK INDEX 2.1 - LD50  3533.0 mg/Kg

INHALATION RISK INDEX   1.4 - LC50

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: Material is extremely destructive to the tissue of

if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting.

Subcutaneous

Mouse

3533 MG/KG

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

VAPOR PRESSURE .898 mm Hg @ 20 °C

FLASH POINT  179.6 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

d - Organic Acid/Flammable/Toxic

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Bases, Oxidizing agents, Reducing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate

foam\. Protective Equipment: Wear self-contained breathing apparatus and

protective clothing to prevent contact with skin and eyes.    Specific

Hazard(s): Emits toxic fumes un

Store at 2-8°C

Store under inert gas.

Keep tightly closed. Keep away from heat and open flame. Store under

nitrogen.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on STORAGE: Keep tightly

closed. Keep away from heat and open flame. Store under nitrogen. Store at

2-8░CSPECIAL REQUIREMENTS  Store under inert gas.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34

Risk Statements: Causes burns.

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# PYRUVIC ALDEHYDE DIMETHYL ACETAL    CAS # 6342569

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   3   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. May cause irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

-

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

VAPOR PRESSURE    8.5 mm Hg @ 20 °C

FLASH POINT  78.8 °F

Forms ignitable mixtures in air at room temperature - Danger of remote

ignition and flashback

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE

INCOMPATIBILITIES:Acids, Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: -

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep container closed. Keep away from heat and open flame. Store in

a cool dry place.    -

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 10 36/38

Risk Statements: Flammable. Irritating to eyes and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.