# VAL-TYR-VAL    CAS # 17355225

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

Store at -20°C

REACTIVE PROPERTIES

STORAGE   Store at -20°C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VALERALDEHYDE    CAS # 110623

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   F   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   3   0

ACUTE TOXICTY RISK INDEX 1.9 - LD50  5660.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes severe eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

FLASH POINT  39.2 °F

Store in an explosion-proof refrigerator or tightly stoppered in a

well-ventilated area

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   CORROSIVE

INCOMPATIBILITIES:Oxidizing agents, Strong bases, Strong  reducing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid breathing vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep container closed. Keep

away from heat, sparks, and open flame. Store in a cool dry place. Store

under nitrogen.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: F Xi

Indication of Danger: Highly Flammable. Irritant.

R: 11 38 41

Risk Statements: Highly flammable. Irritating to skin. Risk of

serious damage to eyes.

S: 16 26 33 39

Safety Statements: Keep away from sources of ignition - no

smoking. In case of contact with eyes, rinse immediately with

plenty of water and seek medical advice. Take precautionary

measures against static discharges. Wear eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VALERIC ACID (N-)    CAS # 109524

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   1   0

ACUTE TOXICTY RISK INDEX 1.9 - LD50  1935.5 mg/Kg

INHALATION RISK INDEX   1.0 - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: Harmful if inhaled. Material is extremely

destructive to the tissue of the mucous membranes and upper

respiratory tract.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause: Nausea, headache, and vomiting. To the best

of our knowledge, the chemical, physical, and toxicological

properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE .150 mm Hg @ 20 °C

FLASH POINT  204.8 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VALEROLACTAM (D-)    CAS # 675207

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  235 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VALEROLACTONE GRADE (DELTA-)    CAS # 542289

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  234 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at -20°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

-20░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 41

Risk Statements: Risk of serious damage to eyes.

S: 26 39

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VALEROYL CHLORIDE    CAS # 638299   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   .   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   3   1   W

INHALATION HAZARD  INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin. Causes burns.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. To the best of our knowledge, the chemical, physical,

and toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

FLASH POINT  89.6 °F

Forms ignitable mixtures in air at room temperature - Danger of remote

ignition and flashback

Reacts violently with water

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   REACTIVE

INCOMPATIBILITIES:Water, Alcohols, Oxidizing agents, Strong bases.

FIRE EXTINGUISHER:  Carbon dioxide. Dry chemical powder. Do not use water.

TOXIC EMISSIONS WHEN BURNED: Hydrogen chloride gas Phosgene gas

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat, sparks, and open flame. Store in a cool dry place. Store under

nitrogen\. SPECIAL REQUIREMENTS  Readily hydrolyzed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 10 20 34

Risk Statements: Flammable. Harmful by inhalation. Causes burns.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VALINE (BOC,TERT,L-)    CAS # 13734413

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

Store at -20°C

Keep tightly closed. Store in a cool dry place.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place\. Store at -20░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VALINE (DL-)    CAS # 516063

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

The levorotary (l) forms of leucine, isoleucine, and valine have

carcinomas. Science 231, 843, 21 February 1986.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure.     Suitable: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VALINE (FMOC,L-)    CAS # 68858208

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VALINE (L-)    CAS # 72184

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

The levorotary (l) forms of leucine, isoleucine, and valine have

been found to have tumor-promoting activity for bladder

carcinomas. Science 231, 843, 21 February 1986. To the best of

our knowledge, the chemical, physical, and toxicological

properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

d - Organic Acid/Flammable/Toxic

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VALINE BENZYL ESTER HYDROCHLORIDE (L-)    CAS # 2462342

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at -20°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

-20░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VALINE ETHYL ESTER HYDROCHLORIDE (L-)    CAS # 17609471

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

Store at -20°C

Keep tightly closed. Store in a cool dry place.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place. Store at -20░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VALINE METHYL ESTER HYDROCHLORIDE (L-)    CAS # 6306521

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VALPROIC ACID SODIUM    CAS # 1069665

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   I   J   K   .

NFPA HAZARD CODES (H,F,R,O)  3   0   1

ACUTE TOXICTY RISK INDEX 2.9 - LD50   670.0 mg/Kg

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: Harmful if absorbed through skin.

Eye Contact: Causes eye irritation.

Inhalation: Harmful if inhaled. Material is irritating to mucous

membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Central nervous system. Liver.

SIGNS AND SYMPTOMS OF EXPOSURE

Other symptoms of exposure can include drowsiness, weakness,

lethargy, jaundice, skin rash, and transient alopecia with

regrowth of curly hair. Exposure can cause: Gastrointestinal

disturbances. Ataxia.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: T

Indication of Danger: Toxic.

R: 61 20/21/22 36/37/38

Risk Statements: May cause harm to the unborn child. Harmful by

inhalation, in contact with skin and if swallowed. Irritating to

eyes, respiratory system and skin.

S: 22 45 36/37/39

Safety Statements: Do not breathe dust. In case of accident or

if you feel unwell, seek medical advice immediately (show the

label where possible). Wear suitable protective clothing,

gloves, and eye/face protection. Keep container tightly closed

in a cool well-ventilated place.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VANADIUM    CAS # 1214621

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   C   .   .   .   .   .   I   J   K   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

EXTREMELY TOXIC - DESIGNATED AREA MAY BE REQUIRED

ACUTE TOXICTY RISK INDEX 4.7 - LD50    10.0 mg/Kg

NEUROTOXIC - RISK INDEX   4.0

INHALATION RISK INDEX   <1  - LC50

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:   TOXIC

FIRE EXTINGUISHER: NONCOMBUSTIBLE.    USE EXTINGUISHING MEDIA APPROPRIATE TO

SURROUNDING FIRE CONDITIONS.

REACTIVE PROPERTIES

REACTS VIOLENTLY WITH BROMINE PENTAFLUORIDE. CATALYZE THE DECOMPOSITION

OFPERFORMIC ACID, RESULTING IN AN EXPLOSION. A MIXTURE OF CALCIUM AND

VANADIUMPENTOXIDE THAT WAS CONTAMINATED WITH SULFUR AND SOME MOISTURE CAUSED

A SEVEREFIRE. REACTS VIOLENTLY PRODUCING FLAME WITH CHLORINE TRIFLUORIDE.

REACTSWITH LITHIUM AT 400C RISING RAPIDLY TO 760C.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VANADIUM (V) OXIDE    CAS # 1314621

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   C   D   .   F   G   .   I   J   K   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

EXTREMELY TOXIC - DESIGNATED AREA MAY BE REQUIRED

ACUTE TOXICTY RISK INDEX 5 - LD50    10.0 mg/Kg

NEUROTOXIC - RISK INDEX   4.0

INHALATION HAZARD  INHALATION RISK INDEX   4.0 - LC50    126.0

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

Vanadium compounds can cause metallic taste and a greenish

discoloration of the tongue. To the best of our knowledge, the

chemical, physical, and toxicological properties have not been

thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

VAPOR PRESSURE    8.0 mm Hg @ 20 °C

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC   CORROSIVE

INCOMPATIBILITIES:Strong acids, Strong oxidizing agents.

FIRE EXTINGUISHER:  Noncombustible. Use extinguishing media appropriate to

surrounding fire conditions.

TOXIC EMISSIONS WHEN BURNED: Vanadium/vanadium oxides

REACTIVE PROPERTIES

HANDLING: Avoid contact with eyes, skin, and clothing. STORAGE: Keep

container closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T N

Indication of Danger: Toxic. Dangerous for the environment.

R: 20/22 37 48/23 51/53 63 68

Risk Statements: Harmful by inhalation and if swallowed.

Irritating to respiratory system. Toxic: danger of serious

damage to health by prolonged exposure through inhalation. Toxic

to aquatic organisms, may cause long-term adverse effects in the

aquatic environment. Possible risk of harm to the unborn child.

Possible risk of irreversible effects.

S: 36/37 38 45 61

Safety Statements: Wear suitable protective clothing and gloves.

In case of insufficient ventilation, wear suitable respiratory

equipment. In case of accident or if you feel unwell, seek

medical advice immediately (show the label where possible).

Avoid release to the environment. Refer to special

instructions/safety data sheets.

OSHA REGULATORY LIMITS

OSHA Permissible Exposure Limit .05 mg/m3

ACGIH RECOMMENDED LIMITS

ACGIH Threshold Limit Value .05 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VANADIUM (V) OXIDE    CAS # 7440622

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   C   D   .   .   .   .   I   J   K   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

EXTREMELY TOXIC - DESIGNATED AREA MAY BE REQUIRED

ACUTE TOXICTY RISK INDEX 4.7 - LD50    10.0 mg/Kg

NEUROTOXIC - RISK INDEX   4.0

INHALATION HAZARD  INHALATION RISK INDEX   4.0 - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

Vanadium compounds can cause metallic taste and a greenish

discoloration of the tongue. To the best of our knowledge, the

chemical, physical, and toxicological properties have not been

thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

VAPOR PRESSURE    8.0 mm Hg @ 20 °C

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong acids, Strong oxidizing agents.

FIRE EXTINGUISHER:  Noncombustible. Use extinguishing media appropriate to

surrounding fire conditions.

TOXIC EMISSIONS WHEN BURNED: Vanadium/vanadium oxides

REACTIVE PROPERTIES

HANDLING: Avoid contact with eyes, skin, and clothing. STORAGE: Keep

container closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

Immediately Dangerous to Life and Health  35 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VANADIUM BORIDE    CAS # 12007373

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 20/21/22

Risk Statements: Harmful by inhalation, in contact with skin and

if swallowed.

S: 36

Safety Statements: Wear suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VANADIUM(III) CHLORIDE    CAS # 7718981

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   G   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  3   0   0

ACUTE TOXICTY RISK INDEX 3.2 - LD50   350.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: May be harmful if inhaled. Material is extremely

destructive to the tissue of the mucous membranes and upper

respiratory tract.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, an

vomiting. Material is extremely destructive to tissue of the

mucous membranes and upper respiratory tract, eyes, and skin. To

the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

TOXIC EMISSIONS WHEN BURNED: Hydrogen chloride gas

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Light sensitive. Air and moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34 22

Risk Statements: Causes burns. Harmful if swallowed.

S: 26 27 28 45 36/39

Safety Statements: In case of contact with eyes, rins   immediately with

plenty of water and seek medical advice. Take

off immediately all contaminated clothing. After contact with

skin, wash immediately with plenty of water. In case of accident

or if you feel unwell, seek medical advice immediately (show the

label where possible). Wear suitable protective clothing and

eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VANADIUM(III) OXIDE    CAS # 1314347

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   F   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 3 - LD50  5639.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50   4000.0

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Harmful if inhaled. Material is irritating to mucous

membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Severe overexposure may be evidenced by a greenish-black   discoloration

of the tongue. To the best of our knowledge, the

chemical, physical, and toxicological properties have not been

thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC   CORROSIVE

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Air sensitive. Handle and store under inert gas.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 20 36/37/38

Risk Statements: Harmful by inhalation. Irritating to eyes,

respiratory system and skin.

S: 26

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VANADIUM(IV) CHLORIDE    CAS # 7632511   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  3   0   1   W

ACUTE TOXICTY RISK INDEX 3.5 - LD50   160.0 mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   3.1 - LC50

ROUTE OF EXPOSURE

Multiple Routes: May be fatal if inhaled, swallowed, or absorbed

through skin. Causes burns.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

VAPOR PRESSURE    7.6 mm Hg @ 20 °C

Reacts violently with water

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

b - Pyrophoric/Water Reactive

WASTE CHARACTERISTIC HAZARD:   REACTIVE   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Water.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place. Handle   and store under nitrogen\. Incompatible Materials:

Do not allow contact with water

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: T

Indication of Danger: Toxic.

R: 14 23/24/25 34

Risk Statements: Reacts violently with water. Toxic by

inhalation, in contact with skin and if swallowed. Causes burns.

S: 26 27 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Take

off immediately all contaminated clothing. Wear suitable

protective clothing, gloves, and eye/face protection. In case of

accident or if you feel unwell, seek medical advice immediately

(show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VANADIUM(IV) OXIDE SULFATE HYDRATE    CAS # 12439962

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   J   K   L

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 3 - LD50   448.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

SENSITIZATION

Sensitization: Prolonged or repeated exposure may cause allergic

reactions in certain sensitive individuals.

TARGET ORGAN(S) OR SYSTEM(S)

Blood. Lungs. Brain.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Sulfur oxides Vanadium/vanadium oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.   R: 22

Risk Statements: Harmful if swallowed.

S: 36

Safety Statements: Wear suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VANADYL (IV) ACETYLACETONATE    CAS # 3153262

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: Harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Nerves.

SIGNS AND SYMPTOMS OF EXPOSURE

May liberate 2,4-pentanedione upon decomposition.

2,4-Pentanedione has the following toxicological hazards: toxic,

irritant, neurological hazard, teratogen, possible mutagen,

target organ - thymus. In humans, 2,4-pentanedione is reported

to cause contact dermatitis and contact urticaria. To the best

of our knowledge, the chemical, physical, and toxicological

properties have not been thoroughly investigated.

CONDITIONS AGGRAVATED BY EXPOSURE

May cause nervous system disturbances.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Vanadium/vanadium oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Air sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22 36/37/38

Risk Statements: Harmful if swallowed. Irritating to eyes,

respiratory system and skin.

S: 22 26 36

Safety Statements: Do not breathe dust. In case of contact with

eyes, rinse immediately with plenty of water and seek medical

advice. Wear suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VANADYL PHTHALOCYANINE    CAS # 13930886

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VANADYL SULFATE    CAS # 123334203

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   J   K   L

NFPA HAZARD CODES (H,F,R,O)  2   0   0

ACUTE TOXICTY RISK INDEX 3 - LD50   448.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

SENSITIZATION

Sensitization: Prolonged or repeated exposure may cause allergic

reactions in certain sensitive individuals.

TARGET ORGAN(S) OR SYSTEM(S)

Blood. Lungs. Brain.

SIGNS AND SYMPTOMS OF EXPOSURE

Vanadyl Sulfate Hydrate (VOSO4.xH2O) is a mixture of hydrates

from Vanadyl Sulfate trihydrate to Vanadyl Sulfate pentahydrate

(VOSO4.5H2O, CAS# 12439-96-2). Vanadyl Sulfate pentahydrate is

toxic by ingestion (LD50 Rat Oral = 448mg/kg). Vanadyl Sulfate

Hydrate should be handled as if it possesses the same hazards.

Vanadium compounds can cause metallic taste and a greenish

discoloration of the tongue. To the best of our knowledge, the

chemical, physical, and toxicological properties have not been

thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Sulfur oxides Vanadium/vanadium oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Store under inert gas. Keep

tightly closed. Store in a   cool dry place\. SPECIAL REQUIREMENTS Handle and

store under inert gas. Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22

Risk Statements: Harmful if swallowed.

S: 36

Safety Statements: Wear suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VANCOMYCIN    CAS # 1404939

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  3   0   0

ACUTE TOXICTY RISK INDEX 1.7 - LD50 10000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SENSITIZATION

skin: May cause allergic skin reaction.

TARGET ORGAN(S) OR SYSTEM(S)

Kidneys. Ears.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

Store at 2-8°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid breathing dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 43

Risk Statements: May cause sensitization by skin contact.

S: 36/37

Safety Statements: Wear suitable protective clothing and gloves.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VANCOMYCIN HYDROCHLORIDE    CAS # 123409007

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 1.7 - LD50 10000.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SENSITIZATION

skin: May cause allergic skin reaction.

TARGET ORGAN(S) OR SYSTEM(S)

Kidneys. Ears.

SIGNS AND SYMPTOMS OF EXPOSURE

May cause gastrointestinal pain, vomiting, flushing of face and

ears, mild hypotension, rash, tinnitus, nerve deafness.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Hydrogen chloride gas

Store at 2-8°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 43

Risk Statements: May cause sensitization by skin contact.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VANILLIC ACID    CAS # 121346

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VANILLIN    CAS # 121335

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   0   0

ACUTE TOXICTY RISK INDEX 2.5 - LD50  1580.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract. May be harmful if inhaled.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

VAPOR PRESSURE .010000 mm Hg @ 20 °C

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong bases, Strong reducing agents,  Perchloric acid,

Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. SPECIAL

REQUIREMENTS  Air, light, and moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22

Risk Statements: Harmful if swallowed.

S: 22-24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VANILLIN (O-)    CAS # 148538

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

ACUTE TOXICTY RISK INDEX 2.1 - LD50  4290.3 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Absorption: May be harmful if absorbed through the skin.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

Multiple Routes: Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  235 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong reducing  agents, Strong

bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22 36/37/38

Risk Statements: Harmful if swallowed. Irritating to eyes,

respiratory system and skin.

S: 23 36

Safety Statements: Do not breathe vapor. Wear suitable

protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VANILLIN AZINE    CAS # 1696602

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 2.4 - LD50  1720.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

Store at 2-8°C

Keep tightly closed. Store in a cool dry place.

REACTIVE PROPERTIES

HANDLING: Avoid contact and inhalation. Do not get in eyes, on skin, on

clothing. STORAGE: Keep tightly closed. Store in a cool dry place. Store at

2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VASCULAR ENDOTHELIAL GROWTH FACTOR    CAS # 127464602

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong acids, Strong bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at -20°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

-20░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VEGETABLE OIL    CAS # 8001227

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  0   1   0

ACUTE TOXICTY RISK INDEX 1.5 - LD50 16500.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Inhalation: Material may be irritating to mucous membranes and

upper respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption.

SENSITIZATION

Respiratory: May cause allergic respiratory reaction.

CONDITIONS AGGRAVATED BY EXPOSURE

The toxicological properties have not been thoroughly

investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  235 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VERAPAMIL HYDROCHLORIDE (-)    CAS # 152114

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   E   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   0   0

INHALATION HAZARD  INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: Toxic if absorbed through skin.

Eye Contact: May cause eye irritation.

Inhalation: Toxic if inhaled. Material may be irritating to

mucous membranes and upper respiratory tract.

Ingestion: Toxic if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Cardiovascular system.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

Exposure can cause: Nausea, dizziness, and headache. Other

symptoms of exposure can include flushing rash, constipation,

bradycardia, hypotension, and atrioventricular block.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen chloride gas Nitrogen  oxides

Store at 2-8°C

Heat and light sensitive.

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

2-8░C SPECIAL REQUIREMENTS    Heat and light sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: T

Indication of Danger: Toxic.

R: 23/24/25

Risk Statements: Toxic by inhalation, in contact with skin and

if swallowed.

S: 36/37-45

Safety Statements: Wear suitable protective clothing and gloves.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VERATROLE    CAS # 91167

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   J   K   .

NFPA HAZARD CODES (H,F,R,O)  1   2   0

ACUTE TOXICTY RISK INDEX 2.7 - LD50   890.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled.

Ingestion: Harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  161.6 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Water spray.

REACTIVE PROPERTIES

HANDLING: Avoid prolonged or repeated exposure. Do not breathe vapor. Avoid

contact with eyes, skin, and clothing. STORAGE: Keep tightly closed. Keep

away from heat and open flame. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 22

Risk Statements: Harmful if swallowed.

S: 36

Safety Statements: Wear suitable protective clothing.

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  3.5 mg/m3

DOE Short Term Exposure Limit  10 mg/m3

DOE Ceiling Limit 75 mg/m3

Immediately Dangerous to Life and Health   400 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VINYL ACETATE    CAS # 108054   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   .   F   G   H   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   3   2

ACUTE TOXICTY RISK INDEX 2.2 - LD50  2900.0 mg/Kg

NEUROTOXIC - RISK INDEX   4.0

INHALATION HAZARD  INHALATION RISK INDEX   3.0 - LC50  21786.0

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Harmful if inhaled. Material is irritating to mucous

membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Central nervous system. Liver. Cardiovascular system.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

Explosive

VAPOR PRESSURE   89.6 mm Hg @ 20 °C

FLASH POINT  17.6 °F

Store in an explosion-proof refrigerator or tightly stoppered in a

well-ventilated area

Forms explosive unstable peroxides

Test periodically or discard

SEGREGATION: SHELF # 3

STORAGE GROUP(S):

k - Explosive/Unstable

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   REACTIVE   TOXIC   CORROSIVE

INCOMPATIBILITIES:Acids, Bases, Oxidizing agents, Peroxides.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat, sparks, and open flame.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: F

Indication of Danger: Highly Flammable.

R: 11

Risk Statements: Highly flammable.

S: 16 23 29 33

Safety Statements: Keep away from sources of ignition - no

smoking. Do not breathe vapor. Do not empty into drains. Take

precautionary measures against static discharges.

OSHA REGULATORY LIMITS

OSHA Permissible Exposure Limit 10 ppm

OSHA Short Term Exposure Limit  20 ppm

ACGIH RECOMMENDED LIMITS

ACGIH Threshold Limit Value 10 ppm

ACGIH Short Term Exposure Limit 15 mg/m3

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VINYL BROMIDE    CAS # 593602   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    A   .   .   D   .   .   .   .   I   .   K   .

NFPA HAZARD CODES (H,F,R,O)  2   4   0

SPECIAL CARCINOGEN - DESIGNATED AREA MAY BE REQUIRED

CHRONIC TOXICITY RISK INDEX 3.4 - TD50    18.5 mg/Kg

ACUTE TOXICTY RISK INDEX 3 - LD50   500.0 mg/Kg

NEUROTOXIC - RISK INDEX   3.0

INHALATION HAZARD  INHALATION RISK INDEX   4.3 - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: May be harmful if inhaled. Can cause rapid

suffocation. Material is irritating to mucous membranes and

upper respiratory tract.

Ingestion: Harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Liver.

SIGNS AND SYMPTOMS OF EXPOSURE

Inhalation of vapors may cause dizziness, an irregular

heartbeat, narcosis, nausea, or asphyxiation.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Gas

Flammable

VAPOR PRESSURE  820.6 mm Hg @ 20 °C

SEGREGATION: SHELF # 3

STORAGE GROUP(S):

h - Flammable/Pyrophoric Gas

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Peroxides Copper,  Copper alloys,

Plastics.

FIRE EXTINGUISHER:  Use water spray or fog nozzle to keep cylinder cool. Move

cylinder away from fire if there is no risk.

TOXIC EMISSIONS WHEN BURNED: Hydrogen bromide gas

Store at 2-8°C

Contents under pressure. Keep away from heat, spaks or open flame.

Close valve when not in use and when empty. Make sure cylinder is properly

secured when in use or st

REACTIVE PROPERTIES

HANDLING: Do not breathe gas. Do not get in eyes, on skin, on clothing. Avoid

prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away from

heat, sparks, and open flame. Use with equipment rated for cylinder pressure,

and    of compatible materials of construction. Close valve when not in

use and when empty. Make sure cylinder is properly secured when    in use or

stored Cylinder temperature should not exceed 125░F    (52░C).    Unsuitable:

Store away from heat and direct sunlight Store at 2-8░C SPECIAL REQUIREMENTS

Contents under pressure.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: F+ T

Indication of Danger: Extremely Flammable. Toxic.

R: 45 12

Risk Statements: May cause cancer. Extremely flammable.

S: 53 45

Safety Statements: Restricted to professional users. Attention -

Avoid exposure - obtain special instructions before use. In case

of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VINYL CHLOROFORMATE    CAS # 5130245

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   E   .   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   3   0

INHALATION HAZARD  INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be fatal if absorbed through skin.

Eye Contact: Lachrymator.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be toxic

by inhalation.

Ingestion: May be fatal if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting. Inhalation may result in spasm, inflammation and edema

of the larynxand bronchi, chemical pneumonitis, and pulmonary

edema. Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Flammable

VAPOR PRESSURE   45.5 mm Hg @ 20 °C

FLASH POINT  24.8 °F

Store in an explosion-proof refrigerator or tightly stoppered in a

well-ventilated area

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

INCOMPATIBILITIES:Amines, Alcohols, Strong bases, Acids.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Do not use water.

TOXIC EMISSIONS WHEN BURNED: Phosgene gasHydrogen chloride gas

Store at 2-8°C

Keep tightly closed. Keep away from heat, sparks, and open flame. Refrigerate

on arrival.

Vent periodically. Light and moisture sensitive.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. Open    carefully. STORAGE: Keep

tightly closed. Keep away from heat, sparks, and open flame. Refrigerate on

arrival. Store at 2-8░C SPECIAL REQUIREMENTS    Vent periodically. Light and

moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: F-T

Indication of Danger: Highly Flammable. Toxic.

R: 23/24/25-34

Risk Statements: Toxic by inhalation, in contact with skin and

if swallowed. Causes burns.

S: 16-26-36

Safety Statements: Keep away from sources of ignition - no

smoking. In case of contact with eyes, rinse immediately with

plenty of water and seek medical advice. Wear suitable

protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VINYL METHYL ETHER    CAS # 107255   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   .   F   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   4   2

ACUTE TOXICTY RISK INDEX 2 - LD50  4900.0 mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   3.7 - LC50 ********

ROUTE OF EXPOSURE

Inhalation: May be harmful if inhaled. Can cause rapid

suffocation.

TARGET ORGAN(S) OR SYSTEM(S)

Liver. Kidneys.

SIGNS AND SYMPTOMS OF EXPOSURE

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Gas

Flammable

Explosive

VAPOR PRESSURE 1308.0 mm Hg @ 20 °C

FLASH POINT  -76 °F

Store in an explosion-proof refrigerator or tightly stoppered in a

well-ventilated area

Forms explosive unstable peroxides

Test periodically or discard

SEGREGATION: SHELF # 3

STORAGE GROUP(S):

h - Flammable/Pyrophoric Gas

k - Explosive/Unstable

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   REACTIVE   TOXIC   CORROSIVE

INCOMPATIBILITIES:Copper, Copper alloys, Oxidizing agents.

REACTIVE PROPERTIES

HANDLING: Avoid contact and inhalation. Avoid prolonged or    repeated

exposure. STORAGE: Keep away from heat, sparks, and open flame. Cylinder

temperature should not exceed 125░F (52░C)\. SPECIAL REQUIREMENTS  Air

sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: F+

Indication of Danger: Extremely Flammable.

R: 12

Risk Statements: Extremely flammable.

S: 9 16 33

Safety Statements: Keep container in a well-ventilated place.

Keep away from sources of ignition - no smoking. Take

precautionary measures against static discharges.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VINYLANILINE (4-)    CAS # 1520214

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. May be harmful if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  210 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

Store at 2-8°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VINYLANISOLE (4-)    CAS # 637694

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   2   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. May cause irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE .164 mm Hg @ 20 °C

FLASH POINT  161.6 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

Water spray.

Store at 2-8°C

Keep tightly closed. Keep away from heat and open flame.

REACTIVE PROPERTIES

HANDLING: Avoid breathing vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat and open flame. Store at 2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 23 24/25

Safety Statements: Do not breathe vapor. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VINYLBENZOIC ACID (4-)    CAS # 1075496

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   F   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   CORROSIVE

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at 2-8°C

Keep tightly closed. Store in a cool dry place.

REACTIVE PROPERTIES

HANDLING: Avoid contact and inhalation. Do not get in eyes, on skin, on

clothing. STORAGE: Keep tightly closed. Store in a cool dry place. Store at

2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VINYLBENZYL CHLORIDE (4-)    CAS # 1592207   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   G   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  3   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: Harmful if absorbed through skin. Readily

absorbed through skin.

Eye Contact: Causes burns. Lachrymator.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be harmful

if inhaled   Ingestion: Harmful if swallowed.

SENSITIZATION

skin: May cause allergic skin reaction.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting.

IRRITATION DATA

Eyes

Human

Remarks: Severe irritation effect

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE .360 mm Hg @ 20 °C

FLASH POINT  221 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen chloride gas

Store at 2-8°C

Keep tightly closed. Handle and store under nitrogen.

May develop pressure. Open carefully. Store under inert gas.

REACTIVE PROPERTIES

HANDLING: Open carefully. Do not breathe vapor. Do not get   in eyes, on

skin, on clothing. Avoid prolonged or repeated   exposure. STORAGE: Keep

tightly closed. Handle and store under nitrogen\. Store at 2-8░C SPECIAL

REQUIREMENTS   May develop pressure. Open carefully. Store under inert gas.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 21/22 34 43

Risk Statements: Harmful in contact with skin and if swallowed.

Causes burns. May cause sensitization by skin contact.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VINYLBENZYL CHLORIDE (META,PARA-)    CAS # 57458410

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   E   .   G   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION HAZARD  INHALATION RISK INDEX   <1  - LC50

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

WASTE CHARACTERISTIC HAZARD:

FIRE EXTINGUISHER: CARBON DIOXIDE, DRY CHEMICAL POWDER OR appropriate foam.

TOXIC EMISSIONS WHEN BURNED: S OF:    HYD

REACTIVE PROPERTIES

CONTAINS P-VINYLBENZYL CHLORIDE, CHEM. ABSTRACT REGISTRY NO. 1592-20-    7.

INHIBITED WITH 50-100 PPM TERT-BUTYLCATECHOL AND 500-2000 PPM

NITROMETHANE.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VINYLBENZYL CHLORIDE (PARA-)    CAS # 30030252   HAZARDOUS CHEMICAL OF

CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   G   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  3   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: Harmful if absorbed through skin. Readily

absorbed through skin.

Eye Contact: Causes burns. Lachrymator.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be harmful

if inhaled.

Ingestion: Harmful if swallowed.

SENSITIZATION

skin: May cause allergic skin reaction.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen chloride gas

Store at 2-8°C

Keep tightly closed.

Light and moisture sensitive.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

2-8░C SPECIAL REQUIREMENTS  Light and moisture sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 21/22 34 43

Risk Statements: Harmful in contact with skin and if swallowed.

Causes burns. May cause sensitization by skin contact.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VINYLCARBAZOLE (9-)    CAS # 1484135

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   .   .   .   J   K   L

NFPA HAZARD CODES (H,F,R,O)  0   0   0

ACUTE TOXICTY RISK INDEX 3.5 - LD50   161.3 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: Xn N

Indication of Danger: Harmful. Dangerous for the environment.

R: 21/22 38 43 50/53 68

Risk Statements: Harmful in contact with skin and if swallowed.

Irritating to skin. May cause sensitization by skin contact.

Very toxic to aquatic organisms, may cause long-term adverse

effects in the aquatic environment. Possible risk of

irreversible effects.

S: 36/37 22 23 60 61

Safety Statements: Wear suitable protective clothing and gloves.

Do not breathe dust. Do not breathe vapor. This material and its

container must be disposed of as hazardous waste. Avoid release

to the environment. Refer to special instructions/safety data

sheets.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VINYLCYCLOHEXENE DIOXIDE    CAS # 106876   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   D   E   F   G   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   1   0

ACUTE TOXICTY RISK INDEX 3 - LD50  2130.0 mg/Kg

INHALATION HAZARD  INHALATION RISK INDEX   <1  - LC50   4577.3

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: Readily absorbed through skin. Toxic if

absorbed through skin.

Eye Contact: Causes eye irritation.

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract. Toxic if inhaled.

Ingestion: Toxic if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Thymus. Blood. Male reproductive system.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  224.6 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC   CORROSIVE

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at 2-8°C

Store under inert gas.

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store at

2-8░C SPECIAL REQUIREMENTS  Store under inert gas.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: T

Indication of Danger: Toxic.

R: 23/24/25-68/20/21/22

Risk Statements: Toxic by inhalation, in contact with skin and

if swallowed. Harmful: possible risk of irreversible effects

through inhalation, in contact with skin, and if swallowed.

S: 23-24-45

Safety Statements: Do not breathe vapor. Avoid contact with

skin. In case of accident or if you feel unwell, seek medical

advice immediately (show the label where possible).

OSHA REGULATORY LIMITS

OSHA Permissible Exposure Limit 10 ppm

ACGIH RECOMMENDED LIMITS

ACGIH Threshold Limit Value .1 ppm

US DEPARTMENT OF ENERGY TEEL'S

DOE Occupational Exposure Limit  .1 ppm

DOE Short Term Exposure Limit  .35 ppm

DOE Ceiling Limit 2.5 ppm

Immediately Dangerous to Life and Health   75 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VINYLIMIDAZOLE (1-)    CAS # 1072635

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   2   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns.

Inhalation: Material is extremely destructive to the tissue of

the mucous membranes and upper respiratory tract. May be harmful

if inhaled.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  177.8 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Keep away

from heat, sparks, and open flame\. SPECIAL REQUIREMENTS  Hygroscopic. Heat

and light sensitive.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 22 34

Risk Statements: Harmful if swallowed. Causes burns.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VINYLNAPHTHALENE (2-)    CAS # 827543   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   I   .   .   .

NFPA HAZARD CODES (H,F,R,O)  1   1   0

SOLVENT NARCOTIC OR NEUROTOXIN

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes eye irritation.

Inhalation: Harmful if inhaled. Material may be irritating to

mucous membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Eyes. Kidneys. Liver. Heart.

SIGNS AND SYMPTOMS OF EXPOSURE

Exposure can cause: Gastrointestinal disturbances. May cause

convulsions.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  235 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

Store at 2-8°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. Store at

2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 20/22 36/38

Risk Statements: Harmful by inhalation and if swallowed.

Irritating to eyes and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VINYLPHENYLBORONIC ACID (4-)    CAS # 2156049

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin. May cause irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Boron oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep container closed. Store

in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 20/21/22

Risk Statements: Harmful by inhalation, in contact with skin and

if swallowed.

S: 36

Safety Statements: Wear suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VINYLPHOSPHONIC ACID    CAS # 1746038

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  3   1   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Multiple Routes: Harmful if swallowed, inhaled, or absorbed

through skin.

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

Ccombustible

FLASH POINT  235 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   TOXIC

INCOMPATIBILITIES:Strong bases, Finely powdered metals.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Phosphorous oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: C

Indication of Danger: Corrosive.

R: 34

Risk Statements: Causes burns.

S: 26 36/37/39 45

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing, gloves, and eye/face protection.

In case of accident or if you feel unwell, seek medical advice

immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VINYLPHTHALIMIDE (N-)    CAS # 3485845

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

REACTIVE PROPERTIES

HANDLING: Do not breathe dust. Avoid contact with eyes, skin, and clothing.

STORAGE: Keep tightly closed. Store in a cool dry place.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 36/37/38

Risk Statements: Irritating to eyes, respiratory system and skin.

S: 26 36

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wear

suitable protective clothing.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VINYLPYRIDINE (4-)    CAS # 100436   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   E   .   G   .   .   J   K   L

NFPA HAZARD CODES (H,F,R,O)  3   2   2

ACUTE TOXICTY RISK INDEX 5 - LD50   100.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50    117.8

ROUTE OF EXPOSURE

skin Contact: Causes burns.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: Causes burns. Lachrymator.

Inhalation: May be harmful if inhaled. Material is extremely

destructive to the tissue of the mucous membranes and upper

respiratory tract.

Ingestion: Toxic if swallowed.

SENSITIZATION

Sensitization: May cause allergic respiratory and skin reactions

SIGNS AND SYMPTOMS OF EXPOSURE

Material is extremely destructive to tissue of the mucous

membranes and upper respiratory tract, eyes, and skin.

Inhalation may result in spasm, inflammation and edema of the

larynxand bronchi, chemical pneumonitis, and pulmonary edema.

Symptoms of exposure may include burning sensation, coughing,

wheezing, laryngitis, shortness of breath, headache, nausea, and

vomiting.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

Explosive

FLASH POINT  118.4 °F

Forms explosive unstable peroxides

Test periodically or discard

SEGREGATION: SHELF # 3

STORAGE GROUP(S):

a - Organic Base/Flammable/Toxic

k - Explosive/Unstable

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   REACTIVE   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

FIRE EXTINGUISHER:  Carbon dioxide, dry chemical powder, or appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

Store at -20°C

Store under inert gas.

Store under inert gas. Keep tightly closed. Keep away from heat and open

flame.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Do not get in eyes, on skin, on clothing.

Avoid prolonged or repeated exposure. STORAGE: Store under inert gas. Keep

tightly closed. Keep away from heat and open flame. Store at -20░C SPECIAL

REQUIREMENTS  Store under inert gas.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: T

Indication of Danger: Toxic.

R: 10 25 34 42/43

Risk Statements: Flammable. Toxic if swallowed. Causes burns.

May cause sensitization by inhalation and skin contact.

S: 16 23 26 36/37/39 45

Safety Statements: Keep away from sources of ignition - no

smoking. Do not breathe vapor. In case of contact with eyes,

rinse immediately with plenty of water and seek medical advice.

Wear suitable protective clothing, gloves, and eye/face

protection. In case of accident or if you feel unwell, seek

medical advice immediately (show the label where possible).

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VINYLPYRROLIDONE (1,2-)    CAS # 88120   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    A   .   .   .   .   .   G   .   .   .   K   .

NFPA HAZARD CODES (H,F,R,O)  2   1   2

SPECIAL CARCINOGEN - DESIGNATED AREA MAY BE REQUIRED

CHRONIC TOXICITY RISK INDEX 3.6 - TD50    12.0 mg/Kg

ACUTE TOXICTY RISK INDEX 3 - LD50  1470.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50   3200.0

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: Harmful if absorbed through skin   Eye Contact: Causes

severe eye irritation.

Inhalation: Harmful if inhaled. Material is irritating to mucous

membranes and upper respiratory tract.

Ingestion: Harmful if swallowed.

TARGET ORGAN(S) OR SYSTEM(S)

Liver. Ears.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

Explosive

VAPOR PRESSURE .0781 mm Hg @ 20 °C

FLASH POINT  203 °F

Forms explosive unstable peroxides

Test periodically or discard

SEGREGATION: SHELF # 3

STORAGE GROUP(S):

a - Organic Base/Flammable/Toxic

k - Explosive/Unstable

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   REACTIVE   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides

Store at 2-8°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Do not breathe vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. Store at

2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU DIRECTIVES CLASSIFICATION

Symbol of Danger: Xn

Indication of Danger: Harmful.

R: 20/21/22 37 40 41 48/20

Risk Statements: Harmful by inhalation, in contact with skin and

if swallowed. Irritating to respiratory system. Limited evidence

of a carcinogenic effect. Risk of serious damage to eyes.

Harmful: danger of serious damage to health by prolonged

exposure through inhalation.

S: 26 36/37/39

Safety Statements: In case of contact with eyes, rinse

immediately with plenty of water and seek medical advice. Wea   suitable

protective clothing, gloves, and eye/face protection.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VINYLTOLUENE    CAS # 25013154   HAZARDOUS CHEMICAL OF CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  2   2   0

ACUTE TOXICTY RISK INDEX 3 - LD50  5000.0 mg/Kg

INHALATION RISK INDEX   1.4 - LC50   9145.0

ROUTE OF EXPOSURE

skin Contact: Causes skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Material is irritating to mucous membranes and upper respiratory

tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

VAPOR PRESSURE    1.1 mm Hg @ 20 °C

FLASH POINT  125.6 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD:   IGNITABLE   TOXIC

INCOMPATIBILITIES:Strong oxidizing agents, Strong acids.

Store at 2-8°C

Keep container closed. Keep away from heat, sparks, and open flame.

REACTIVE PROPERTIES

HANDLING: Avoid breathing vapor. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep container closed. Keep

away from heat, sparks, and open flame\. Store at 2-8░C

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

Symbol of Danger: Xi

Indication of Danger: Irritant.

R: 10 38

Risk Statements: Flammable. Irritating to skin.

S: 36

Safety Statements: Wear suitable protective clothing.

OSHA REGULATORY LIMITS

OSHA Permissible Exposure Limit 100 ppm

ACGIH RECOMMENDED LIMITS

ACGIH Threshold Limit Value 50 ppm

ACGIH Short Term Exposure Limit 100 ppm

Immediately Dangerous to Life and Health  400 ppm

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VINYLTRIPHENYLPHOSPHONIUM BROMIDE    CAS # 5044520   HAZARDOUS CHEMICAL OF

CONCERN

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

Inhalation: Material is irritating to mucous membranes and upper

respiratory tract.

Multiple Routes: May be harmful by inhalation, ingestion, or

skin absorption. Causes eye and skin irritation.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong oxidizing agents.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Hydrogen bromide gas

Store at 2-8°C

Hygroscopic.

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid contact and inhalation. Do not get in eyes, on skin, on

clothing. STORAGE: Keep tightly closed. Store at 2-8░C SPECIAL REQUIREMENTS

Hygroscopic.

GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION

EU ADDITIONAL CLASSIFICATION

S: 22 24/25

Safety Statements: Do not breathe dust. Avoid contact with skin

and eyes.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VITAMIN B12    CAS # 58288509

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   L

NFPA HAZARD CODES (H,F,R,O)  0   0   0

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SENSITIZATION

Sensitization: Prolonged or repeated exposure may cause allergic

reactions in certain sensitive individuals.

SIGNS AND SYMPTOMS OF EXPOSURE

To the best of our knowledge, the chemical, physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Solid

SEGREGATION: SHELF # 2

STORAGE GROUP(S):

g - Non-Reactive/Non-Hazardous

WASTE CHARACTERISTIC HAZARD:

INCOMPATIBILITIES:Strong mineral acids.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

TOXIC EMISSIONS WHEN BURNED: Nitrogen oxides Phosphorous oxides

Store at 2-8°C

Keep tightly closed.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed\. Store at

2-8░C

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within The UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.

# VITAMIN K1    CAS # 84800

A  Special Carcinogen        E  Dermal Hazard   I  Neurotoxin

B  Human Terato\Repro Haz    F  Corrosive       J  Suspect Carcinogen

C  Highly Toxic              G  Eye Damage      K  Suspect Terato\Repro Haz

D  Inhalation Hazard         H  STEL            L  Sensitizers

HAZARD INDEX    .   .   .   .   .   .   .   .   .   .   .   .

NFPA HAZARD CODES (H,F,R,O)  0   1   0

ACUTE TOXICTY RISK INDEX 1.2 - LD50 33487.0 mg/Kg

INHALATION RISK INDEX   <1  - LC50

ROUTE OF EXPOSURE

skin Contact: May cause skin irritation.

skin Absorption: May be harmful if absorbed through the skin.

Eye Contact: May cause eye irritation.

Inhalation: May be harmful if inhaled. Material may be

irritating to mucous membranes and upper respiratory tract.

Ingestion: May be harmful if swallowed.

SIGNS AND SYMPTOMS OF EXPOSUR   To the best of our knowledge, the chemical,

physical, and

toxicological properties have not been thoroughly investigated.

PHYSICAL CHARACTERISTICS

PHYSICAL STATE:  Liquid

Ccombustible

FLASH POINT  235 °F

SEGREGATION: SHELF # 1

STORAGE GROUP(S):

l - Flammable/Combustible Solvent

WASTE CHARACTERISTIC HAZARD: IGNITABLE

INCOMPATIBILITIES:Strong oxidizing agents, Strong bases.

FIRE EXTINGUISHER:  Water spray. Carbon dioxide, dry chemical powder, or

appropriate foam.

REACTIVE PROPERTIES

HANDLING: Avoid inhalation. Avoid contact with eyes, skin, and clothing.

Avoid prolonged or repeated exposure. STORAGE: Keep tightly closed. Store in

the dark\. SPECIAL REQUIREMENTS  Light sensitive.

The information presented in the OPMSDS is intended as a synopsis of relative hazard characteristics for this chemical, for application within the UMass-Boston Chem/XL Laboratory Program. This information is derived from a wide range of sources documented in that program. While these sources are considered credible, the user is cautioned that the university cannot guarantee the accuracy nor accept responsibility for damages which may arise from errors, omissions, or the use of this information in any context other than intended. The user is strongly encouraged to seek additional information whenever feasible.